3GPP TSG-RAN WG3 Meeting #105	R3-194776

Ljubljana, Slovenia, 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.300 BLCR for introducing NR SON features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the support of SON for NR |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capture agreements for NR SON into stage 2 specification: - Creating sepcific sections for NR SON - Implement agreements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR SON features are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.X (new), 15.X.1 (new), 15.X.2 (new), 15.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

15.X	Self-optimisation

15.X.1	Support for Mobility Load Balancing

Editor’s note: This section captures the stage 2 descriptions for Mobility Load Balancing

15.X.1.1	General

Editor’s note: the content of this section is FFS.

The objective of mobility load balancing is to distribute cell load evenly among cells or to transfer part of the traffic from congested cell, or to offload users from one cell or carrier or RAT to achieve network energy saving, This can be done by means of optimization of cell reselection/handover parameters and handover actions. The automation of such optimisation can provide high quality user experience, while simultaneously improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

Both intra-RAT and intra-system inter-RAT load balancing scenarios should be supported.

In general, support for mobility load balancing consists of one or more of following functions:

-	 Load reporting;

- Load balancing action based on handovers;

-	 Adapting handover and/or reselection configuration.

15.X.1.2 Load reporting

Editor’s note: the content of this section is FFS.

The load reporting function is executed by exchanging load information over the Xn/X2/F1/E1 interfaces.

Editor’s note: load related information and its definition are FFS.

The following load related information should be supported which consists of,

-	Radio resource usage (DL/UL/SUL GBR PRB usage, DL/UL/SUL non-GBR PRB usage, DL/UL/SUL total PRB usage);

-	TNL load indicator (UL/DL /SUL TNL load: low, mid, high, overload);

-	Cell Capacity Class value (UL/DL relative capacity indicator);

To achieve load reporting function, Resource Status Reporting Initiation & Resource Status Reporting procedures are used.

15.X.2	Support for Mobility Robustness Optimization

Editor’s note: This section captures the stage 2 descriptions for Mobility Robustness Optimization

Editor’s note:All message names in this section are FFS.

15.X.2.1	General

Editor’s note: the content of this section is FFS.

Mobility Robustness Optimisation aims at detecting and enabling correction of following problems:

-	Connection failure due to intra-system or inter-system mobility;

-	Inter-system Unnecessary HO (too early inter-system HO with no radio link failure);

-	Inter-system HO ping-pong.

15.X.2.2	Connection failure due to intra-system mobility

Editor’s note: the content of this section is FFS.

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

- [Intra-system Too Late Handover] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

- [Intra-system Too Early Handover] An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- [Intra-system Handover to Wrong Cell] An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In addition, MRO provides means to distinguish the above problems from NR coverage related problems and other problems, not related to mobility.

Detection mechanism:

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	[Intra-system Too Late Handover]

There is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)

-	[Intra-system Too Early Handover]

There is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect is the cell that served the UE at the last handover initialisation.

-	[Intra-system Handover to Wrong Cell]

There is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The detection of the above events, when involving more than one NG-RAN node, is enabled by the Failure Indication and Handover Report procedures.

15.X.2.3	Connection failure due to inter-system mobility

Editor’s note: the content of this section is FFS.

One of the functions of Mobility Robustness Optimization is to detect connection failures that occurred due to Too Early or Too Late inter-system handovers. The UE makes the RLF Report available to the NG-RAN node after re-establishment attempt or after reconnecting from idle mode. These problems are defined as follows:

- [Inter-system/ Too Late Handover] An RLF occurs after the UE has stayed in a cell belong to NG-RAN node which connects with 5GC for a long period of time; the UE attempts to re-connect to an E-UTRAN cell which connects with EPC.

- [Inter-system/ Too Early Handover] An RLF occurs shortly after a successful handover from a E-UTRAN cell which connects with EPC to a target cell in a NG-RAN node which connects with 5GC; the UE attempts to re-connect to the source cell or to another E-UTRAN cell which connects with EPC.

The UE makes the RLF Report available to a NG-RAN node, when RLF happens in 5GS and the UE re-connects to a NG-RAN node. Availability of the RLF Report at the RRC connection setup or at a handover to NG-RAN node is the indication that the UE suffered a connection failure and that the RLF Report from this failure was not yet delivered to the network.

The NG-RAN node receiving the RLF Report from the UE may forward the report to the NG-RAN node that served the UE before the reported connection failure using the FAILURE INDICATION message over Xn or by means of the uplink RAN configuration transfer procedure and downlink RAN configuration transfer over NG. If present in the RLF Report, the radio measurements may be used to identify lack of coverage as the potential cause of the failure.

Detection mechanism:

Detection mechanisms for Too Late Inter-system Handover and Too Early Inter-system Handover are carried out through the following:

-	[Too Late Inter-system Handover]

The connection failure occurs while being connected to a NG-RAN node, and there is no recent handover for the UE prior to the connection failure i.e., the UE reported timer is absent or larger than the configured threshold, e.g., Tstore_UE_cntxt, and the first node where the UE attempts to re-connect is a E-UTRAN node which connects with EPC.

-	[Too Early Inter-system Handover]

The connection failure occurs while being connected to a NG-RAN node, and there is a recent inter-system handover for the UE prior to the connection failure i.e., the UE reported timer is smaller than the configured threshold, e.g., Tstore_UE_cntxt, and the first cell where the UE attempts to re-connect and the node that served the UE at the last handover initialisation are both E-UTRAN node which connects with EPC.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure.In case the failure is a Too Early Inter-system Handover, the NG-RAN node receiving the FAILURE INDICATION message may inform the E-UTRAN node which connects with EPC by means of RAN Configuration Transfer procedure over NG.

15.X.3	Support for RACH Optimization

Editor’s note: This section captures the stage 2 descriptions for RACH Optimization

--------------------------------------------------End of the change-----------------------------------------------------