3GPP TSG-RAN3 Meeting #105bis	R3-194912

Chongqing, China, 14-18 October 2019

3GPP TSG-RAN WG2 Meeting #107	R2-1911565

Prague, Czech Republic, 26th – 30th August 2019

Title:	LS on AS key derivation for conditional handover

Response to:	-

Release:	Release 16

Work Item:	NR_Mob_enh-Core, LTE_feMob-Core

Source:	TSG RAN WG2

To:	TSG SA WG3

Cc:	TSG RAN WG3

Contact Person:

#### Name:	Art Ishii

E-mail Address:	ishiia@sharplabs.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R2-1910807, R2-1910810

1. Overall Description:

RAN2 is currently working on the details of Conditional Handover (CHO), where the UE may be provided by the source eNB/gNB a handover configuration with triggering condition(s). The UE executes a handover to a candidate cell when the triggering condition is met. It has been agreed that the handover configuration can include a single or multiple candidate cells for CHO. RAN2 agreed for Release 16, intra-eNB CHO, intra-gNB-CU CHO, X2-based CHO and Xn-based CHO is supported and S1-based CHO or N2-based CHO is not supported.

In a legacy handover, whenever the AS key (KeNB/KgNB) is to be updated, the source eNB/gNB shall perform a vertical key derivation in case it has an unused {NH, NCC} pair, otherwise, it shall perform a horizontal derivation from the currently active KeNB/KgNB. The derived {KeNB*/ KNG-RAN*, NCC} will be sent to the target eNB/gNB. According to TS 33.401 the AS key shall always be refreshed during handover whereas according to TS 33.501 if there is change in the PDCP anchor point for the DRB(s) of the UE the AS key shall be refreshed else whether to update or retain the AS key is based on operator policy.

RAN2 assumes that, in CHO, if there are multiple candidate cells that require key updates, the above principle will be applied to each of the candidate cells (refer to the attachments).

RAN2 assumes there are no security issues for the AS key derivation for multiple candidate cells in CHO.  However, RAN2 would like to get feedback from SA3 whether there are any security issues.

2. Actions:

To SA3 group.

ACTION: 	RAN2 respectfully asks SA3 to provide any feedback about if  there are any security issues for CHO.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#107-Bis		14 – 18 October 2019	Chongqing, China

3GPP RAN2#108		18 – 22 November 2019	Reno, US