3GPP TSG-RAN3 Meeting #105bis	R3-195128

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0242 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CIoT CN Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When connecting to 5GC, it is needed to: - allow CN selection based on different CIoT features by the RAN node |  |  |  |  |  |  |  |  |
| Summary of change: |  | - introduce CIoT Support Indicator in NGAP: NG SETUP RESPONSE message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN node cannot select proper CN node for CIoT UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1.2, 8.7.1.2, 9.2.5.1, 9.2.6.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----Start of the First Change----

8.7.1	NG Setup

8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the CIoT Support Indicator IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall consider this information when selecting an appropriate AMF for the UE.

----Start of the Next Change----

9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Served GUAMI List |  | 1 |  |  | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| CIoT Support Indicator |  |  | 9.3.3.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

----Start of the Next Change----

9.3.3.xxx	CIoT Support Indicator

This IE indicates the supported CIoT features by the AMF to the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CIoT Support Indicator | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates whether the AMF supports the corresponding CIoT features. First Bit = LTE-M, Second Bit = NB-IoT, Third Bit = CIoT CP, Other bits shall be ignored by the NG-RAN node. |

----Start of the Next Change----

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

CIoT-SupportIndicator,

Cause,

CellIDListForRestart,

CNAssistedRANTuning,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformationForInactive,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EN-DCSONConfigurationTransfer,

EUTRA-CGI,

//skip the unchanged part

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-CIoT-SupportIndicator,

id-CNAssistedRANTuning,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformationForInactive,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-ENDC-SONConfigurationTransferDL,

id-ENDC-SONConfigurationTransferUL,

id-EUTRA-CGI,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

//skip the unchanged part

-- **************************************************************

--

#### -- INTERFACE MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- NG Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID			CRITICALITY reject	TYPE GlobalRANNodeID			PRESENCE mandatory	}|

{ ID id-RANNodeName				CRITICALITY ignore	TYPE RANNodeName				PRESENCE optional}|

{ ID id-SupportedTAList			CRITICALITY reject	TYPE SupportedTAList			PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX		CRITICALITY ignore	TYPE PagingDRX					PRESENCE mandatory	}|

{ ID id-UERetentionInformation	CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- NG SETUP RESPONSE

--

-- **************************************************************

NGSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupResponseIEs} },

...

}

NGSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName						CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-ServedGUAMIList				CRITICALITY reject	TYPE ServedGUAMIList			PRESENCE mandatory	}|

{ ID id-RelativeAMFCapacity			CRITICALITY ignore	TYPE RelativeAMFCapacity		PRESENCE mandatory	}|

{ ID id-PLMNSupportList				CRITICALITY reject	TYPE PLMNSupportList			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional		}|

{ ID id-CIoT-SupportIndicator		CRITICALITY ignore	TYPE CIoT-SupportIndicator		PRESENCE optional		},

...

}

----Start of the Next Change----

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-Cause,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

//skip the unchanged part

CellSize ::= ENUMERATED {verysmall, small, medium, large, ...}

CellType ::= SEQUENCE {

cellSize		CellSize,

iE-Extensions		ProtocolExtensionContainer { {CellType-ExtIEs} }	OPTIONAL,

...

}

CellType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CIoT-SupportIndicator ::= BIT STRING (SIZE(32))

CNAssistedRANTuning ::= SEQUENCE {

expectedUEBehaviour					ExpectedUEBehaviour												OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {CNAssistedRANTuning-ExtIEs} }	OPTIONAL,

...

}

CNAssistedRANTuning-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

----Start of the Next Change----

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

//skip the unchanged part

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-CIoT-SupportIndicator								ProtocolIE-ID ::= aaa

END

-- ASN1STOP

----End of the Changes----