3GPP TSG-RAN WG3 Meeting #105bis	R3-195158

# Chongqing, P.R. China, 14th – 18th Octorber 2019

Agenda Item:	17.3.2

Source:	Ericsson

Title: 	Provisioning of UE’s TSC traffic pattern related information

Document for:	Discussion and Approval

# 1	Information

In [1], the Work Item “Support of NR Industrial Internet of Things (IoT)”  is approved and one of the objectives is:

- Specify enhancements to satisfy QoS for wireless Ethernet when using TSC traffic patterns, including

  - Support of provisioning, from Core Network to RAN and between RAN nodes (e.g. upon handover), of UE’s TSC traffic pattern related information such as message periodicity, message size, message arrival time at gNB (DL) and UE (UL) [RAN3].

  - Support for multiple simultaneous active semi-persistent scheduling (SPS) configurations for a given BWP of a UE. [RAN2, RAN1].

  - Support for shorter SPS periodicities than the existing ones [RAN2, RAN1].

This paper discusses the provision of the TSC traffic patterns.

# 2	Discussion

In [2], SA2 has agreed the solution that 5G core network should provide / relay TSN traffic pattern of the incoming TSN traffic to gNB, so that gNB can use the information to do better radio optimization. The TSN traffic pattern is available at the 5G core interacting with TSN CNC and it could be provided to the RAN beforehand.

The information of the traffic pattern such as at least periodicity and message size would be required. Also, information on when the periodic data arrives i.e. a time reference or offset would be helpful.

Note: What is the periodicity in reference to, e.g. absolute time, is FFS. How does Message size relate to MDBV defined for 5GS QoS is FFS.

As show in the below figures extract from [2], the additional TSN traffic pattern A is sent to gNB in downlink, and the additional TSN traffic pattern C is sent to gNB in uplink, for radio optimization.

Delivery of additional TSN traffic pattern to RAN for radio optimization (downlink)

The procedure in [2] is show in below chart.

Figure 6.x.2.1-1 5GS distributes additional TSN traffic pattern to gNB for radio optimization purpose The delivery of TSN traffic pattern used by current node for 5GS output scheduling has been described in the Solution, the following procedure describes the difference that related to deliver additional TSN traffic pattern used by preceding node. - CNC distributes the TSN QoS requirements and traffic pattern (specific for current node) to 5G virtual bridge via TSN AF. For radio optimization purpose, the AF can request the additional traffic pattern for the preceding node from the CNC (e.g. triggered by CNC configuration event), i.e. when CNC sends TSN QoS requirements and “traffic pattern B” to AF as part of bridge configuration request, AF then asks CNC for additional TSN traffic pattern information of the preceding node. Note*: During the 5G virtual bridge registration stage, the 5GS may indicate the needs of traffic pattern for both current bridge and preceding node. Then CNC may be configured to send both traffic patterns (e.g. both “TSN traffic pattern B” and “traffic pattern A” in figure 1) to 5G bridge. However the CNC behaviour is out of scope of 3GPP. - The additional TSN traffic pattern used by preceding node are forwarded to PCF (directly or via NEF) - PCF finds suitable 5G QoS policies and rules that can fulfil the TSN QoS characteristics (i.e. mapping TSN QoS characteristics to 5G QoS profiles). - PCF triggers the PDU session modification procedure to establish a new 5G QoS flow according to the selected QoS policies and rules. SMF configures UE, gNB and UPF for QoS enforcement according to existing 3GPP procedure (see TS 23.501 clause 5.7). Every QoS flow has a QoS flow Identifier (QFI), therefore for every mapped TSN flows, the QFI can be used to identify the link between TSN traffic profile and QoS flow. An example mapping can be: QFI 5G QoS profile TSN QoS characteristics TSN traffic pattern used by current node TSN traffic pattern used by preceding node. In both uplink and downlink direction, PCF distributes the additional TSN traffic pattern used by preceding node to RAN via SMF and AMF. The additional TSN traffic pattern used by preceding node and associated QFI reference are sent from SMF to RAN via N2 information using existing PDU session establishment/modification procedure. New information element may need to be introduced in N2 information to carry the additional TSN traffic pattern and QFI reference. PDU session resources setup/modification procedure can be used to carry the additional TSN information from AMF to gNB, e.g. NGAP over N2 [TS 38.413]. The QFI reference is used by gNB to link the additional TSN traffic pattern to a specific TSN flow. - PCF response to TSN AF (directly or via NEF). - TSN AF response to CNC.

## 2.1	RAN3 Impacts

In the above steps, the step 4 is in the scope of RAN3, to provision the additional TSN traffic pattern from SMF to RAN. Following the communication between SMF and RAN node, the SMF IE could be used.

In TS 38.413, the QoS Flow list is included in the SMF IE transfer in the PDU Session Resource Setup/Modify Request.

A natural way would be to include a “Traffic Characteristics” in the structure to convey the additional TSN traffic pattern information.

#### 9.3.4.1 PDU Session Resource Setup Request Transfer This IE is transparent to the AMF. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. PDU Session Aggregate Maximum Bit Rate; O; 9.3.1.102; This IE shall be present when at least one non-GBR QoS flow is being setup.; YES; reject. UL NG-U UP TNL Information; M; UP Transport Layer Information 9.3.2.2; UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs.; YES; reject. Additional UL NG-U UP TNL Information; O; UP Transport Layer Information 9.3.2.2; UPF endpoint of the additional NG-U transport bearer, for delivery of UL PDUs.; YES; reject. Data Forwarding Not Possible; O; 9.3.1.63; This IE may be present in case of HANDOVER REQUEST message and shall be ignored otherwise.; YES; reject. PDU Session Type; M; 9.3.1.52; YES; reject. Security Indication; O; 9.3.1.27; YES; reject. Network Instance; O; 9.3.1.113; YES; reject. QoS Flow Setup Request List; 1; YES; reject. >QoS Flow Setup Request Item; 1..<maxnoofQoSFlows>; -. >>QoS Flow Identifier; M; 9.3.1.51; -. >>QoS Flow Level QoS Parameters; M; 9.3.1.12; -. >>E-RAB ID; O; 9.3.2.3; -. >>Traffic Characteristics; O; x.y.z; Details in TS 23.501 [9]. This IE carry the additional Traffic pattern information associated with the QFI.; -. Range bound; Explanation. maxnoofQoSFlows; Maximum no. of QoS flows allowed within one PDU session. Value is 64..

The other RAN3 specification has the similar QoS Flow List structure.

To include the “Traffic Characteristics” at the same level as the “QoS Flow Level QoS Parameters” brings the most flexibility, as we believe that it can be modified separately, without need to modify the QoS parameters.

Proposal 1:   RAN3 to agree to include the “Traffic Characteristics” in the QoS Flow List, at the same level as “QoS Flow Level QoS Parameters”.

# 3	Proposal

Proposal 1:   RAN3 to agree to include the “Traffic Characteristics” in the QoS Flow List, at the same level as “QoS Flow Level QoS Parameters”.

[3] to [6] are submitted to show the implementation in the NGAP, XnAP, F1AP and E1AP.

# 4	Reference

[1]	RP-190276	New WID: Support of NR Industrial Internet of Things (IoT)

[2]	S2-1901234	Additional TSN traffic pattern for RAN optimization, Ericsson

[3]	R3-195327	Provisioning of UE’s TSC traffic pattern related information, Ericsson

[4]	R3-195328	Provisioning of UE’s TSC traffic pattern related information, Ericsson

[5]	R3-195329	Provisioning of UE’s TSC traffic pattern related information, Ericsson

[6]	R3-195159	Provisioning of UE’s TSC traffic pattern related information, Ericsson