3GPP TSG-RAN WG3 Meeting #105-bis	R3-185210

Chongqing, P.R. China, 14-18 Oct 2019

Agenda item:	15.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Management of identifiers during CHO

Document for:	Discussion

# 1	Introduction

At RAN3 #105, it was discussed if any limitations to the UE ID handling is needed. It was also declared that the current principles shall not be changed. In this paper, we consider the situation currently and conclude how the source and target shall handle UE context in case multiple preparation is applied.

# 2	Discussion

UE-associated signalling, defined in the TS 36.401 / TS 38.401, refers to signalling related to a single UE. In most cases such signalling used two UE IDs, one from the initiator node and the other from the peer node (old/new ID or MN/SN ID). However, there is a number of messages on both, X2 and Xn, which enable using only the sender’s UE ID – and which are still UE-associated (UE associated signalling may exist before UE context is created). Normally, such messages are used when the sender does not yet know the ID allocated at the peer node. However, when both IDs are available, they shall be used to identify the UE in the signalling, too.

Observation 1: So far, “UE-associated signalling” meat signalling identified with at least the sender’s UE ID. Receiver’s UE ID is not always used, but when used, it identifies UE association.

In the context of CHO and multiple preparation towards the same target node, it has been considered that both nodes shall have full freedom in the UE context management. There is no issue about the source: if it manages each CHO request as a separate UE context (a new UE ID) the target must follow it, because it has no indication the requests concern the same UE. So, the only possible issue is the situation when the source uses the same UE ID, but the target prefers to allocate new UE ID to each acknowledged request.

As observed above, when two UE IDs are known to both nodes, each UE association is identified by the pair. Therefore, if the target allocates a new UE ID, a new association is created. Since now on, the source has to handle two associations.

Observation 2: The existing principles governing UE association are not violated if both, the source and the target, have full freedom in the scope of UE context handling.

One problematic scenario may be source-initiated CHO cancellation: if the source has to cancel a 2nd or following CHO preparation before it receives the target’s UE ID, it can’t use the ID received before, because it is not sure how the target manages UE contexts. However, the problem has its other side: if the source does use two IDs, but the target stores all preparations within a single UE context (the same UE ID), the pair does not identify particular preparation, either. This was acknowledged already at RAN3 #105 and to enable identification, it was decided to use the target cell ID (the list of target cells to cancel was agreed to be added to the existing HO Cancel procedure). The same can now be used for cancelling of the following preparation, when the target’s ID is not known.

One may note, that the problem is not new: in the case of a classic HO and multiple preparation towards the same target node (not common, but not forbidden, either), the source can’t cancel one preparation only!

Observation 3: The source may use the target cell ID to cancel CHO preparation if the target node’s UE ID is not yet delivered to the source. This is improvement over the classic HO.

Considering that multiple HO preparation towards the same target node is allowed, and each is identified with the target cells ID, data forwarding may be considered. Each HO preparation may use separate TEIDs. Therefore, in the case that the target allocated the same UE ID to all preparations, but different TEIDs, the source must know which TEID to use once the UE accesses the target node. To enable it, the target cell ID shall be included in the Handover Success procedure.

Proposal 1: The Handover Success procedure shall carry the target cell ID and the target’s tunnel endpoint used in case of late forwarding shall correspond to the target cell ID reported in the Handover Success.

Yet another issue related to UE identification is cancelling all other HO preparations, once the UE accesses the target node – at this moment, all other preparations become redundant. The safe way, which secures all possible implementations, is explicit cancellation of all other preparations, irrespectively from the node they were made at. However, this may cause excessive signalling – and likely, unnecessary. So, some optimisations can be considered.

Even if explicit cancelling of redundant preparations in other nodes is likely inevitable, preparations made towards the same node as the one where the UE accessed the target cell may be cancelled automatically, by the target node itself. Of course, only if it can identify them as belonging to the same UE. Since it was proposed at RAN3 #105 that such handling shall not be mandated, the automatic cancellation may concern only the same UE association (i.e. pair of UE IDs).

Proposal 2: Once the target cell reports the handover success, all CHO preparations made for the same pair of UE IDs are cancelled at the target node.

# 3	Conclusions

In this paper, we analyse how the 3 IDs used in the HO (source’s UE ID, target’s UE ID and the target cell ID) can be employed in order to handle all the features of CHO. We make following observations:

Observation 1: So far, “UE-associated signalling” meat signalling identified with at least the sender’s UE ID. Receiver’s UE ID is not always used, but when used, it identifies UE association.

Observation 2: The existing principles governing UE association are not violated if both, the source and the target, have full freedom in the scope of UE context handling.

Observation 3: The source may use the target cell ID to cancel CHO preparation if the target node’s UE ID is not yet delivered to the source. This is improvement over the classic HO.

This allows to make following proposals:

Proposal 1: The Handover Success procedure shall carry the target cell ID and the target’s tunnel endpoint used in case of late forwarding shall correspond to the target cell ID reported in the Handover Success.

Proposal 2: Once the target cell reports the handover success, all CHO preparations made for the same pair of UE IDs are cancelled at the target node.

The proposals are implemented in two TPs, for XnAP BL CR [1] and for X2AP BL CR [2].

# References

- R3-195211, RAN3 #105-bis

- R3-195212, RAN3 #105-bis