3GPP TSG-RAN3 Meeting #105bis	R3-195333

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transmission order of UE level NAS-PDU and PDU session NAS-PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 38.413, the UE level NAS-PDU and PDU session level NAS-PDU are included in the following NGAP messages. • PDU SESSION RESOURCE SETUP REQUEST • INITIAL CONTEXT SETUP REQUEST According to section 4.2.2 of TS 38.331, only PDU session level NAS PDU(s) can be piggybacked in RRC reconfiguration message, while UE level NAS PDU is in DLInformationTransfer message. It is not clear which kind of NAS-PDU is transmitted first since the NAS-PDU is expected to be received in sequence of Sequence Number. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the UE level NAS-PDU is sent first when the gNB receives both UE level NAS-PDU and PDU session level PDU in a single NGAP message. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the transport of NAS-PDU messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear which kind of NAS-PDU(s) is transmitted first. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change Begins>>>>>>>>>>>>>>>>>>>>

7.6	Transport of NAS Messages

NR provides reliable in-sequence delivery of NAS messages over SRBs in RRC, except at handover where losses or duplication can occur when PDCP is re-established. In RRC, NAS messages are sent in transparent containers. Piggybacking of NAS messages can occur in the following scenarios:

-	At bearer establishment/modification/release in the DL;

-	For transferring the initial NAS message during connection setup and connection resume in the UL.

NOTE:	In addition to the integrity protection and ciphering performed by NAS, NAS messages can also be integrity protected and ciphered by PDCP.

Multiple NAS messages can be sent in a single downlink RRC message during PDU Session Resource establishment or modification. In this case, the order of the NAS messages contained in the RRC message shall be in the same order as that in the corresponding NG-AP message in order to ensure the in-sequence delivery of NAS messages.

When NG-RAN nodes receives one mobility management NAS-PDU and one or more session management NAS-PDUs in a single NG-AP message, the NG-RAN shall pass to the UE the mobility management NAS-PDU first.

<<<<<<<<<<<<<<<<<<<< Change Ends>>>>>>>>>>>>>>>>>>>>