3GPP TSG-RAN3 Meeting #105bis	R3-195335

Chongqing, China, 14-18 October 2019

Title: 	Correction of NAS transparent container

Source: 	Huawei

Agenda item:	9.3.1

Document for:	Discussion

1. Introduction

The correction of NAS transparent container was discussed during RAN3#105 meeting. In [1], the conclusion was that this issue will be further discussed at RAN3#105bis meeting. In this document we give further discussion on this issue on the basis of the offline progress.

2. Background

At RAN3#105 meeting, two companies agree to add the “value part of” towards the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container” so as to align with the TS 24.501 specification.

One company raises the issue that this may impact on descriptions in TS 38.331 considering the TS 38.331 has a reference to the TS 24.501 with the descriptions as “The content is defined in TS 24.501 [23]”.

3. Discussion

In TS 38.413 the NASC information is included in the HANDOVER REQUEST as follows:

| NASC | O |  | NAS-PDU 9.3.3.4 | Containing either the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container” specified in TS 24.501 [26]. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

However, according to the TS 24.501, only the value part of the “Intra N1 mode NAS transparent container” or the “S1 mode to N1 mode NAS transparent container” information element is included within the RRC message sent to the UE. Specifically,

In clause 9.11.2.6 in TS 24. 501:

- The value part of the Intra N1 mode NAS transparent container information element is included in specific information elements within some RRC messages sent to the UE.

In clause 9.11.2.9 in TS 24.501:

- The value part of the S1 mode to N1 mode NAS transparent container information element is included in specific information elements within some RRC messages sent to the UE.

Hence there are mis-alignments between TS 38.413 and TS 24. 501.

Observation 1: For NASC, the descriptions in TS 38.413 is not aligned with that in TS 24.501.

Further, in TS 38.331, the following NAS-container is included in the RRCReconfiguration message.

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

Observation 2: in TS 38.331, only the content reference to TS 24.501 is included within the NAS-Container.

In light of discussion above, there is a need to correct the NASC descriptions in TS 38.413. Otherwise, this may incur the interoperability issue. In order to keep align with the TS 24.501 and TS 38.331 and avoid the impact on the descriptions in TS 38.331, it is proposed to update the semantics description to be “Containing contents specified in TS 24.501”.

This change could be aligned with the existing NAS Security Parameters from NG-RAN in Handover Command message, which is given in the appendix.

Update the semantics description on NASC IE in the HANDOVER REQUEST message to be “Containing contents specified in TS 24.501”.

4. Conclusion

Based on the discussion in this paper, we propose to:

Observation 1: For NASC, the descriptions in TS 38.413 is not aligned with that in TS 24.501.

Observation 2: in TS 38.331, only the content reference to TS 24.501 is included within the NAS-Container.

Proposal 1:	Update the semantics description on NASC IE in the HANDOVER REQUEST message to be “Containing contents specified in TS 24.501”.

- The corresponding CR is provided in [2].

5. Reference

- R3-194665, Summary of offline discussion on NAS transparent container, Huawei.

- R3-195336	Correction of NAS transparent container, Huawei

5. Appendix- NAS Security Parameters from NG-RAN in Handover Command message

9.3.3.26	NAS Security Parameters from NG-RAN

This IE provides security related parameters for inter-system handover from NG-RAN to E-UTRAN via the eNB to the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NAS Security Parameters from NG-RAN | M |  | OCTET STRING | Coded as the value part of the N1 mode to S1 mode NAS transparent container IE defined in TS 24.501 [26]. |