3GPP TSG-RAN3 Meeting #105bis	R3-195412

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for SON BL CR for TS 38.423): MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current version of specification does not include the details for mobility optimization function. In order to support the mobility optimization function, some new information elements should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the detailed information elements into Failure indication and HO report procedures for supporting mobility optimization function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of mobility optimization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x, 8.4.y, 9.1.3.x, 9.1.3.y,9.2.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Based on the BL CR R3-194771 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

### 8.4.x	Failure Indication

#### 8.4.x.1	General

The purpose of the Failure Indication procedure is to transfer information regarding RRC re-establishment attempts, or received RLF Reports, between NG-RAN nodes. The signalling takes place from the NG-RAN node at which a re-establishment attempt is made, or an RLF Report is received, to an NG-RAN node to which the UE concerned may have previously been attached prior to the connection failure. This may aid the detection of radio link failure, handover failure cases.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: Failure Indication, successful operation

NG-RAN node2 initiates the procedure by sending the FAILURE INDICATION message to NG-RAN node1, following a re-establishment attempt or an RLF Report reception from a UE at NG-RAN node2 in handover case, when NG-RAN node2 considers that the UE may have previously suffered a connection failure at a cell controlled by NG-RAN node1.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

### 8.4.y	Handover Report

#### 8.4.y.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.y.2	Successful Operation

Figure 8.4.y.2-1: Handover Report, successful operation

An NG-RAN node initiates the procedure by sending an HANDOVER REPORT message to another NG-RAN node. By sending the message NG-RAN node1 indicates to NG-RAN node2 that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then the NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

#### 8.4.y.3	Unsuccessful Operation

Not applicable.

#### 8.4.y.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.x	FAILURE INDICATION

This message is sent by the NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.X |  |  |  | YES |  | ignore |  |
| CHOICE presence of RLF report |  | M |  |  |  |  |  |  |  |  |  |  |  |
| > No RLF report |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Failure cell PCI |  | M |  |  |  | 9.2.2.10 |  | Physical Cell Identifier |  | YES |  | ignore |  |
| >>Re-establishment cell CGI |  | M |  |  |  | NG-RAN CGI 9.2.2.x |  |  |  | YES |  | ignore |  |
| >>C-RNTI |  | M |  |  |  | BIT STRING (SIZE (16)) |  | C-RNTI contained in the RRCReestablishment Request message (TS 38.331 [10])or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) |  | YES |  | ignore |  |
| >>ShortMAC-I |  | M |  |  |  | BIT STRING (SIZE (16)) |  | ShortMAC-I contained in the RRCReestablishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) |  | YES |  | ignore |  |
| > RLF report available |  |  |  |  |  |  |
| >>UE RLF Report Container | M |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10]) or TS 36.331 [14]) | YES | ignore |

#### 9.1.3.y	HANDOVER REPORT

This message is sent by the NG-RAN node1 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CHOICE Handover Report Type | M |  |  |  |  |  |
| > HO too early |  |  |  |  |  |  |
| >> Source cell CGI | M |  | NG-RAN CGI 9.2.2.X | CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| >> Failure cell CGI | M |  | NG-RAN CGI 9.2.2.X | CGI of target cell for handover procedure (in NG-RAN node 1) | YES | ignore |
| >>Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| >>Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| >>Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from NG-RAN node 2. | YES | ignore |
| >>UE RLF Report Container | O |  | OCTET STRING | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| > HO to wrong cell |  |  |  |  |  |  |
| >> Source cell CGI | M |  | NG-RAN CGI 9.2.2.X | CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| >> Failure cell CGI | M |  | NG-RAN CGI 9.2.2.X | CGI of target cell for handover procedure (in NG-RAN node 1) | YES | ignore |
| >> Re-establishment cell CGI | M |  | NG-RAN CGI 9.2.2.X | CGI of cell where UE attempted re-establishment | YES | ignore |
| >>Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| >>Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| >>Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from NG-RAN node 2. | YES | ignore |
| >>UE RLF Report Container | O |  | OCTET STRING | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |

#### 9.2.2.10	NG-RAN Cell PCI

This IE defines physical cell ID of a cell served by an NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAT | M |  |  |  |
| >nr |  |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID |
| >e-utra |  |  |  |  |
| >>E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell ID |

#### 9.2.2.x	NG-RAN CGI

This IE is used to globally identify a cell in NG-RAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAT | M |  |  |  |
| >nr |  |  |  |  |
| >>NR CGI | M |  | 9.2.2.7 |  |
| >e-utra |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.2.2.8 |  |

End of the next change