3GPP TSG-RAN WG3 Meeting #105bis	R3-195538

Chongqing, China, Oct. 14th – 18th, 2019

Agenda item:		20.2.2

Source:	LG Electronics Inc.

Title:	Issues on F1 for support of sidelink resource mode 1 and 2

Document for:		Discussion

1. Introduction

In last meeting, the issues on F1 for support of sidelink resource mode 1 and 2 were initially discussed, but not much time was spent online. A summary was given in [3] for the open issues. This paper is to investigate the issues again. The corresponding proposals are also provided.

2. Discussion

In TR 38.885 [2], RAN1 has agreed to support the following resource allocation modes:

- Mode 1: BS schedules SL resource(s) to be used by UE for SL transmission(s)

- Dynamic resource allocation

- Configured grant Type 1

- Configured grant Type 2 (activate/deactivate configured grant with DCI)

- Mode 2: UE determines, i.e. BS does not schedule, SL transmission resource(s) within SL resources configured by BS/network or pre-configured SL resources

Issue 1: OAM configures NR V2X resource pool to CU or DU

During the study of NR V2X, the following impact was identified in TR 38.885 [2]:

- Impacts on F1 interface

  - An impact on the F1 interface includes whether the resource pool for side link communication is configured in the gNB-CU or gNB-DU.

Therefore, we need to make a conclusion on that NR V2X resource pool is configured by OAM whether to CU or DU. From practical point of view, it is better for OAM to configure it to CU for the following three reasons:

Firstly, generally CU may control many DUs and it can take the role of coordination on deciding the V2X sidelink resource configurations (for SIB broadcasting) of different DUs.

Secondly, in LTE the V2X resource pool coordination between eNBs was performed by OAM. If the same principle is applied for CU/DU based NG-RAN, OAM will do the coordination between CUs on V2X resource pools. It seems to be more convenient for OAM to configure the pool to CU.

Thirdly, for CU/DU split based NG-RAN, only the parameters of MIB and SIB1 were configured to DU while the parameters on other SIBs were configured to CU. In addition, according to the decision of legacy LTE V2X mode 4, it was defined to broadcast using SIB Type 21. Therefore, it is also very straightforward to use other SIB (e.g., SIB21) in NR V2X for broadcasting. Thus, the parameters, e.g., resource pool, on this SIB should be configured to CU.

According to the analysis above, the following proposal is suggested to RAN3:

Proposal 1): OAM configures NR V2X resource pool to CU.

Issue 2: how to support the resource allocation modes for connected mode UE

For connected mode UE, UE can request the resource allocation from gNB. One example is shown in the Figure below.

Figure 1. Signalling between CU and DU to support mode 1 of NR Resource Allocation

Basically, DU has the lower layer PHY/MAC, which is in charge of scheduling for the UE specific request. Thus it is logical that gNB-DU is responsible for the scheduling of sidelink resource for mode 1 dynamic case and type 2.

On the other hand, according to the legacy LTE V2X mode 3 case (TS 36.331), the following RRC configuration is sent to UE.

…

Thus, when CU receives the resource request from UE, it triggers the procedure to DU for getting the V2X SL Resource Configuration from DU. After DU allocate the SL resource for this UE, it passes this V2X SL Resource Configuration to CU, which generates the RRC message with it and finally sends the RRC message to UE. UE will use this information for the resource.

In legacy LTE V2X, UE type, i.e., P-UE or V-UE, is necessary information for eNB to select the resource. Thus, it is also necessary for CU to pass this information to DU so that DU can decide finally the resource based on UE type.

Proposal 2): For resource allocation Mode 1 (dynamic case and type 2 case), gNB-DU is responsible for the scheduling of sidelink resource. gNB-DU transmits the V2X SL configuration information to the gNB-CU, which uses it to generate the RRC message and forwards it to the UE.

Proposal 3): UE type, i.e., P-UE or V-UE, is transmitted from CU to DU for DU to decide the resource pool to use.

On the other hand, for resource allocation mode 2 and mode 1 type 1, the configured resource pool or the configured grant is relatively static. In addition, according to proposal 1 since OAM configures NR V2X resource pool to CU, thus gNB-CU can allocate the resource pool/grant to UE directly, generate the RRC message and sends it to the UE.

Proposal 4): For resource allocation Mode 2 and Mode 1 type 1, gNB-CU allocates the resource pool/grant to UE directly, generate the RRC message and sends it to the UE.

Issue 3: How to support on-demand broadcast (V2X SIB) for idle or inactive mode UE

Firstly, on message 1 based on-demand broadcast request from idle/inactive UE, message 1 goes to DU directly and DU can gives the response to UE by broadcasting the requested V2X SIB. This is based on the current specification, that is, DU can get all the other SIBs (except SIB1) from CU, which encodes them, by F1 Setup procedures. Therefore, there is no specification impact for message 1 based on-demand broadcast request from idle/inactive UE.

Observation 1): There is no RAN3 impacts on message 1 based on-demand broadcast (V2X SIB) request from idle/inactive UE.

On message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE, One example is shown below:

Figure 2. Signalling between CU and DU to support mode 2 of NR Resource Allocation

According the discussion before, MIB and SIB1 are in charged by DU. The location of encoding other SIBs are in CU, which is the principle. Therefore, if this principle has to be kept, the location of SIB (e.g., SIB21) for NR V2X should locate in the CU.

As shown in the figure above, UE transmits the on demand broadcast request to CU, which can make a decision and indicates the SIB index to DU. DU finally broadcast the resource to UE based on the index from CU.

For supporting the procedure above, CU needs to tell the DU in advance on the information elements of V2X SIB (e.g., SIB21) by F1 Setup Response message, which is shown as follows:

------------------------------------------------------------------------------------------------------------------------

9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | - | - |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| gNB-CU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |

------------------------------------------------------------------------------------------------------------------------------------------------

However, this is similar to the defined SIB2~SIB9 of the current specification. There is no additional RAN3 impacts foreseen. RAN2 needs to define the SIB for V2X e.g. SIB21 as in legacy.

Observation 2): No additional RAN3 signaling impacts are required on supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE. The impacts are on RAN2, i.e., defining the SIB for V2X e.g. SIB21 as in legacy in RRC Specification.

Based on the analysis above, the following proposal is suggested to RAN3:

Proposal 5): On supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE, the location of encoding SIB for V2X resource broadcasting is in CU.

3. Conclusion

In this contribution, the issues of F1 were investigated in order to support NR sidelink resource allocation mode 1 and mode 2. The following proposal is suggested to RAN3:

Proposal 1): OAM configures NR V2X resource pool to CU.

Proposal 2): For resource allocation Mode 1 (dynamic case and type 2 case), gNB-DU is responsible for the scheduling of sidelink resource. gNB-DU transmits the V2X SL configuration information to the gNB-CU, which uses it to generate the RRC message and forwards it to the UE.

Proposal 3): UE type, i.e., P-UE or V-UE, is transmitted from CU to DU for DU to decide the resource pool to use.

Proposal 4): For resource allocation Mode 2 and Mode 1 type 1, gNB-CU allocates the resource pool/grant to UE directly, generate the RRC message and sends it to the UE.

Observation 1): There is no RAN3 impacts on message 1 based on-demand broadcast (V2X SIB) request from idle/inactive UE.

Observation 2): No additional RAN3 signaling impacts are required on supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE. The impacts are on RAN2, i.e., defining the SIB for V2X e.g. SIB21 as in legacy in RRC Specification.

Proposal 5): On supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE, the location of encoding SIB for V2X resource broadcasting is in CU.

4. References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- 3GPP TR 38.885 V1.0.0: Study on Vehicle-to-Everything

- R3-194739, “Summary of Offline Discussion: List of open issues related to F1 for support of NR sidelink resource mode 1 and mode 2”, LG Electronics

- 3GPP TR 23.786 V1.1.0: Study on architecture enhancements for EPS and 5G System to support advanced V2X services