3GPP TSG-RAN3 Meeting #105bis	R3-195562

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for MDT BL CR for TS 38.463): L2 measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order to get the whole RAN part of UL and DL delay, the CU-CP sends the measurement period to the CU-UP, and the CU-UP sends the delay measurement results in CU-UP to the CU-CP. The CU-CP sends the whole RAN part of UL and DL delay to CU-UP. CU-UP sends the results to the UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the UL and DL delay period in the Bearer Context Setup, Bearer Context Modification Request message. - Introduce one new Class 2 message Measurement Report. The CU-UP sends the delay measurement results measured by the CU-UP to the CU-CP via this new message. - Introduce one new Class 2 message Measurement Results Information. The CU-CP sends the whole RAN part of delay measurement results to the CU-UP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network cannot get the whole RAN part of UL and DL delay and the CN will not get the RAN part of delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1,8.3.1,8.3.2,8.3.x(new), 8.3.y(new),9.2.2.xx(new), 9.2.2.xy(new), 9.3.3.2,9.3.3.11,9.3.1.xx1(new),9.4.3,9.4.4,9.4.5,9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

# 8	E1AP procedures

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| gNB-CU-UP E1 Setup | GNB-CU-UP E1 SETUP REQUEST | GNB-CU-UP E1 SETUP RESPONSE | GNB-CU-UP E1 SETUP FAILURE |
| gNB-CU-CP E1 Setup | GNB-CU-CP E1 SETUP REQUEST | GNB-CU-CP E1 SETUP RESPONSE | GNB-CU-CP E1 SETUP FAILURE |
| gNB-CU-UP Configuration Update | GNB-CU-UP CONFIGURATION UPDATE | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-UP CONFIGURATION UPDATE FAILURE |
| gNB-CU-CP Configuration Update | GNB-CU-CP CONFIGURATION UPDATE | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-CP CONFIGURATION UPDATE FAILURE |
| E1 Release | E1 RELEASE REQUEST | E1 RELEASE RESPONSE |  |
| Bearer Context Setup | BEARER CONTEXT SETUP REQUEST | BEARER CONTEXT SETUP RESPONSE | BEARER CONTEXT SETUP FAILURE |
| Bearer Context Modification (gNB-CU-CP initiated) | BEARER CONTEXT MODIFICATION REQUEST | BEARER CONTEXT MODIFICATION RESPONSE | BEARER CONTEXT MODIFICATION FAILURE |
| Bearer Context Modification Required (gNB-CU-UP initiated) | BEARER CONTEXT MODIFICATION REQUIRED | BEARER CONTEXT MODIFICATION CONFIRM |  |
| Bearer Context Release (gNB-CU-CP initiated) | BEARER CONTEXT RELEASE COMMAND | BEARER CONTEXT RELEASE COMPLETE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Bearer Context Release Request (gNB-CU-UP initiated) | BEARER CONTEXT RELEASE REQUEST |
| Bearer Context Inactivity Notification | BEARER CONTEXT INACTIVITY NOTIFICATION |
| DL Data Notification | DL DATA NOTIFICATION |
| UL Data Notification | UL DATA NOTIFICATION |
| Data Usage Report | DATA USAGE REPORT |
| gNB-CU-UP Counter Check | GNB-CU-UP COUNTER CHECK |
| gNB-CU-UP Status Indication | GNB-CU-UP STATUS INDICATION |
| MR-DC Data Usage Report | MR-DC DATA USAGE REPORT |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Results Information | MEASUREMENT RESULTS INFORMATION |

Next Change

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT SETUP RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	For each established PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each established PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each established DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each established DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Existing Allocated S1 DL UP Transport Layer Information IE or the Existing Allocated NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may re-use the indicated resources already allocated for this bearer context. If the gNB-CU-UP decides to re-use the indicated resources, it shall include the S1 DL UP Unchanged IE or the NG DL UP Unchanged IE in the BEARER CONTEXT SETUP RESPONSE message.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [20].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message.

If the DL UP Parameters IE is contained in the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall configure the corresponding information.

If the Security Indication IE includes Maximum Integrity Protected Data Rate IE in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP node shall store the received Maximum Integrity Protected Data Rate IE and use it to enforce the traffic corresponding to this IE for the concerned PDU Session and concerned UE, as specified in TS 23.501 [20].

If the Security Indication IE indicates security policy as “preferred” in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the Security Result IE in the BEARER CONTEXT SETUP RESPONSE message.

If the Security Indication IE indicates security policy as “required” in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the Security Indication IE indicates security policy as “not needed” in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall not perform user plane integrity protection nor perform ciphering for the concerned PDU Session.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall use this value when enforcing the maximum integrity protected data rate for the UE.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-UP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path.

If the PDCP SN Status Information IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the QoS Flow Mapping Indication IE is contained in the QoS Flows Information To Be Setup IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take it into account as specified in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

If Delay Measurement Period IE is contained in the MeasurementConfiguration IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store this information and use it for delay measurement of the DRB.

Next Change

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully modified shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be modified shall be included in the PDU Session Resource Failed To Modify List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Security Information IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Aggregate Maximum Bit Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the E-UTRAN QoS IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDCP SN Status Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the UL COUNT Value IE and the DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP SN Status Information IE is contained in the DRB To Setup List IE or the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the DL UP Parameters IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Cell Group To Add IE or the Cell Group To Modify IE or the Cell Group To Remove IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall add or modify or remove the corresponding cell group.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501 [20].

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the SDAP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Flow Mapping Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-CP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION REQUEST message, and the gNB-CU-UP shall also include two UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the Bearer Context Modification (gNB-CU-CP initiated) procedure, i.e. only one UP Transport Layer Information per Cell Group ID is present in DL UP Parameters IE for the concerned DRB, then the gNB-CU-UP shall consider that PDCP duplication is deconfigured for this DRB. If such Bearer Context Modification (gNB-CU-CP initiated) procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the New UL TNL Information Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the new UP Transport Layer Information in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Security Indication IE indicates security policy as “preferred” in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the Security Result IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Security Indication IE indicates security policy as “required” in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

If the Security Indication IE indicates security policy as “not needed” in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall not perform user plane integrity protection nor perform ciphering for the concerned PDU Session.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

If the QoS Flow Mapping Indication IE is contained in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Data Discard Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to “Required”, the gNB-CU-UP shall consider that a RAN Paging Failure occurred for that UE. The gNB-CU-UP shall discard the user plane data for that UE and consider that the bearer context is still suspended.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take it into account for each DRB, as specified in TS 28.552 [22].

If the DRB QoS IE is contained within the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account for each DRB, as specifed in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message including Activity Notification Level IE and its value does not match the current bearer context, the gNB-CU-UP shall ignore the Activity Notification Level IE and also the requested modification of inactivity timer.

If Delay Measurement Period IE is contained in the MeasurementConfiguration IE within the DRB To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store this information and use it for delay measurement of the DRB.

If Delay Measurement Period IE is contained in the MeasurementConfiguration IE within the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store this information and use it for delay measurement of the DRB.

Next Change

### 8.3.x	Measurement Report

#### 8.3.x.1	General

This procedure is initiated by the gNB-CU-UP to report the measurement results.

The procedure uses UE-associated signalling.

#### 8.3.x.2	Successful Operation

Figure 8.3.x.2-1: Measurement Report procedure. Successful operation.

The gNB-CU-UP initiates the procedure by sending a MEASUREMENT REPORT message.

#### 8.3.x.3	Abnormal Conditions

Not applicable.

### 8.3.y	Measurement Results Information

#### 8.3.y.1	General

This procedure is initiated by the gNB-CU-CP to inform the measurement results calculated by the gNB-CU-CP.

The procedure uses UE-associated signalling.

#### 8.3.y.2	Successful Operation

Figure 8.3.y.2-1: Measurement Results Information procedure. Successful operation.

The gNB-CU-CP initiates the procedure by sending a MEASUREMENT RESULTS INFORMATION message.

#### 8.3.y.3	Abnormal Conditions

Not applicable.

Next Change

#### 9.2.2.xx	MEASUREMENT REPORT

This message is sent by the gNB-CU-UP to provide the measurement result calculated by the gNB-CU-UP to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Measurement Results List |  | 1 |  |  | YES | reject |
| >DRB Measurement Results Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >> UL Delay Measurement Result | O |  | FFS |  | - |  |
| >> DL Delay Measurement Result | O |  | FFS |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

#### 9.2.2.xy	MEASUREMENT RESULTS INFORMATION

This message is sent by the gNB-CU-CP to provide the measurement result calculated by the gNB-CU-CP .

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Measurement Results Information List |  | 1 |  |  | YES | reject |
| >DRB Measurement Results Information Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >> RAN part of UL Delay Result | O |  | FFS |  | - |  |
| >> RAN part of DL Delay Result | O |  | FFS |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

Next Change

### 9.3.3	Container and List IE definitions

#### 9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when at least one Non-GBR QoS Flows is being setup. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Existing Allocated NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >Network Instance | O |  | 9.3.1.62 | Shall be ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB. | YES | ignore |
| >>>Measurement Configuration | O |  | 9.3.1.xx1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next Change

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Indication | O |  | 9.3.1.23 | This IE is not used in this release. | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >PDU Session Data Forwarding Information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 | Shall be ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>Measurement Configuration | O |  | 9.3.1.xx1 |  | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>DRB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | This IE is included to be used for indicating that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. It shall only contain UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side. | YES | reject |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>Measurement Configuration | O |  | 9.3.1.xx1 |  | YES | ignore |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

Next Change

#### 9.3.1.xx1	Measurement Configuration

The IE contains the measurement configuration for the gNB-CU-UP to measure.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Delay Measurement Period | M |  | FFS |  |

Next Change

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

ResetAcknowledge,

ErrorIndication,

GNB-CU-UP-E1SetupRequest,

GNB-CU-UP-E1SetupResponse,

GNB-CU-UP-E1SetupFailure,

GNB-CU-CP-E1SetupRequest,

GNB-CU-CP-E1SetupResponse,

GNB-CU-CP-E1SetupFailure,

GNB-CU-UP-ConfigurationUpdate,

GNB-CU-UP-ConfigurationUpdateAcknowledge,

GNB-CU-UP-ConfigurationUpdateFailure,

GNB-CU-CP-ConfigurationUpdate,

GNB-CU-CP-ConfigurationUpdateAcknowledge,

GNB-CU-CP-ConfigurationUpdateFailure,

BearerContextSetupRequest,

BearerContextSetupResponse,

BearerContextSetupFailure,

BearerContextModificationRequest,

BearerContextModificationResponse,

BearerContextModificationFailure,

BearerContextModificationRequired,

BearerContextModificationConfirm,

BearerContextReleaseCommand,

BearerContextReleaseComplete,

BearerContextReleaseRequest,

BearerContextInactivityNotification,

DLDataNotification,

ULDataNotification,

DataUsageReport,

E1ReleaseRequest,

E1ReleaseResponse,

GNB-CU-UP-CounterCheckRequest,

GNB-CU-UP-StatusIndication,

MRDC-DataUsageReport,

PrivateMessage,

MeasurementReport,

MeasurementResultsInformation

FROM E1AP-PDU-Contents

id-reset,

id-errorIndication,

id-gNB-CU-UP-E1Setup,

id-gNB-CU-CP-E1Setup,

id-gNB-CU-UP-ConfigurationUpdate,

id-gNB-CU-CP-ConfigurationUpdate,

id-e1Release,

id-bearerContextSetup,

id-bearerContextModification,

id-bearerContextModificationRequired,

id-bearerContextRelease,

id-bearerContextReleaseRequest,

id-bearerContextInactivityNotification,

id-dLDataNotification,

id-uLDataNotification,

id-dataUsageReport,

id-gNB-CU-UP-CounterCheck,

id-gNB-CU-UP-StatusIndication,

id-mRDC-DataUsageReport,

id-privateMessage,

id-measurementReport,

id-measurementResultsInformation

FROM E1AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

E1AP-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome		UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

gNB-CU-UP-E1Setup							|

gNB-CU-CP-E1Setup							|

gNB-CU-UP-ConfigurationUpdate				|

gNB-CU-CP-ConfigurationUpdate				|

e1Release									|

bearerContextSetup							|

bearerContextModification					|

bearerContextModificationRequired			|

bearerContextRelease						,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

bearerContextReleaseRequest				|

bearerContextInactivityNotification		|

dLDataNotification						|

uLDataNotification						|

dataUsageReport							|

gNB-CU-UP-CounterCheck					|

gNB-CU-UP-StatusIndication			|

mRDC-DataUsageReport					|

privateMessage							|

measurementReport						|

measurementResultsInformation,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

gNB-CU-UP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-UP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-UP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-UP-E1Setup

CRITICALITY				reject

}

gNB-CU-CP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-CP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-CP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-CP-E1Setup

CRITICALITY				reject

}

gNB-CU-UP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-UP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-UP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-UP-ConfigurationUpdate

CRITICALITY				reject

}

gNB-CU-CP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-CP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-CP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-CP-ConfigurationUpdate

CRITICALITY				reject

}

e1Release E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E1ReleaseRequest

SUCCESSFUL OUTCOME		E1ReleaseResponse

PROCEDURE CODE			id-e1Release

CRITICALITY				reject

}

bearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextSetupRequest

SUCCESSFUL OUTCOME		BearerContextSetupResponse

UNSUCCESSFUL OUTCOME	BearerContextSetupFailure

PROCEDURE CODE			id-bearerContextSetup

CRITICALITY				reject

}

bearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequest

SUCCESSFUL OUTCOME		BearerContextModificationResponse

UNSUCCESSFUL OUTCOME	BearerContextModificationFailure

PROCEDURE CODE			id-bearerContextModification

CRITICALITY				reject

}

bearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequired

SUCCESSFUL OUTCOME		BearerContextModificationConfirm

PROCEDURE CODE			id-bearerContextModificationRequired

CRITICALITY				reject

}

bearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseCommand

SUCCESSFUL OUTCOME		BearerContextReleaseComplete

PROCEDURE CODE			id-bearerContextRelease

CRITICALITY				reject

}

bearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseRequest

PROCEDURE CODE			id-bearerContextReleaseRequest

CRITICALITY				ignore

}

bearerContextInactivityNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextInactivityNotification

PROCEDURE CODE			id-bearerContextInactivityNotification

CRITICALITY				ignore

}

dLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLDataNotification

PROCEDURE CODE			id-dLDataNotification

CRITICALITY				ignore

}

measurementReport E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementReport

PROCEDURE CODE			id-measurementReport

CRITICALITY				ignore

}

measurementResultsInformation E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementResultsInformation

PROCEDURE CODE			id-measurementResultsInformation

CRITICALITY				ignore

}

uLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULDataNotification

PROCEDURE CODE			id-uLDataNotification

CRITICALITY				ignore

}

dataUsageReport E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataUsageReport

PROCEDURE CODE			id-dataUsageReport

CRITICALITY				ignore

}

gNB-CU-UP-CounterCheck E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-CounterCheckRequest

PROCEDURE CODE			id-gNB-CU-UP-CounterCheck

CRITICALITY				ignore

}

gNB-CU-UP-StatusIndication 	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-StatusIndication

PROCEDURE CODE			id-gNB-CU-UP-StatusIndication

CRITICALITY			ignore

}

privateMessage E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

mRDC-DataUsageReport	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MRDC-DataUsageReport

PROCEDURE CODE			id-mRDC-DataUsageReport

CRITICALITY				ignore

}

END

-- ASN1STOP

Next Change

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Measurement-Results-List,

DRB-Measurement-Results-Information-List,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-DRB-Measurement-Results-List,

id-DRB-Measurement-Results-Information-List,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset

FROM E1AP-Constants;

Next Change

-- **************************************************************

--

#### -- MR-DC DATA USAGE REPORT

--

-- **************************************************************

MRDC-DataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MRDC-DataUsageReportIEs } },

...

}

MRDC-DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-PDU-Session-Resource-Data-Usage-List	CRITICALITY ignore		TYPE PDU-Session-Resource-Data-Usage-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REPORT

--

-- **************************************************************

MeasurementReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MeasurementReportIEs } },

...

}

MeasurementReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-DRB-Measurement-Reuslts-List	CRITICALITY ignore		TYPE DRB-Measurement-Reuslts-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT RESULTS INFORMATIONREPORT

--

-- **************************************************************

MeasurementResultsInformation ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MeasurementResultsInformationIEs } },

...

}

MeasurementResultsInformationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-DRB-Measurement-Reuslts-Information-List	CRITICALITY ignore		TYPE DRB-Measurement-Reuslts-Information-List	PRESENCE mandatory},

...

}

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-MeasurementConfiguration,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

Next Change

#### -- D

----------------skip  the unchanged text-------------

DRB-ID	::=	INTEGER (1..32, ...)

DRB-Measurement-Results-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Measurement-Results-Item

DRB-Measurement-Results-Item	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-Delay-Measurement-Result								Delay-Result							OPTIONAL,

dL-Delay-Measurement-Result								Delay-Result							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Measurement-Results-Item-ExtIEs } }	OPTIONAL,

...

}

DRB-Measurement-Results-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Measurement-Results-Information-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Measurement-Results-Information-Item

DRB-Measurement-Results-Information-Item	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-RAN-Part-Delay-Result								Delay-Result							OPTIONAL,

dL-RAN-Part-Delay-Result								Delay-Result							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Measurement-Results-Information-Item-ExtIEs } }	OPTIONAL,

...

}

DRB-Measurement-Results-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-EUTRAN

DRB-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-NG-RAN

DRB-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-UP-Transport-Parameters					UP-Parameters							OPTIONAL,

pDCP-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

flow-Setup-List								QoS-Flow-List							OPTIONAL,

flow-Failed-List							QoS-Flow-Failed-List					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Next Change

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional}|

{ID id-MeasurementConfiguration	CRITICALITY ignore	EXTENSION MeasurementConfiguration	PRESENCE optional},

...

}

DRB-To-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-NG-RAN

DRB-To-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer					OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional}|

{ID id-MeasurementConfiguration	CRITICALITY ignore	EXTENSION MeasurementConfiguration	PRESENCE optional},

...

}

Next Change

DRB-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-NG-RAN

DRB-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration						OPTIONAL,

pDCP-Configuration							PDCP-Configuration						OPTIONAL,

dRB-Data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request							PDCP-SN-Status-Request						OPTIONAL,

pdcp-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

cell-Group-To-Add							Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify						Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove						Cell-Group-Information					OPTIONAL,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List				OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject	EXTENSION QoS-Flow-List	PRESENCE optional}|

{ID id-DRB-QoS	CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters	PRESENCE optional}|

{ID id-MeasurementConfiguration	CRITICALITY ignore	EXTENSION MeasurementConfiguration	PRESENCE optional},

...

}

Next Change

#### -- M

MaxDataBurstVolume  ::= INTEGER (0..4095, ...)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MaxPacketLossRate ::= INTEGER (0..1000, ...)

MeasurementConfiguration ::= SEQUENCE {

DelayMeasurementPeriod				DelayMeasurementPeriod		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MeasurementConfiguration-ExtIEs} } OPTIONAL,

...

}

MeasurementConfiguration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Next Change

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired										ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS															ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID													ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance											ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication									ProtocolIE-ID ::= 80

id-DRB-Measurement-Results-List								ProtocolIE-ID ::= xxx

id-DRB-Measurement-Results-Information-List					ProtocolIE-ID ::= yyy

id-MeasurementConfiguration									ProtocolIE-ID ::= zzz

END

-- ASN1STOP

End Change