3GPP TSG-RAN3 Meeting #105bis	R3-195573

Chongqing, China, 14-18 October 2019

Title: 	(TP for NR_Mob_enh BL CR for TS 38.423): Handover modification for CHO

Source: 	Huawei

Agenda item:	15.3.1

Document for:	pCR

1. Introduction

In RAN3#105 meeting [1], some details for CHO cancelling are agreed as below:

CHO Cancel procedure from the target node shall not be used in legacy HO

Both source and target nodes are allowed to initiate per-target cell CHO cancel; target node is only allowed to cancel a prepared CHO

But the FFSs left in RAN3#104 meeting [2] are still not concluded:

Editor’s note: FFS whether we allow to modify the prepared CHO resources and how (re-using the existing HO  Preparation or new procedure?).

Editor’s note: FFS how to handle the source-initiated RRC reconfiguration for an accepted but ongoing CHO.

In this paper, we would discuss the details for CHO configuration modification.

2. Discussion

The first FFS is:

Editor’s note: FFS whether we allow to modify the prepared CHO resources and how (re-using the existing HO  Preparation or new procedure?).

It was agreed that the source node initiates the procedure by sending the HANDOVER REQUEST message to the target node to request the preparation of resources for a conditional handover. After the CHO preparation procedure between the source node and each candidate target node is finished, the UE would be configured with CHO configurations for those prepared candidate cells.

However, during the evaluation of the CHO execution conditions by the UE and before one target cell is matched the condition, the source node may want to modify the prepared CHO resources at the candidate target nodes. For example, the source node may want to add/modify/release the established DRBs, which may cause mirror update at the prepared target node.

Proposal 1: The source node should be able to modify the prepared CHO resources at the target node.

If the source node wants to modify the prepared CHO configurations, it may inform the candidate target cell about the updated configurations by reusing the existing HO Preparation procedure, or a new procedure. The drawback of using existing HO request message is that the unchanged mandatory IEs shall be transferred in the message in case of CHO resource modification. This will increase the processing complexity at the candidate target side due to the update verification for the mandatory IEs.

Using a new medication procedure makes the specification more readable.

Therefore, it is preferred to define a new procedure to modify the prepared CHO configurations, e.g. Conditional Handover Modification procedure.

Proposal 2: To define a new class 1 procedure to modify the prepared CHO resources at the target node.

3. Conclusion

In this paper, we mainly discuss about CHO configuration modification and we have the following proposals:

Proposal 1: The source node should be able to modify the prepared CHO resources at the target node.

Proposal 2: To define a new class 1 procedure to modify the prepared CHO resources at the target node.

The TP for NR_Mob_enh BL CR for TS 38.423 is provided in ANNEX.

The TP for LTE_feMob BL CR for TS 36.423 is provided in [3].

4. Reference

[1] Chairman notes, “Draft Report of 3GPP TSG RAN WG2 meeting #105”, Slovenia.

[2] Chairman notes, “Draft Report of 3GPP TSG RAN WG2 meeting #104”, USA.

[3] R3-195574	(TP for LTE_feMob BL CR for TS 36.423): Handover modification for CHO.

ANNEX - TP for NR_Mob_enh BL CR for TS 38.423

Start of changes

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Conditional Handover Modification | CONDITIONAL HANDOVER MODIFICATION REQUEST | CONDITIONAL HANDOVER MODIFICATION ACKNOWLEDGE | CONDITIONAL HANDOVER MODIFICATION FAILURE |

Next change

### 8.2.Y	Conditional Handover Modification

#### 8.2.Y.1	General

The Conditional Handover Modification procedure is used to inform the peer NG-RAN node to modify the prepared CHO configurations.

The procedure uses UE-associated signalling.

#### 8.2.Y.2	Successful Operation

Figure 8.2.Y.2-1: Conditional Handover Modification, successful operation

The source NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER MODIFICATION  REQUEST message to the candidate target NG-RAN node to inform to modify the prepared CHO configurations.

When the prepared CHO configurations are modified successfully, the candidate target NG-RAN node would send the CONDITIONAL HANDOVER MODIFICATION ACKNOWLEDGE message to the source NG-RAN node.

#### 8.2.Y.3	Unsuccessful Operation

Figure 8.2.Y.3-1: Conditional Handover Modification, unsuccessful operation

If the candidate target NG-RAN node does not admit the request for CHO configurations modification, or a failure occurs during the CHO configurations modification, the candidate target NG-RAN node shall send the CONDITIIONAL HANDOVER MODIFICATION FAILURE message to the source NG-RAN node.

8.2.Y.4	Abnormal Conditions

Next change

9.1.1.x	HANDOVER MODIFICATION

This message is sent by the source NG-RAN node to the target NG-RAN node to modify the prepared resources for a conditional handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | ignore |
| PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 |  | – |  |
| PDU Session Resources To Be Modify List |  | 1 | 9.2.1.x |  | – |  |
| PDU Session Resources To Be Release List |  | 1 | 9.2.1.y |  |  |  |

Next change

9.2.1.x	PDU Session Resources To Be Modify List

This IE contains PDU session resource related information to be modified at the target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources To Be Modify List |  | 1 |  |  | – |  |
| >PDU Session Resources To Be Modify Item |  | 1 .. <maxnoof PDU sessions > |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | O |  | 9.2.3.21 |  | – |  |
| >>PDU Session Resource Aggregate Maximum Bitrate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 | This IE shall be present when at least one Non-GBR QoS Flow has been setup. | – |  |
| >>UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| >>Additional UL NG-U UP TNL Information at UPF List | O |  | Additional UP Transport Layer Information 9.2.1.32 | Additional UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | YES | ignore |
| >>Source DL NG-U TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | Indicates the possibility to keep the NG-U GTP-U tunnel termination point at the target NG-RAN node. | – |  |
| >>Security Indication | O |  | 9.2.3.52 |  | – |  |
| >>PDU Session Type | O |  | 9.2.3.19 |  | – |  |
| >>Network Instance | O |  | 9.2.3.85 | This IE shall be ignored if the Common Network Instance IE is present. | – |  |
| >>QoS Flows To Be Add or Modify List |  | 1 |  |  | – |  |
| >>>QoS Flows To Be Modify Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>>>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 |  | – |  |
| >>>>E-RAB ID | O |  | INTEGER (0..15, …) |  | – |  |
| >>Qos Flows To be Release List |  | 1 |  |  |  |  |
| >>>QoS Flow To be Release Item |  | 1..<maxnoofQoSFlows> |  |  |  |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>>>Cause | M |  | 9.3.1.2 |  |  |  |
| >>Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 |  | – |  |
| >> Common Network Instance | O |  | 9.2.3.92 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

9.2.1.y	PDU Session Resources To Be Modify List

This IE contains PDU session resource related information to be release at the target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resources to Release List |  | 1 |  |  | YES | reject |
| >PDU Session Resources to Release Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

End of change