3GPP TSG-RAN WG3 Meeting #105bis	R3-195677

Chongqing, China, October 14th – 18th, 2019

Agenda item:		22.2

Source:	Intel Corporation

Title:	(TP for FS_enh_disagg_gNB for TR 38.823): DDDS enhancements for retransmitted packets

Document for:		Other

# 1		Introduction

This contribution proposes TP for TR 38.823 based on enhancements proposed for DDDS as discussed in [1].

# 2		Reference

[1] R3-195676, “DDDS enhancements for retransmitted packets”, Intel Corporation

# 3		TP for TR 38.823

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 5.2	Possible Solutions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 5.2.2	Solution for Scenario 2

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 5.2.2.2	Highest PDCP SN approach with up to which NR-U SN

##### 5.5.2.2.1	Motivation

In Rel-15, DDDS for retransmitted packets relies on reporting PDCP SN corresponding to the highest in-sequence NR-U SN. This was decided mainly due to backward compatibility and inter-operability.

For retransmitted packets, “retransmission” is flagged by CU. This is for prioritization at DU (i.e. re-ordering based on PDCP SN). Although not mandated, DU’s re-ordering is critical to minimize service interruptions.

However, the current Rel-15 solution is suboptimal when DU performs re-ordering. For example, suppose that DU re-orders lower PDCP SN packets after successfully delivering SN #103 as in Figure 5.2.2.2.1-1 (assuming that SN #1, #2,… arrives at DU before transmitting SN #104):

Figure 5.2.2.2.1-1. DU’s re-ordering makes the current Rel-15 solution sub-optimal

- Reporting a PDCP SN corresponding to the highest in-sequence NR-U SN (current Rel-15) will be fixed to #103, until a next NR-U SN (i.e. corresponds to #104) is successfully delivered.

- In other words, there won’t be any new feedback until all the retransmitted packets with lower PDCP SN than #104 (and higher NR-U SN than “B”) are delivered successfully.

- If they are in large volume (e.g. the Centralized Retransmission scheme), this could result in a suboptimal behavior for CU, because those large number of already delivered retransmitted packets cannot be freed up in a timely fashion.

In summary, the main issue with the current Rel-15 solution is as follows:

- DU’s reordering based on PDCP SN could result in earlier NR-U SN packets (with higher PDCP SN) to be kept de-prioritized.

- Such de-prioritization could lead DDDS feedback sluggish and inefficient buffer management for CU.

##### 5.5.2.2.2	Solution Description

To mitigate this problem, the following solution is proposed for DDDS in TS 38.425 [XX]:

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 |  | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  |  | Highest Transmitted NR PDCP SN Ind | Highest Delivered NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Highest Retransmitted End NR-U SN Ind | Highest Retransmitted NR PDCP SN Ind |  | Highest Delivered Retransmitted End NR-U SN Ind |  | Highest Delivered Retransmitted NR PDCP SN Ind |  | Data rate Ind. | Retransmitted NR PDCP SN Ind | Delivered Retransmitted NR PDCP SN Ind | Cause Report | 1 |
| Desired buffer size for the data radio bearer |  |  |  |  |  |  |  |  |  |  | 4 |
| Desired Data Rate |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  |  | 0 or (6* Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Cause Value |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Highest Successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| End of NR-U Sequence Number for Highest Successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Highest Retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| End of NR-U Sequence Number for Highest Retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

5.5.3.A	Highest Delivered Retransmitted NR PDCP SN Ind

Description: This parameter indicates the presence of highest successfully delivered retransmitted PDCP Sequence Number.

Value range: {0= Highest Successfully delivered retransmitted NR PDCP Sequence Number not present, 1= Highest Successfully delivered retransmitted NR PDCP Sequence Number}.

Field length: 1 bit.

5.5.3.B	Highest Delivered Retransmitted End NR-U SN Ind

Description: This parameter indicates the presence of End of NR-U Sequence Number for Highest successfully delivered retransmitted NR PDCP Sequence Number.

Value range: {0= End of NR-U Sequence Number for Highest Successfully delivered retransmitted NR PDCP Sequence Number not present, 1= End of NR-U Sequence Number for Highest Successfully delivered retransmitted NR PDCP Sequence Number }.

Field length: 1 bit.

5.5.3.C	Highest Retransmitted NR PDCP SN Ind

Description: This parameter indicates the presence of highest retransmitted PDCP Sequence Number.

Value range: {0= Highest Retransmitted NR PDCP Sequence Number not present, 1= Highest Retransmitted NR PDCP Sequence Number present }.

Field length: 1 bit.

5.5.3.D	Highest Retransmitted End NR-U SN Ind

Description: This parameter indicates the presence of End of NR-U Sequence Number for Highest retransmitted NR PDCP Sequence Number.

Value range: {0= End of NR-U Sequence Number for Highest Successfully delivered retransmitted NR PDCP Sequence Number not present, 1= End of NR-U Sequence Number for Highest Successfully delivered retransmitted NR PDCP Sequence Number }.

Field length: 1 bit.

5.5.3.E	Highest Successfully delivered retransmitted NR PDCP Sequence Number

Description: This parameter indicates highest successfully delivered NR PDCP SN in-sequence among retransmission NR PDCP PDUs.

Value range: {0..218-1}.

Field length: 3 octets.

5.5.3.F	End of NR-U Sequence Number for Highest successfully delivered retransmitted NR PDCP Sequence Number

Description: This parameter indicates a NR-U sequence number up to which the reported Highest successfully delivered retransmitted NR PDCP Sequence Numbe should be applied for retransmission NR PDCP PDUs.

Value range: {0..218-1}.

Field length: 3 octets.

5.5.3.G	Highest retransmitted NR PDCP Sequence Number

Description: This parameter indicates highest transmitted NR PDCP SN in-sequence among retransmission NR PDCP PDUs.

Value range: {0..218-1}.

Field length: 3 octets.

5.5.3.H	End of NR-U Sequence Number for Highest successfully delivered retransmitted NR PDCP Sequence Number

Description: This parameter indicates a NR-U sequence number up to which the reported Highest retransmitted NR PDCP Sequence Numbe should be applied for retransmission NR PDCP PDUs.

Value range: {0..218-1}.

Field length: 3 octets.

##### 5.5.2.2.3	Solution Principles and Evaluations

The proposed solution has the following principles:

- Relying on “highest” PDCP SN feedback can embrace the advantage of freeing up CU’s buffer for what has been successfully delivered based on PDCP SN.

- By giving “up to which NR-U SN” retransmitted packets have been already delivered to the UE and can be freed up by the reported “highest” SN, CU can confine a range of retransmitted packets for which the reported “highest” SN should be applied.

- The proposed solution is a generalized version of the current Rel-15 solution, where “up to which NR-U SN” is always fixed to that of the PDCP SN reported, thus cannot be worse than Rel-15.

For example in Figure 5.2.2.2.1-1’s scenario, CU is now able to know that “OK, among all the retransmitted packets whose PDCP SN lower than or equal to #103, only those up to NR-U SN “G” are successfully delivered”, and thus free up #1, #2, #3, #4, #5, and #103 from its buffer as exactly intended:

Figure 5.2.2.2.3-1. Proposed solution applied to the Figure 5.2.2.2.1-1’s scenario