3GPP TSG-RAN WG3 Meeting #105bis	R3-195706

# Chongqing, China, 14th - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.401 | CR | 0095 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Network sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the initial registration signalling flow, in step 7, the RRC-Container with the RRC message shall include the RRC message received both in step 3 and step 5 in the case of RRC message re-direction. In TS38.473, it was described that “If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the DL RRC MESSAGE TRANSFER may contain the PLMN Assistance Info for Network Sharing IE, if available at the gNB-CU and may use it as specified in TS 38.401 [4]”. However, there is no description/clue in stage2 how to use the PLMN Assistance Info for Network Sharing IE. Another issue is that for the RRC Connection Reestablishment case, the principle that paralle initiating procedures towards multiple gNB-CUs should not be precluded as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification text is added in TS38.401. Some editorial updates. Impact assessment towards the previous version of the specification: This CR has isolated impact on Network sharing from functional and procedural points of view. This CR has no impact on ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network sharing can not be supported completely in R15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2, 8.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.11	Support of Network Sharing with multiple cell-ID broadcast

8.11.1	General

This section describes necessary additions as compared to the case where network sharing with multiple cell-ID broadcast is not applied.

The signalling flows in the subsequent sections assuming 2 sharing operators, A and B. The F1-C signalling transport deployment used is indicated within the subsequent sections.

8.11.2	Initial Registration – separate PLMN signalling

The signalling flow for Initial Registration for network sharing with multiple cell-ID broadcast with separate per-PLMN signalling is shown in Figure 8.11.2-1.

In this example message flow

-	the UE is assumed to not provide an ue-Identity from which the DU is able to deduce the PLMN ID.

-	each F1-C interface instance uses a separate signalling transport or share signalling transport with other F1-C interface instances.

-	the gNB-DUA/B entity shown in Figure 8.11.2-1 is a simplified representation of the gNB-DUA of PLMN A, the gNB DUB of PLMN B and respective radio resources of the shared cell.

Figure 8.11.2-1: UE Initial Access procedure and network sharing with multiple cell-ID broadcast

NOTE:	Steps 1-5 are defined in clause 8.1. Note, that the selectedPLMN-Identity is provided in step 5.

6.	The gNB-DUA sends the F1AP UE CONTEXT RELEASE REQUEST message to the gNB-CUA. including a Cause set to “PLMN not served by the CU”.

7.	The gNB-DUB sends the F1AP INITIAL UL RRC MESSAGE to the gNB-CUB. including the NR CGI associated with PLMNB, the C-RNTI indicated by the gNB-DUA at step 2, and the RRC-Container with the RRC message received in step 3 and step 5.

8. The gNB-CUA triggers the F1AP UE Context Release procedure.

NOTE: Initiating procedures from gNB-DUA towards gNB-CUA and from gNB-DUB to gNB-CUB in parallel is not precluded.

8.11.3	RRC Connection Reestablishment – separate PLMN signalling

The signalling flow for RRC Connection Reestablishment for network sharing with multiple cell-ID broadcast with separate per-PLMN signalling is shown in Figure 8.x.3-1.

In this example message flow

-	each F1-C/Xn-C interface instance uses either a separate signalling transport or a share signalling transport with other interface instances.

-	the New gNB-DUA/B entity shown in Figure 8.x.3-1 is a simplified representation of the New gNB-DUA of PLMN A, the New gNB DUB of PLMN B and respective radio resources of the shared cell.

Figure 8.x.3-1: RRC Connection Reestablishment and network sharing with multiple cell-ID broadcast

1.	The UE sends the RRCReestablishmentRequest.

2A-5A.	Depicts the case where the UE context could not be retrieved by the new gNB-CUA. In step 2A, the NR CGI associated to PLMNA is indicated. In step 5A, the gNB-CUA would prepare the possibility to revert back to normal RRC Connection Establishment, indicating that the UE Context was not retrievable and may include the re-directed RRC message as received in step 1. After step 5A, the gNB-DUA may redirect the UE towards the PLMN indicated in DL RRC message transfer, if the PLMN assistance information is provided by the gNB-CUA.The New gNB-DUA is supposed to trigger the release the UE-associated signalling connection (not shown).

2B-5B.	Depicts the case where the UE context was retrievable by the New gNB-CUB. In step 2B, the NR CGI associated to PLMNB is indicated. Step 2B also includes the C-RNTI allocated at reception of step 1.

6-8.	The RRC Connection Reestablishment continues with the New gNB-CUB.

NOTE:	If all gNB-CUs indicate that the UE context is not retrievable, the RRC connection reestablishment falls back to RRC Connection setup, as described in section 8.x.2.

NOTE: Initiating procedures from gNB-DUA towards gNB-CUA and from gNB-DUB to gNB-CUB in parallel is not precluded.