3GPP TSG RAN WG3#105bis Meeting                        R3-195707

# Chongqing, China, 14th – 18th Oct 2018

Agenda item:    9.3.4

Source: 	ZTE

Title: 	Discussion on SCell failure in CU-DU deployment

Document for:	Discussion and Approval

Introduction

According to TS38.470, the SCell management was described in the F1 UE context management function:

With this function, the gNB-CU requests the setup of the SCell(s) at the gNB-DU side, and the gNB-DU accepts all, some or none of the SCell(s) and replies to the gNB-CU. The gNB-CU requests the removal of the SCell(s) for the UE.

In this paper, the SCell failure in DU side in both EN-DC and standalone CA will be discussed.

Discussion

Nowadays only gNB-CU can trigger the setup/removal of the SCell(s), however, there has the case that the (S)gNB-DU detects the SCell failure, e.g., RLC failure, in this case, it should be allowed to remove this SCell by the gNB-DU.

According to current TS38.473, there is no way for the gNB-DU to inform the gNB-CU about the SCell removal via F1 signalling. In order to fix this, here we propose to add the removal SCell list in the UE CONTEXT MODIFICATION REQUIRED message.

Observation 1: In the case of the (S)gNB-DU detects the SCell failure, e.g., RLC failure, it should be allowed to remove this SCell by the gNB-DU.

Proposal 1: Add the removal SCell list in the UE CONTEXT MODIFICATION REQUIRED message from gNB-DU to gNB-CU.

The corresponding stage2 and stage3 CRs are provided in [1][2].

Conclusion

The following proposals are provided:

Proposal 1: Add the removal SCell list in the UE CONTEXT MODIFICATION REQUIRED message from gNB-DU to gNB-CU.

Reference

- R3-195708 SCell failure in CU-DU deployment for TS38.470 ZTE

- R3-195709 SCell failure in CU-DU deployment for TS38.473 ZTE