3GPP TSG-RAN3 Meeting #105bis	R3-195777

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MR-DC for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The support of non-public network is agreed in RP-191563. There is a need to support the NPN for MR-DC connected with 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Public Integrated Network NPN and standalone NPN for MR-DC are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NPN feature in MR-DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 10.2, 10.5, 10.9, 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

CAG	Closed Access Group

CAG ID	Closed Acces Group Identifier

DC	Intra-E-UTRA Dual Connectivity

EN-DC	E-UTRA-NR Dual Connectivity

MCG	Master Cell Group

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NID	Network identifier

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NPN	Non-Public Network

NR-DC	NR-NR Dual Connectivity

PNI-NPN             Public Network Integrated NPN

SCG	Secondary Cell Group

SN	Secondary Node

SNPN	Stand-alone Non-Public Network

Next change

10.2	Secondary Node Addition

<Unchanged Text Omitted>

10.2.2	MR-DC with 5GC

The Secondary Node (SN) Addition procedure is initiated by the MN and is used to establish a UE context at the SN in order to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the initial SCG serving cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). For NPN, this procedure can also be used to provide SCG radio resources by the MN considering the NPN restriction information. Figure 10.2.2-1 shows the SN Addition procedure.

Editor’s note: the naming of NPN restriction information is FFS.

<Unchanged Text Omitted>

Next change

10.5	Secondary Node Change (MN/SN initiated)

<Unchanged Text Omitted>

10.5.2	MR-DC with 5GC

MN initiated SN Change

The MN initiated SN change procedure is used to transfer a UE context from the source SN to a target SN and to change the SCG configuration in UE from one SN to another. For NPN, this procedure can be used to change the SCG configuration considering the NPN restriction information.

Editor’s note: the naming of NPN restriction information is FFS.

The Secondary Node Change procedure always involves signalling over MCG SRB towards the UE.

Next change

10.9	eNB/gNB to Master Node change

<Unchanged Text Omitted>

10.9.2	MR-DC with 5GC

The ng-eNB/gNB to MN change procedure is used to transfer UE context data from a source ng-eNB/gNB to a target MN that adds an SN during the handover. Only the cases where the source node and the target MN belong to the same RAT (i.e. they are both ng-eNBs or both gNBs) are supported. For NPN, this procedure can be used during handover considering the NPN restriction information.

Editor’s note: the naming of NPN restriction information is FFS

Next change

11.1	Roaming and Access Restrictions

The principles for conveying roaming and access restriction info for EN-DC are described in TS 36.300 [2].

For MR-DC with 5GC, SCG (re)selection at the SN is based on roaming and access restriction information in SN. If roaming and access restriction information is not available at the SN, the SN shall consider that there is no restriction for SCG (re)selection. Therefore, MN needs to convey the up-to-date roaming and access restriction information to SN via XnAP messages. For NPN, the SCG (re)selection at the SN is based on the NID for SNPN, or the allowed CAG ID list and CAG-only indicator for PNI-NPN. Therefore, MN needs to convey the up-to-date NPN restriction information to SN via XnAP messages.

Editor’s note: the naming of NPN restriction information is FFS.

End of change