3GPP TSG-RAN3 Meeting #105bis	R3-195783

Chongqing, China, 14-18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0477 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR_IIOT support to TS 38.473 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce enhancements to support TSC QoS flows, which use a Delay Critical GBR resource type and TSC Assistance Information as specified in TS 23.501 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of TSC Assistance Information to the GBR QoS Flow Information IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSC deterministic QoS is not possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.47, 9.3.1.49, 9.3.1.x (new), 9.3.1.y (new), 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Text Proposal for TS 38.473

9.3.1.47	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name |  |  | Presence |  | Range | IE type and reference |  |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QoS Priority Level |  |  | M |  |  | INTEGER (1..127) |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Packet Delay Budget |  |  | M |  |  | 9.3.1.51 |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Packet Error Rate |  |  | M |  |  | 9.3.1.52 |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| 5QI |  |  | O |  |  | INTEGER (0..255,...) |  |  | This IE contains the dynamically assigned 5QI as specified in TS 23.501 [21]. |  | - |  |  |
| Delay Critical |  |  | C-ifGBRflow |  |  | ENUMERATED (delay critical, non-delay critical) |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Averaging Window |  |  | C-ifGBRflow |  |  | 9.3.1.53 |  |  | For details see TS 23.501 [21]. |  | - |  |  |
| Maximum Data Burst Volume |  |  | O |  |  | 9.3.1.54 |  |  | For details see TS 23.501 [21]. This IE shall be included if the Delay Critical IE is set to “delay critical” and shall be ignored otherwise. |  | - |  |  |
| CN Packet Delay Budget | O |  | FFS | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

Next Change

9.3.1.49	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name |  | Presence |  |  | Range | IE type and reference |  |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5QI |  | M |  |  |  | INTEGER (0..255,...) |  |  | This IE contains the standardized or pre-configured 5QI as specified in TS 23.501 [21] |  | - |  |  |
| Priority Level |  | O |  |  |  | INTEGER (1..127) |  |  | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |  | - |  |  |
| Averaging Window |  | O |  |  |  | 9.3.1.53 |  |  | This IE applies to GBR QoS Flows only. For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |  | - |  |  |
| Maximum Data Burst Volume |  | O |  |  |  | 9.3.1.54 |  |  | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. If the 5QI refers to a non-delay critical QoS flow the IE shall be ignored. |  | - |  |  |
| CN Packet Delay Budget | O |  | FFS | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

Next Change

9.3.1.x	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

Editor’s Note:	It is FFS whether to include this IE within the QoS Flow Level QoS Parameters IE, or at the same level as the QoS Flow Level QoS Parameters IE. The IE may be present in case of GBR QoS flows and is ignored otherwise.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.y |  |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.y |  |

9.3.1.y	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | FFS | Periodicity as specified in TS 23.501 [9]. |
| Burst Arrival Time | O |  | FFS | Burst Arrival Time as specified in TS 23.501 [9]. |

Next Change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-IgnorePRACHConfiguration,

id-CNpacketDelayBudget,

id-nonCNpacketDelayBudget,

<Unchanged Text Omitted>

-- the exact value is FFS

BurstArrivalTime ::= INTEGER (0..549755813887,...)

<Unchanged Text Omitted>

Dynamic5QIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CNpacketDelayBudget	CRITICALITY ignore	EXTENSION CNpacketDelayBudget		PRESENCE optional},

...

}

<Unchanged Text Omitted>

NonDynamic5QIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-nonCNpacketDelayBudget	CRITICALITY ignore	EXTENSION NonCNpacketDelayBudget		PRESENCE optional},

...

}

<Unchanged Text Omitted>

#### -- the exact value is FFS

Periodicity ::= INTEGER (0..100000,...)

<Unchanged Text Omitted>

TscAssistanceInfo ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime,

iE-Extensions			ProtocolExtensionContainer { { TscAssistanceInfo-ExtIEs} } OPTIONAL,

...

}

TscAssistanceInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TscTrafficCharacteristics ::= SEQUENCE {

tscAssistanceInfo-dl	TscAssistanceInfo,

tscAssistanceInfo-ul	TscAssistanceInfo,

iE-Extensions			ProtocolExtensionContainer { { TscTrafficCharacteristics-ExtIEs} } OPTIONAL,

...

}

TscTrafficCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged Text Omitted>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-CNpacketDelayBudget								ProtocolIE-ID ::= xxx

id-nonCNpacketDelayBudget							ProtocolIE-ID ::= yyy

End of Text Proposal for TS 38.473