# 3GPP TSG-RAN WG3 #105bis	Tdoc R3-195847

Chongqing, China, 14th – 18th October

Agenda Item:	9.3.4

Source:	Ericsson

Title:	Use of SCTP ports for multiple TNLA.

Document for:	Discussion, Decision

# Introduction

In the previous meeting the use of SCTP ports to establish multiple TNLAs was discussed. In this contribution we will elaborate further on the issue and provide our thoughts and understanding.

# Discussion

In the previous meeting, the discussions on the use of SCTP ports for the establishment of multiple TNLAs, culminated in the following conclusion:

Proposal: it is proposed to agree to CRs to TS38.412, TS38.422, TS38.462 and TS38.47 provided in R3-194680, R3-194681, R3-194682, R3-194683, which capture the following principle (NG case taken as example):

When the AMF request to dynamically add additional SCTP association between the NG-RAN node/AMF pair, the SCTP Destination Port number value may be 38412, or any dynamic port value (IETF RFC 6335 [x]).

In the following we will discuss the case of the first SCTP association.

The first SCTP association is autonomously triggered by the gNB-DU and for that it needs to be issued on a well-known port. Acknowledging this fact prompted the assignment of IANA ports to NG AP, XnAP, F1 AP and E1 AP. So, for the very first SCTP INIT initiated by an SCTP client to an SCTP server, e.g. in the CU-DU architecture case, this is initiated by the gNB-DU towards an gNB-CU-CP, the IANA port needs to be used.

Indeed, the F1AP allows the gNB-CU-CP to signal a GNB-CU CONFIGURATION UPDATE message containing a gNB-CU TNL Association To Add List IE. For each TNLA to be added an SCTP port to be used is specified by the CP Transport Layer Information IE. On the contrary, the first SCTP association established does not rely on previous signalling to exchange the port numbers to be used. Hence, for the very first SCTP association established between a gNB-DU and a gNB-CU, the IANA port 38472 specified in TS 38.472 shall be used.

The main reason for using a standard IANA is to avoid extra configuration and to allow simple firewall configuration. Configuring a specific server port for every first SCTP association and server-client pair has extra overhead in terms of firewall configurations and manual server/client configuration. To avoid overcomplicating the procedure, the standard port should be used.

We believe that the current description in TS 38.412, TS 38.422, TS 38.462 and TS 38.472 is adequate and clear. So, we propose to keep the current formulation in TS 38.412, TS 38.422, TS 38.462 and TS 38.472.

Proposal 1: We propose to keep the current formulation regarding TNLA establishment in TS 38.412, TS 38.422, TS 38.462 and TS 38.472

# Conclusion

### In this contribution the issue of the use of SCTP ports for multiple TNLAs has been discussed and the following proposal was made.

Proposal 1: We propose to keep the current formulation regarding TNLA establishment in TS 38.412, TS 38.422, TS 38.462 and TS 38.472