3GPP TSG-RAN WG3 Meeting #106	R3-196480

# Reno, NV, USA, November 18th – 22nd 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR TS 38.300 Mapping of Uplink Traffic to Backhaul RLC Channels |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  |  | Date: |  | 2019-11-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Describing the mapping of UL traffic to IAB BH RLC channels |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description the mapping of UL traffic to IAB BH RLC channels |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 description of UL traffic mapping to BH RLC channels missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev1 (pre-RAN3#105bis): rebased on Sep19 version of TS 38.300 Rev2: submission of the BL CR to RAN3#106 |
| --- | --- |

-------------------------------------------Change 1-------------------------------------------

### 4.x.y	Mapping of Uplink Traffic to Backhaul RLC Channels

The IAB-donor CU configures the IAB-node with mappings between upstream F1- and non-F1-traffic originated at the IAB-node, and the appropriate BAP routing ID and Backhaul RLC channel. A separate mapping is configured:

-  for each F1-U GTP-U tunnel,

-  for non-UE associated F1AP messages,

-  for UE-associated F1AP messages of each UE.

Editor’s Note: The mapping for non-F1 traffic is FFS.

Multiple mappings can contain the same Backhaul RLC channel.

-------------------------------------------End of changes ------------------------------------------