3GPP TSG-RAN WG3 Meeting #106	R3-196599

Reno, Nevada, USA, Nov. 18th – 22th, 2019

Agenda item:		20.2.2

Source:	LG Electronics Inc.

Title:	Discussion on F1 for support of sidelink resource mode 1

Document for:		Discussion

1. Introduction

In last meeting, several agreements were achieved on F1 issues for support of sidelink resource mode 1. But still there are some issues left for decision. This paper is to further investigate the open issues again. The corresponding proposals are also provided.

2. Discussion

In last meeting, the following agreements were achieved for mode 1:

- For mode 1 (all types) DU is responsible for SL resource allocation

- DU is responsible for LTE V2X sidelink resource allocation Mode 3 (SPS scheduling)

In [3], the following open issues were captured for further discussion on mode 1:

  - How the gNB-DU gets to know that UE currently requests the resource as a V-UE or P-UE?

- a) Based on the authorization IE (P-UE or V-UE) in the current F1 BLCR

- b) implementation

- c) new IEs in F1

  - For both standalone case and NR Uu controlling LTE sidelink case, how to handle sidelink resource requesting and configuration in case of CU/DU split (F1 signaling)?

- a) New F1 procedures (like in R3-195936)

- b) existing F1 procedures, e.g., UE CONTEXT SETUP/ MODIFICATION procedures

* Note: for either option a) or option b), the key issues are on how to handle the parameters in UEAssistanceInformation and SidelinkUEInformation (defined/to be defined in RAN2), whether all the parameters should be passed to DU or only partial? Whether the paraments are transparent in CU or some should be kept in CU? Whether additional F1 information is needed for NR Uu controlling LTE sidelink case?

Issue 1: For both standalone case and NR Uu controlling LTE sidelink case, how to handle sidelink resource requesting and configuration in case of CU/DU split (F1 signaling)?

It can been from the summary of last meeting, the key issues are on how to handle the parameters in UEAssistanceInformation and SidelinkUEInformation.

RAN2 is working on defining the parameters, which can be referred in the BLCR for TS 38.331 [4][5]. The following procedures are related to this issue:

- UEAssistanceInformation (section 5.7.4)

- SidelinkUEinformationNR (section 5.X.3)

- SidelinkUEinformationEUTRA (section 5.X.4)

For UEAssistanceInformation, UE can set its contents for SPS assistant information (LTE sidelink communication) and also for configured grant assistant information for NR sidelink communication. When CU receives UEAssistanceInformation, it should be passed to DU.

For sidelinkUEinformationNR, a UE capable of NR sidelink communication may initiate the procedure to request assignment of dedicated resources for NR sidelink communication transmission.

On the other hand, for SidelinkUEinformationEUTRA, the purpose of this procedure is to inform the network that the UE is interested or no longer interested to receive V2X sidelink communication, as well as to request assignment or release of transmission resource for V2X sidelink communication. The initiation and the procedure for the transmission of SidelinkUEInformationEUTRA follow the procedure specified for V2X sidelink communication in subclause 5.10.2 of TS 36.331. This can be applied for the case of NR Uu controlling LTE sidelink in case of CU/DU split.

The specification impact for RAN3 is to find or redefine the F1 message to include the information listed above. There are two solutions:

- a) New F1 procedures (like in R3-195936)

- b) existing F1 procedures, e.g., UE CONTEXT SETUP/ MODIFICATION procedures

Solution a) is to define a new class 1 F1 procedure specially for sidelink resource request, which seems to be cleaner if compared with the existing UE Context Modification procedure. Currently, UE Context Modification procedure includes mainly the SCell To Be Setup/Removed, SRB To Be Setup/Released, DRB To Be Setup/Modified/Released etc. RAN2 defined new RRC messages for sidelink. So it is slightly preferred to define new F1 procedures for sidelink resource request/configuration.

Based on the analysis above, the following proposal is suggested:

Proposal 1): It is suggested to define new F1 procedure for sidelink resource request and configuration, which includes the UEAssistanceInformation, SidelinkUEinformationNR, SidelinkUEinformationEUTRA IEs etc.

Issue 2: How the gNB-DU gets to know that UE currently requests the resource as a V-UE or P-UE?

For connected mode UE, UE can request the resource allocation from gNB. One example is described in case of CU/DU split scenario. When CU receives the resource request from UE, it triggers the procedure to DU for getting the V2X SL Resource Configuration from DU. After DU allocate the SL resource for this UE, it passes this V2X SL Resource Configuration to CU, which generates the RRC message with it and finally sends the RRC message to UE. UE will use this information for the resource.

In legacy LTE V2X, UE type, i.e., P-UE or V-UE, is necessary information for eNB to select the resource. Thus, it is also necessary for CU to pass this information to DU so that DU can decide finally the resource based on UE type.

For solving the issue above, three solutions were listed up:

- a) Based on the authorization IE (P-UE or V-UE) in the current F1 BLCR

- b) implementation

- c) new IEs in F1

Solution b) cannot solve the issue since DU needs to decide the resource grant for the UE based on the received information.

Solution a) is to use the authorization IE (P-UE or V-UE) contained in the UE Context Setup/Modification procedure. This may mix the behavior of CU on authorization and the current request from UE. By definition, the authorization is based on subscription information, which should be passed from AMF to CU and then to DU. While, the current UE request may depend on the situation of UE’s power. On the other hand, the limitation is that UE Context Modification procedure has to be used for sidelink resource request and configuration in F1. Solution c) is cleaner way without restriction.

Proposal 2): UE type, i.e., P-UE or V-UE, is transmitted from CU to DU for DU to decide the resource pool to use.

3. Conclusion

In this contribution, the issues of F1 were investigated in order to support NR sidelink resource allocation mode 1. The following proposals are suggested to RAN3:

Proposal 1): It is suggested to define new F1 procedure for sidelink resource request and configuration, which includes the UEAssistanceInformation, SidelinkUEinformationNR, SidelinkUEinformationEUTRA IEs etc.

Proposal 2): UE type, i.e., P-UE or V-UE, is transmitted from CU to DU for DU to decide the resource pool to use.

4. References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- 3GPP TR 38.885 V1.0.0: Study on Vehicle-to-Everything

- R3-196194, “Summary of Offline Discussion: List of open issues related to F1 for support of NR sidelink resource mode 1 and mode 2”, LG Electronics

- R2-1912377, “Running CR to TS 38.331 for 5G V2X with NR sidelink”, Huawei

- [Draft] R2-19xxxxx, “[107bis#91][V2X] 38.331 running CR (Huawei) [Part 2: CR review])”, Huawei