3GPP TSG-RAN WG3 #106			                                                                  R3-196642

# Reno, USA, 18th –22nd November 2019

Agenda item:			14.3.2

Source:					ZTE

Title:	(TP to BL CR #0188 Introduction of Suspend-Resume for 5GC (UP common part) for TS38.413): Delete the UE CONTEXT SUSPEND FAILURE procedure

Document for:		Approval

- Introduction

Based on the BL CR of Introduction of Suspend-Resume for 5GC (UP common part) to 38.413, “UE Context Suspend: unsuccessful operation”(e.g. UE CONTEXT SUSPEND FAILURE) procedure is additionally introduced relative to the UE Context Suspend procedure in 36.413.

Based on the following considerations:

1)  If the DL data is coming, which is transferred from UPF to eNB other than from AMF, AMF may not know whether the DL data is coming.

2)  Based on the current specification, there is no restriction on whether the Uu connection or the Ng connection is firstly suspended, which is based on the eNB implementation. If the Uu connection is suspend firstly (i.e., UE has already suspend lower layer) and AMF replies the "suspend failure", the eNB’s subsequent behaviour is not clear.

So, we suggest to remove the UE CONTEXT SUSPEND FAILURE procedure by this TP.

- Text Propose (TS38.473 15.7.0)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE |  |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8.3.X	UE Context Suspend

8.3.X.1	General

The purpose of the UE Context Suspend procedure is to suspend the UE-associated logical NG-connection and release the NG-U transport bearer with the 5GC while keeping the UE context in the NG-RAN node.

In this version of the specification, this procedure applies only if the NG-RAN node is an ng-eNB.

8.3.X.2	Successful Operation

Figure 8.3.X.2-1: UE Context Suspend: Successful operation.

The NG-RAN node initiates the procedure by sending the UE CONTEXT SUSPEND REQUEST message to the AMF.

Upon receipt of the UE CONTEXT SUSPEND REQUEST message the AMF shall act as defined in TS 23.502 [10].

Upon receipt of the UE CONTEXT SUSPEND RESPONSE message the NG-RAN node shall suspend the UE context, the UE-associated logical NG-connection and the related PDU session contexts and send the UE to RRC_IDLE.

If the Information on Recommended Cells and RAN Nodes for Paging IE is included in the UE CONTEXT SUSPEND REQUEST message, the AMF shall, if supported, store it and may use it for subsequent paging.

If the Cell Identifier and Coverage Enhancement Level IE is included in the UE CONTEXT SUSPEND REQUEST message, the AMF shall, if supported, store it and use it for subsequent paging. (FFS)

If the Security Context IE is included in the UE CONTEXT SUSPEND RESPONSE message, the NG-RAN node shall store the received Security Context IE in the UE context and remove any existing unused stored {NH, NCC} as specified in TS 33.501 [13].

8.3.X.3	Unsuccessful Operation

Void.

8.3.X.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>