3GPP TSG-RAN3 Meeting #106	R3-197163

Reno, Nevada, US, 18-22 November 2019

Title: 	(TP for SON BL CR for TS 38.423): Intra-System and Inter-System Connection Failure

Source: 	Huawei

Agenda item:	10.2.1.1

Document for:	pCR

# 1. Introduction

This is a TP for SON BL CR for TS 38.423

# Annex – TP for BL CR TS38.423

Start of the change

#### 9.1.3.x	FAILURE INDICATION

This message is sent by the NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node 1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.X |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  |  |  |
| > RRC Reestab |  |  |  |  |  |  |
| >>Failure cell PCI | C- ifUERLFReportAbsent |  | 9.2.2.10 | Physical Cell Identifier | YES | ignore |
| >>Re-establishment cell CGI | C- ifUERLFReportAbsent |  | NG-RAN CGI 9.2.2.x |  | YES | ignore |
| >>C-RNTI | C- ifUERLFReportAbsent |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>ShortMAC-I | C- ifUERLFReportAbsent |  | BIT STRING (SIZE (16)) | ShortMAC-I contained in the RRC Re-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | YES | ignore |
| >>UE RLF Report | O |  | 9.2.2.Z | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10] or TS 36.331 [14]) | YES | ignore |
| > RRC Setup |  |  |  |  |  |  |
| >>UE RLF Report | O |  | 9.2.2.Z | RLF -Report-r16 IE contained in the UEInformationResponse message (TS 38.331) | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUERLFReportAbsent | This IE shall be present if the UE RLF Report Container IE is absent |

#### 9.1.3.y	HANDOVER REPORT

This message is sent by the NG-RAN node1 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter System ping-pong …) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | C- ifUERLFReportAbsent |  | NG-RAN CGI 9.2.2.x | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | C- ifUERLFReportAbsent |  | NG-RAN CGI 9.2.2.X | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter System ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCellandnoUERLFReport |  | NG-RAN CGI 9.2.2.X | NG-RAN CGI of cell where UE attempted re-establishment | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from NG-RAN node 2. | YES | ignore |
| UE RLF Report | O |  | 9.2.2.Z | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCellandnoUERLFReport | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" and if the UE RLF Report IE is absent |
| ifUERLFReportAbsent | This IE shall be present if the UE RLF Report IE is absent |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

Start of next change

#### 9.2.2.Z	UE RLF Report

This IE contains the RLF Report to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RLF report type | M |  |  |  |
| >gNB |  |  |  |  |
| >>UE RLF Report Container | M |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [10]. |
| >ng-eNB |  |  |  |  |
| >> UE RLF Report Container | M |  | OCTET STRING | RLF -Report-r16 IE contained in the UEInformationResponse message defined in TS 36.331 [14]. |

End of the change