3GPP TSG-RAN WG3 Meeting #106	R3-197257

Reno, Nevada, USA, Nov. 18th – 22th, 2019

Agenda item:		20.2.2

Source:	LG Electronics Inc.

Title:	Issues on support of sidelink resource mode 2

Document for:		Discussion

1. Introduction

In last meeting, the issue on sidelink resource mode 2 was discussed, but no conclusion was reached. This paper is to further investigate the open issues again. The corresponding proposals are also provided.

2. Discussion

In [3], the following open issues were captured for further discussion:

…

  - For resource allocation Mode 2, whether gNB-CU or DU allocates the resource to UE for both dedicated signaling based and broadcast based? How to support on-demand broadcast (V2X SIB), e.g., which node encodes SIB and potential F1 signaling support for message 3 based on demand broadcast?

* Note: there is no RAN3 impact for message 1 based on demand broadcast

…

Issue 1: How to support message 3 based on-demand broadcast (V2X SIB) for idle or inactive mode UE

On message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE, One example is shown below:

Figure 1. Signalling between CU and DU to support mode 2 of NR Resource Allocation

According to the current specification, MIB and SIB1 are in charged by DU. The location of encoding other SIBs are in CU, which is the principle. Therefore, if this principle has to be kept, the location of SIB (e.g., SIB21) for NR V2X should be located in the CU.

On the other hand, RAN2 is defining the SIBs for V2X [6]. From the current version of BLCRs, it seems that the main parameters are relatively static for mode 2 from resource pool point of view. Therefore, it is also possible to follow the principle of encoding other SIBs in CU.

SIBX field descriptions

sl-EUTRA-AnchorCarrierFreqList This field indicates the EUTRA anchor carrier frequency list, which can provide the NR sidelink communication configurations.

sl-FreqInfoList This field indicates the NR sidelink communication configuration on some carrier frequency (ies). In this relase, only one entry can be configured in the list.

sl-NR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink communication configurations.

…

As shown in the figure above, UE transmits the on demand broadcast request to CU, which can make a decision and indicates the SIB index to DU. DU finally broadcast the resource to UE based on the index from CU.

For supporting the procedure above, CU needs to tell the DU in advance on the information elements of V2X SIB (e.g., SIB21) by F1 Setup Response message, which is shown as follows:

------------------------------------------------------------------------------------------------------------------------

9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - | - |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | - | - |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| gNB-CU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |

------------------------------------------------------------------------------------------------------------------------------------------------

However, this is similar to the defined SIB2~SIB9 of the current specification. There is no additional RAN3 impacts foreseen.

Observation 1): No additional RAN3 signaling impacts are required on supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE.

Based on the analysis above, the following proposal is suggested to RAN3:

Proposal 1): On supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE, the location of encoding SIB for V2X resource broadcasting is in CU.

Issue 2: How to support dedicated signaling based mode 2 configuration

For RRC connected UE, UE may request mode 2 resource pool to CU by RRC message. If CU is in charge of the mode 2 resource allocation, it does not need to ask DU and get it back from DU. CU can generate RRC reconfiguration including the mode 2 sidelink resource configuration and sends it to UE directly. This can save the signaling between CU and DU.

Proposal 2): On supporting dedicated signaling based mode 2 configuration, CU is in charge of the mode 2 resource allocation.

3. Conclusion

In this contribution, the open issues on support of NR sidelink resource allocation mode 2 were investigated. The following proposals are suggested to RAN3:

Observation 1): No additional RAN3 signaling impacts are required on supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE.

Proposal 1): On supporting message 3 based on-demand broadcast (V2X SIB) request from idle/inactive UE, the location of encoding SIB for V2X resource broadcasting is in CU.

Proposal 2): On supporting dedicated signaling based mode 2 configuration, CU is in charge of the mode 2 resource allocation.

4. References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- 3GPP TR 38.885 V1.0.0: Study on Vehicle-to-Everything

- R3-196194, “Summary of Offline Discussion: List of open issues related to F1 for support of NR sidelink resource mode 1 and mode 2”, LG Electronics

- 3GPP TR 23.786 V1.1.0: Study on architecture enhancements for EPS and 5G System to support advanced V2X services

- R2-1912377, “Running CR to TS 38.331 for 5G V2X with NR sidelink”, Huawei

- [Draft] R2-19xxxxx, “[107bis#91][V2X] 38.331 running CR (Huawei) [Part 2: CR review])”, Huawei