3GPP TSG-RAN3 Meeting #106	R3-197399

Reno, Nevada, US, 18th November – 22nd November 2019

Agenda Item:	10.2.1.5

Source:	Ericsson

Title:	Considerations on successful HO report

Document for:	Approval

# Introduction

The issue of successful HO report will be discussed in RAN2. In this contribution we will discuss support of the report in RAN3.

# Discussion

In RAN2, the topic of successful HO report is up for discussion for the next meeting. We believe that in RAN3, it would be good to take into account the possibility to signal the report of successful HO. The reason for this is that it would be good to prepare the signalling specifications for the case that the Successful HO Report is present. Failure to do so would mean that when RAN2 concludes on the topic RAN3 would have to re-work its specifications rather extensively.

Proposal 1: RAN3 is kindly requested to consider supporting the signalling of successful HO report.

Once the target cell receives the successful handover report, it may inform the serving cell about the potential issues at RLM level contained in the report via Xn based signalling. Similar to RLF reporting mechanism over Xn interface, a successful handover report can be signalled over the Xn interface to the last serving cell.

The source gNB can process successful HO report to optimize a number of configurations, for example:

- the measurement report triggering thresholds (e.g., A3 event triggering threshold) in such a way that the UE receives the handover command including mobility control parameters within a proper time window before RLM issues

- Configuration of RLM RS resources (i.e. list of beams used for RLM), dedicating more suitable beams for RLM purposes, or via a better beam configuration and adaptation to mitigate the mismatch between link beam and control beams’ coverage.

- Proposal 2: It is proposed to capture a solution based on Xn signaling of the successful handover information reported by the UE at target RAN node between RAN nodes involved in the handover process.

Since all the serving cell beam management related configuration takes place at DU, a successful HO report including RLM and Beam Failure Detection (BFD) related issue is required to be forwarded from gNB-CU of the target node to the source gNB and thereof to the source gNB-DU. Therefore, gNB-DU can optimize the beam configurations and provide such new configurations as part of the RRC CellGroupConfig IE, which is signalled to the UE by the gNB-CU.

- Proposal 3: It is proposed to capture a solution based on F1 signaling of the successful handover information from CU to the DU of the RAN node owning the source cell in the handover process.

# Conclusion

### In this contribution the successful HO report has been discussed and the following conclusions and proposals were made:

Proposal 1: RAN3 is kindly requested to consider supporting successful HO report.

- Proposal 2: It is proposed to capture a solution based on Xn signaling of the successful handover information reported by the UE at target RAN node between RAN nodes involved in the handover process.

- Proposal 3: It is proposed to capture a solution based on F1 signaling of the successful handover information from CU to the DU of the RAN node owning the source cell in the handover process.

# TP for SON BL CR for TS 38.423

============ Start of 1st change ==============

#### 9.1.3.y	HANDOVER REPORT

This message is sent by the NG-RAN node1 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, …) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | FFS |  | NG-RAN CGI 9.2.2.x NG-RAN CGI 9.2.2.x | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | C- ifUERLFReportContainerAbsent |  | NG-RAN CGI 9.2.2.X | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1) . If the Handover Report Type is set to “Inter System ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | NG-RAN CGI 9.2.2.X | NG-RAN CGI of cell where UE attempted re-establishment | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | OCTET STRING | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| Successful HO report container FFS | O |  | OCTET STRING | Successful-HO-Report-r16 contained in the UEInformationResponse message (TS 38.331 [10] or TS 36.331 [14]) FFS | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifUERLFReportContainerAbsent | This IE shall be present if the UE RLF Report Container IE is absent |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |