3GPP TSG-RAN WG3 Meeting #106                                                              R3-197569

Reno, Nevada, USA, 18 – 22 November 2019

Agenda item:	13.1

Source: 		Qualcomm Incorporated (Rapporteur)

Title: 	Upper layer parameters to support IAB physical layer operation

Document for:	Information

# 1	Introduction

This paper contains R1-1911724 on Upper layers parameters to support IAB physical layer operation.

# 2	R1-1911724

3GPP TSG RAN WG1 Meeting #98bis							     R1-1911724

Chongqing, China, 14th October– 20th October 2019

Agenda Item:		7.2.3

Source:			WI Rapporteur (Qualcomm Incorporated)

Title: 	Upper layers parameters to support IAB physical layer operation

Document for:		Discussion and decision

### Introduction

This document provides the current view after RAN1 #98 on the required upper layer parameters needed to support the IAB physical layer operation agreed by RAN1.

In case of IAB the parameters to be configured are split between RRC and F1-AP. It is understood that RAN2/RAN3 will decide the signaling mechanisms and RAN1 provides some recommendations based on the expected behavior of the system.

The focus of the parameters listing in this document is on new and extended parameters. IAB specific configuration requirements for existing parameters are also provided, when applicable.

### Extensions of SSBs for inter-IAB-node discovery and measurements

SSB transmission configuration: F1AP signaling from CU to IAB node DU

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB-based Discovery and measurement | New | ssbFrequency | SSB central frequency | ARFCN-ValueNR |  | IAB node specific |  |  |
| SSB-based Discovery and measurement | New | ssbSubcarrierSpacing | SSB subcarrier spacing | FR1: 15khz, 30khz FR2: 120khz, 240khz |  | IAB node specific |  |  |
| SSB-based Discovery and measurement | New | ssbPeriodicity | SSB transmission periodicity: | 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms |  | IAB node specific |  |  |
| SSB-based Discovery and measurement | New | ssbTimingOffset | SSB transmission timing offset | 0, … , (number of half frames within SSB transmission periodicity) – 1 |  | IAB node specific |  | Up to RAN3 whether or not to jointly signaled with ssbPeriodicity |
| SSB-based Discovery and measurement | New | ssb-PositionsInBurst | SSBs to be transmitted in the half frame | length-4 bitmap, length-8 bitmap, length-64 bitmap |  | IAB node specific |  |  |
| SSB-based Discovery and measurement | New | ssb-PhysicalCellID | Physical cell-ID carried by SSB | 0,…,1007 |  | IAB node specific |  | Up to RAN3 whether or not to have a separate signaling |
| SSB-based Discovery and measurement | New | Nr-ssb-TransmisisonConfig | Number of SSB transmission configurations | 1, 2, 3, 4 |  | IAB node specific |  | For IAB node discovery and measurement, the maximum number of STCs that can be configured for an IAB node DU per cell at one frequency location is 4. Each STC is independently configured when multiple STCs are configured. |

SSB reception configuration: RRC signaling from CU to IAB node MT

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB-based Discovery and measurement | New | ssb-MTC-Periodicity | SMTC window periodicity | 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms |  | IAB node specific |  |  |
| SSB-based Discovery and measurement | New | ssb-MTC-Timingoffset | SMTC window timing offset | 0, ..., (number of subframes within SMTC window periodicity) – 1 |  | IAB node specific |  | Up to RAN2 whether or not jointly signaled with ssb-MTCPeriodicity |
| SSB-based Discovery and measurement | New | ssb-MTC-Duration | SMTC window duration | 1, …, 5 (subframes) |  | IAB node specific |  |  |
| SSB-based Discovery and measurement | New | ssb-MTC-pci-List | List of physical cell IDs to be measured | [cell ID 0, …, cell ID M-1 ] |  | IAB node specific |  | The maximum number of M is 64 |
| SSB-based Discovery and measurement | New | ssb-ToMeasure | SSBs To be Measured | length-4 bitmap, length-8 bitmap, length-64 bitmap |  | IAB node specific |  | Up to RAN2 whether or not to have a separate signaling Note: This parameter is included in MeasObjectNR as in Rel-15. |
| SSB-based Discovery and measurement | New | Nr-SMTCConfig | Number of SMTC configurations | 1, 2, 3, 4 |  | IAB node specific |  | For IAB node discovery and measurement, the maximum number of SMTC windows that can be configured for an IAB node is 4. Each SMTC window is provided its own independent configuration (e.g. periodicity, offset, duration) |

### Extension of RACH occasions and periodicities for backhaul RACH resources

For IAB nodes there needs to be the ability to configure the RACH configuration separately from the RACH configuration for UE access. As a result all the parameters in RACH-ConfigGeneric and RACH-ConfigCommon need to have the ability to be configured specifically for IAB nodes.

RACH configuration: RRC signaling from CU to IAB node MT

|  | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PRACH | New | prach-ConfigurationPeriodScaling | Scaling factor to extend the periodicity of the baseline configuration indicated by prach-ConfigurationIndex | {1, 2, 4, 8, 16, 32, 64} |  | IAB node specific and IAB nodes common |  |  |
| PRACH | New | prach-ConfigurationFrameOffset | Frame offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex | {0, …, 63} |  | IAB node specific and IAB nodes common |  |  |
| PRACH | New | prach-ConfigurationSOffset | Subframe/Slot offset for ROs defined in the baseline configuration indicated by prach-ConfigurationIndex | {0, …, 39} |  | IAB node specific and IAB nodes common |  |  |

### Mechanisms for resource multiplexing among backhaul and access links

Resource multiplexing: F1AP signaling between CU and IAB node or Donor DU

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource multiplexing | New | IAB-DU Resource Configuration (name in specification to be determined by RAN2/3) | Per-cell D/U/F resource type configuration + H/S/NA attributes per-resource type | All patterns, periodicities, and slot configs supported in TDD-UL-DL-Config, TDD-UL-DL-SlotConfig and Table 11.1.1-1 in TS38.213 + TBD slot formats with the sequence order UL-Flexible-DL |  | IAB node specific |  | RAN1 #98 Agreements: H/S/NA is applied/determined relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing. The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies). RAN1 #98bis Agreements: H/S/NA attributes for the per-cell DU resource configuration are explicitly indicated per-resource type (D/U/F) in each slot. |
| Resource multiplexing | New | Child IAB-DU Resource Configuration (name in specification to be determined by RAN2/3) | Per-cell D/U/F resource type configuration + H/S/NA attributes per-resource type of a child IAB node | Same as IAB-DU Resource Configuration |  | IAB node specific |  | RAN1 #96bis Agreements: An IAB node has the ability to be made aware of the semi-static DU resource configuration (D/U/F/H/S/NA) of all its child IAB nodes. - in the case the full DU resource configuration information of its child IAB nodes is not necessary, ensure that only the necessary configuration information is signaled to the IAB node. FFS the necessary configuration information. |
| Resource multiplexing | New | IAB-Node Multiplexing Capability | Indicates the multiplexing capability between IAB-MT and IAB-DU functions (TDM required, TDM not required) of an IAB node to for any {MT CC, DU cell} pair | List of {MT CC, DU cell} pairs each with a corresponding value {TDM required, TDM not required} |  | IAB node specific |  | RAN1 #98bis Agreements: The donor CU and the parent node can be made aware of the multiplexing capability between MT and DU (TDM required, TDM not required) of an IAB node to for any {MT CC, DU cell} pair. · Signaling details up to RAN2/RAN3. |

Resource multiplexing: MAC-CE signaling between child IAB-MT and parent IAB-DU/IAB Donor DU

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource multiplexing | New | Desired Guard Symbols (name in specification to be determined by RAN2/3) | Number of symbols the IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between the IAB node MT and DU | TBD (likely [0, 3]) |  | IAB node specific |  | This corresponds to this portion of the following agreement. Note that 8 separate entries are needed. RAN1 #98bis Agreements: A parent IAB node can be made aware of the number of symbols Ng the child IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between child MT and child DU. Separately or additionally, the child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide. • Ng can be provided for each of the [8] possible transitions with potential overlap: MT to DU; DL Tx; UL Rx. DL Rx. UL Tx. DU to MT; DL Rx; UL Tx. DL Tx. UL Rx. • If Ng is not provided it is assumed to be 0 |
| Resource multiplexing | New | Provided Guard Symbols (name in specification to be determined by RAN2/3) | Number of symbols the IAB node uses at the edge (beginning or end) of a slot when there is a transition between the IAB node MT and DU at the child node | TBD (likely [0, 3]) |  | IAB node specific |  | This corresponds to this portion of the following agreement. Note that 8 separate entries are needed. RAN1 #98bis Agreements: A parent IAB node can be made aware of the number of symbols Ng the child IAB node would like the parent IAB node not to use at the edge (beginning or end) of a slot when there is a transition between child MT and child DU. Separately or additionally, the child IAB node can be made aware of the number of guard symbols that the parent IAB node will provide. • Ng can be provided for each of the [8] possible transitions with potential overlap: MT to DU; DL Tx; UL Rx. DL Rx. UL Tx. DU to MT; DL Rx; UL Tx. DL Tx. UL Rx. • If Ng is not provided it is assumed to be 0 |

Resource multiplexing: Dedicated RRC signaling from CU to IAB node MT

| Sub-feature group | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Resource multiplexing | New or Existing | TDD-UL-DL-Config-IAB-MT | Per-MT D/U/F resource configuration | All patterns, periodicities, and slot configs supported in TDD-UL-DL-Config and TDD-UL-DL-SlotConfig + TBD slot formats with the sequence order UL-Flexible-DL |  | IAB node specific |  | Whether to introduce new IEs or enhancements to existing RRC signaling is up to RAN2 |
| Resource multiplexing | New or Existing | SlotFormatCombinationsPerCell-IAB-MT | Used to configure for an IAB-MT the SlotFormatCombinations applicable for one serving cell | Same as SlotFormatCombinationsPerCell + support for indication of slot formats with the sequence order UL-Flexible-DL using reserved entries of Table 11.1.1 of TS 38.213 |  | IAB node specific |  | RAN1#98bis Agreements: Dynamic indication of UL-Flexible-DL (intended to dynamically update the flexible symbols) is supported for IAB-node MTs The dynamic indication of slot formats of UL-Flexible-DL should use DCI format 2_0 and reserved entries in Table 11.1.1-1 in 38.213. |
| Resource multiplexing | New or Existing | SlotFormatCombination-IAB-MT | A SlotFormatCombination-IAB-MT comprises of one or more SlotFormats (see TS 38.211, clause 4.3.2) | Same as SlotFormatCombinationsPerCell + support for indication of slot formats with the sequence order UL-Flexible-DL using reserved entries of Table 11.1.1 of 38.213 |  | IAB node specific |  |  |
| Resource multiplexing | New or Existing | SlotFormatIndicator-IAB-MT | Used to configure for an IAB-MT monitoring for a Group-Common-PDCCH for Slot-Format-Indicators (SFI) | Same as SlotFormatIndicator |  | IAB node specific |  |  |
| Resource Multiplexing | New | AvailabilityIndicator | The IE AvailabilityIndicator is used to configure monitoring a PDCCH for Availability Indicators (AI) | ai-RNTI, list of AvailabilityCombinationsPerCell of SIZE(1..maxNrofAssociatedDUCellsPerMT) |  | IAB node specific |  | RAN1 #98bis Agreements: A DCI following the DCI Format 2_0 structure is used to indicate DU-IA to an IAB node using a new IA-RNTI different from SFI-RNTI. This DCI contains one or multiple fields (similar to SFI-index fields in DCI Format 2_0), each field value is used as the index in a RRC configured AI (Availability Indicator) AvailabilityCombination table (similar to the SFI SlotFormatCombination table). Each entry in the AI AvailabilityCombination table indicates the resource availability for a set of consecutive slots. Each element of each entry in the AI AvailabilityCombination table indicates the resource availability in a slot. The resource availability can take 8 values: Value; Meaning. 0; No resources available. 1; D resources available. 2; U resources available. 3; D and U resources available. 4; F resources available. 5; D and F resources available. 6; U and F resources available. 7; All resources available. The maximum number of entries in the AI AvailabilityCombination table is 512. |
| Resource multiplexing | New | ai-RNTI | RNTI used by an IAB-MT for detection of DCI Format 2-[X] indicating DU-IA to an IAB node | same value range for SFI-RNTI |  | IAB node specific |  |  |
| Resource multiplexing | New | dci-PayloadSize-AI | Total length of the DCI payload scrambled with ai-RNTI | INTEGER (1..maxAI-DCI-PayloadSize), |  | IAB node specific |  |  |
| Resource multiplexing | New | availabilityCombToAddModList | A list of AvailabilityCombinations to add for the IAB-DU's cells | SEQUENCE (SIZE(1.. maxNrofAssociatedDUCellsPerMT)) OF AvailabilityCombinationsPerCell |  | IAB node specific |  |  |
| Resource multiplexing | New | availabilityCombToReleaseList | A list of AvailabilityCombinations to release for the IAB-DU's cells | SEQUENCE (SIZE(1.. maxNrofAssociatedDUCellsPerMT)) OF ServCellIndex |  | IAB node specific |  |  |
| Resource multiplexing | New | AvailabilityCombinationsPerCell | The IE AvailabilityCombinationsPerCell is used to configure the AvailabilityCombinations applicable for one IAB-DU cell | SEQUENCE positionInDCI-AI, iabDuCellId-AI, SEQUENCE of AvailabilityCombination of SIZE (1..maxNrofAvailabilityCombinationsPerSet)) |  | IAB node specific |  |  |
| Resource multiplexing | New | availabilityCombinations | A list with AvailabilityCombinations. Each AvailabilityComination comprises of one or more resourceAvailability | SEQUENCE (SIZE (1..maxNrofAvailabilityCombinationsPerSet)) of AvailabilityCombination |  | IAB node specific |  |  |
| Resource multiplexing | New | positionInDCI-AI | The (starting) position (bit) of the availabilityCombinationId (AI-Index) for the indicated IAB-DU cell (iabDuCellId-AI) within the DCI payload | INTEGER(0..maxAI-DCI-PayloadSize-1) |  | IAB node specific |  |  |
| Resource multiplexing | New | iabDuCellId-AI | The ID of the IAB-DU cell for which the availabilityCombinations are applicable | ServCellIndex |  | IAB node specific |  |  |
| Resource multiplexing | New | AvailabilityCombination | Each AvailabilityCombination comprises of one or more resourceAvailability | availabilityCombinationId, resourceAvailability |  | IAB node specific |  |  |
| Resource multiplexing | New | availabilityCombinationId | This ID is used in the DCI Format 2-X payload to dynamically select this AvailabilityCombination | INTEGER (0..maxNrofAvailabilityCombinationsPerSet-1), maxNrofAvailabilityCombinationsPerSet = 512 |  | IAB node specific |  |  |
| Resource multiplexing | New | resourceAvailability | Indicates the resource availability for a set of consecutive slots in time domain | SEQUENCE {SIZE (1..maxNrofResourceAvailabilityPerCombination) }of INTEGER (0..7) maxNrofResourceAvailabilityPerCombination = 256 The resource availability can take one of 8 values per slot: Value; Meaning. 0; No resources available. 1; D resources available. 2; U resources available. 3; D and U resources available. 4; F resources available. 5; D and F resources available. 6; U and F resources available. 7; All resources available. |  | IAB node specific |  |  |
| Resource multiplexing | New | SearchSpace-IAB | The IE searchSpace-IAB defines how/where to search for PDCCH candidates for an IAB-MT. Each search space is associated with one ControlResourceSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent | Same as SearchSpace, except for new fields: dci-Format2-[X], duration-IAB, monitoringSlotPeriodicityAndOffset-IAB, monitoringSymbolsWithinSlot-IAB, nrofCandidates-IAB, searchSpaceType-IAB |  | IAB node specific |  |  |
| Resource multiplexing | New | dci-Format2-[X] | If configured, IAB-MT monitors the DCI format 2_[X] according to TS 38.213 | List of nrofCandidates-IAB per AggregationLevel {1,2,4,8,16} |  | IAB node specific |  |  |
| Resource multiplexing | New | duration-IAB | Number of consecutive slots that a Search Space lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_[x]. The UE ignores this field for DCI format 2_0 and DCI format 2_[x]. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset) | Same as duration |  | IAB node specific |  |  |
| Resource multiplexing | New | monitoringSlotPeriodicityAndOffset-IAB | Slots for PDCCH Monitoring configured as periodicity and offset. If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_[x], only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10) | Same as monitoringSlotPeriodicityAndOffset |  | IAB node specific |  |  |
| Resource multiplexing | New | monitoringSymbolsWithinSlot-IAB | The first symbol(s) for PDCCH monitoring in the slots configured for PDCCH monitoring (see monitoringSlotPeriodicityAndOffset-IAB and duration-IAB). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the IAB-MT. For DCI format 2_0 or DCI format 2_[x], the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213, clause 10 | Same as monitoringSymbolsWithinSlot |  | IAB node specific |  |  |

### Mechanism to support the “case-1” OTA timing alignment

OTA timing alignment: MAC-CE signaling between child IAB-MT and parent IAB-DU/IAB Donor DU

|  | New or existing parameter | Parameter name in specification | Description | Value range | Default value | IAB node specific/IAB nodes common | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case-1 timing | New | T_delta (name in specification to be determined by RAN2) | (-1/2) of time interval at the parent node between the start of UL RX frame i for the IAB node and the start of DL TX frame i. | TBD by RAN4 | N/A | IAB node specific |  | Agreed to be signaled via MAC-CE in RAN1 #98bis. |