3GPP TSG-RAN3 Meeting #106	R3-197614

Reno, USA, 18 – 22 November, 2019

Title: 	(TP for NR_Mob_enh BL CR for TS 38.423): CHO cancelation

Source: 	Huawei

Agenda item:	15.3.1

Document for:	pCR

1. Introduction

This document contain the XnAP for the following CB:

CB: # 45_CHOcancel

-  merge 6902+6672, 6903+6673

- avoid describing behavior on absence of an IE

- check def and usage of NG CGI -> NG RAN CGI

- check details

(ZTE,HW)

6902 rev in R3-197614 (HW)

6672 rev in R3-197615 (ZTE)

Annex – TP for CHO BL CR for TS 38.423

Start of first change

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: Handover Cancel, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER CANCEL message to the target NG-RAN node. The source NG-RAN node shall indicate the reason for cancelling the handover by means of an appropriate cause value.

If the Candidate Cells To Be Cancelled List IE is included in HANDOVER CANCEL message, the target NG-RAN node shall consider that the source NG-RAN node is cancelling only the handover associated to the cells identified by the included NG-RAN CGI.

If the Candidate Cells To Be Cancelled List IE is included in HANDOVER CANCEL message, the target NG-RAN node shall consider that the source NG-RAN node is cancelling the prepared conditional handovers in the candidate cells associated to the same UE-associated signaling identified by the Source NG-RAN node UE XnAP ID and Target NG-RAN node UE XnAP ID, if included.

End of first change

Start of second change

### 8.2.A	Conditional Handover Cancel

#### 8.2.A.1	General

The Conditional Handover Cancel procedure is used to enable a target NG-RAN node to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

#### 8.2.A.2	Successful Operation

Figure 8.2.A.2-1: Conditional Handover Cancel, successful operation

The target NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source NG-RAN node. The target NG-RAN node shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

At the reception of the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved to the concerned UE context for the concerned UE-associated signalling.

If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the candidate cells associated to the same UE-associated signaling identified by the Source NG-RAN node UE XnAP ID and Target NG-RAN node UE XnAP ID are about to be released.

#### 8.2.A.3	Unsuccessful Operation

Not applicable.

#### 8.2.A.4	Abnormal Conditions

If the CONDITIONAL HANDOVER CANCEL message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

If the Candidate Cells To Be Cancelled List IE included in CONDITIONAL HANDOVER CANCEL are not associated to the same UE-associated signaling connection used for creating before, the source NG-RAN node shall ignore the message.

End of second change