3GPP TSG-RAN WG3 Meeting #106	R3-197658

# Reno, NV, USA, November 18th – 22nd 2019

Agenda Item:	13.2.1.2

Source:	Ericsson

Title:	(TP for NR-IAB BL CR for TS 38.470): Backhaul RLC Channel QoS

Document for:	Agreement

# 1	Introduction

This is a Stage-2 TP corresponding to the F1AP TP on CP traffic mapping, presented in R3-197657.

Proposal: RAN3 to agree the TP for NR-IAB BL CR for TS 38.470 presented in the Annex.

# Annex: TP for NR_IAB BL CR to TS 38.470

-------------------------------------------Change 1-------------------------------------------

### 5.2.3	F1 UE context management function

The F1 UE context management function supports the establishment and modification of the necessary overall UE context.

The establishment of the F1 UE context is initiated by the gNB-CU and accepted or rejected by the gNB-DU based on admission control criteria (e.g., resource not available).

The modification of the F1 UE context can be initiated by either gNB-CU or gNB-DU. The receiving node can accept or reject the modification. The F1 UE context management function also supports the release of the context previously established in the gNB-DU. The release of the context is triggered by the gNB-CU either directly or following a request received from the gNB-DU. The gNB-CU request the gNB-DU to release the UE Context when the UE enters RRC_IDLE or RRC_INACTIVE.

This function can be also used to manage DRBs and SRBs, i.e., establishing, modifying and releasing DRB and SRB resources. The establishment and modification of DRB resources are triggered by the gNB-CU and accepted/rejected by the gNB-DU based on resource reservation information and QoS information to be provided to the gNB-DU. For each DRB to be setup or modified, the S-NSSAI may be provided by gNB-CU to the gNB-DU in the UE Context Setup procedure and the UE Context Modification procedure.

The mapping between QoS flows and radio bearers is performed by gNB-CU and the granularity of bearer related management over F1 is radio bearer level. For NG-RAN, the gNB-CU decides an aggregated DRB QoS profile for each radio bearer based on received QoS flow profile, and provides both aggregated DRB QoS profile and QoS flow profile to the gNB-DU, and the gNB-DU either accepts the request or rejects it with appropriate cause value. With this function, gNB-DU could also notify gNB-CU whether the QoS for already established DRBs is not fulfilled any longer or it is fulfilled again. To support packet duplication for intra-gNB-DU CA as described in TS 38.300 [8], one data radio bearer should be configured with two GTP-U tunnels between gNB-CU and a gNB-DU.

With this function, gNB-CU requests the gNB-DU to setup or change of the SpCell (as defined in TS 38.321 [10]) for the UE, and the gNB-DU either accepts or rejects the request with appropriate cause value.

With this function, the gNB-CU requests the setup of the SCell(s) at the gNB-DU side, and the gNB-DU accepts all, some or none of the SCell(s) and replies to the gNB-CU. The gNB-CU requests the removal of the SCell(s) for the UE.

With this function, the gNB-CU indicates the UL UE AMBR limit to the gNB-DU, and the gNB-DU enforces the indicated limit.

With this function, the gNB-DU indicates that a bearer, or a UE is inactive or active. The gNB-CU consolidates all the serving gNB-DUs for the UE and takes further action.

In addition, for IAB nodes and IAB-donors:

The F1 UE context management function is used to manage BH RLC channels, i.e., establishing, modifying and releasing BH RLC channel resources. The establishment of BH RLC channels is triggered by the IAB-donor-CU. The establishment and modification is accepted/rejected by the IAB-node’s parent, based on e,g, resource reservation information and QoS information to be provided to the IAB-node’s parent.

The DRB QoS profile framework is reused for BH RLC channels carrying DRBs. Prioritization of different CP traffic types (e.g. UE-associated, non-UE-associated) is based on CP traffic type and enforced in the IAB-donor-DU and in IAB nodes; in-sequence delivery over the signaling connection is always ensured.

The IAB-donor-CU associates each backhaul RLC channel carrying control plane traffic with one of the signaled CP type values.

Editor’s note: it is necessary to define an overall term to refer to both IAB-nodes and IAB-donor.

-------------------------------------------End of changes ------------------------------------------