3GPP TSG-RAN WG3 Meeting #106	R3-197704

Reno, NV, USA, 18-22 Nov 2019

Agenda item:	15.3.1.5

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of the offline on support for DL duplication with more than 2 RLCs

Document for:	Discussion

# 1	Introduction

At RAN3 #106 meeting, possible impact of enabling up to 4 RLC connections for UE on the RAN3 specification was considered. This document summarises the following offline discussion.

# 2	Discussion

There were two issues considered:

- Data transfer mechanism for more than 2 RLCs; and

- Coordination of the DL duplication between the MN and the SN.

Data transfer mechanism for more than 2 RLCs

The main question is if separate tunnel is needed for each connection. This depends on the answer why two tunnels were needed for Rel-15 duplication.

Summary: The motivation for separate tunnels in Rel-15 duplication shall be reminded before more tunnel endpoints are added.

Coordination of the DL duplication between the MN and the SN

There were 3 solutions considered: no coordination, partial coordination and full coordination [1]. No coordination follows situation of Rel-15, where no coordination is present. Partial coordination offers better usability of more than 2 RLCs. However, drawbacks of full coordination overcome possible benefits.

Summary: Only no coordination or partial coordination (as proposed in [1]) is to be considered for coordination.

# 3	Conclusions

For data transfer for more than 2 RLCs: The motivation for separate tunnels in Rel-15 duplication shall be reminded before more tunnel endpoints are added.

For coordination of DL duplication of more than 2 RLCs: Only no coordination or partial coordination (as proposed in [1]) is to be considered for coordination.

# References

- R3-196590, RAN3 #106