3GPP TSG-RAN WG3 Meeting #107-e 	R3-200223

# E-meeting, -, 24.02 - 6.03.2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1428 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of the solution for fast link recovery for DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 10.02.2020 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current description of the mechanism suggests the SN configures the UE only in response to the request from the MN. This is not quite optimal, because the SN should have the option to configure the feature on its own and then only to inform the MN about the status. The problem concerns also the modification: the indication that the MN does not need it should be informative only, so that the SN is not hinted to perform particular actions upon receiving it. If the MN requests split SRB1, but the SN rejects that, then it should not wait for the MN to request SRB3 reporting separately – the SN should be able to enable it on its won and just let the MN know about it. Further correction concerns the description of the RRC Transfer. The text existing before the last meeting’s changes listed conditions when the RRC Transfer is to be ignored: If the en-gNB receives an RRC TRANSFER message which includes neither the RRC Container IE in the Split SRB IE nor the RRC container IE in NR UE Report IE, it shall ignore the message. Obviously, the message should not be ignored if the new containers are included. Therefore, the list of IEs that make the message relevant should be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IEs used for the response from the SN are renamed so that they present the status of the configuration, not the response to MN’s request. As such, they are also added to the SN-initiated modification. Description of the RRC Transfer procedure is updated to take the new IEs into account. Since the feature is now completed, the notes indicating the need for changes are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature remains incloplete and so described in the notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.4.2, 8.7.6.2, 8.7.7.2, 8.7.12.2, 9.1.4.2, 9.1.4.6, 9.1.4.8, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

*** First change, skipped text not changed ***

#### 8.7.4.2	Successful Operation

Figure 8.7.4.2-1: SgNB Addition Preparation, successful operation

The MeNB initiates the procedure by sending the SGNB ADDITION REQUEST message to the en-gNB. When the MeNB sends the SGNB ADDITION REQUEST message, it shall start the timer TDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the Full E-RAB Level QoS Parameters IE or in the Requested MCG E-RAB Level QoS Parameters IE or in the Requested SCG E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If the SGNB ADDITION REQUEST message contains the Serving PLMN IE, the en-gNB may use it for RRM purposes.

If the SGNB ADDITION REQUEST message contains the Expected UE Behaviour IE, the en-gNB shall, if supported, store this information and may use it to optimize resource allocation.

If the SGNB ADDITION REQUEST message contains the Handover Restriction List IE, the en-gNB node, if supported, shall store this information and use it to select an appropriate NR cell.

If the SGNB ADDITION REQUEST message contains the MeNB Resource Coordination Information IE, the en-gNB should forward it to lower layers and it may use it for the purpose of resource coordination with the MeNB. The en-gNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The en-gNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the MeNB Coordination Assistance Information IE is contained in the MeNB Resource Coordination Information IE, the en-gNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

The en-gNB shall choose the ciphering algorithm based on the information in the NR UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the SgNB Security Key IE as specified in the TS 33.401 [18].

If the SGNB ADDITION REQUEST message contains the Subscriber Profile ID for RAT/Frequency Priority IE, the en-gNB may use it for RRM purposes.

If the SGNB ADDITION REQUEST message contains the Additional RRM Policy Index IE, the en-gNB may use it for RRM purposes.

The en-gNB shall search for the target NR cell among the NR neighbour cells of the E-UTRAN cell indicated in MeNB Cell ID IE, as specified in the TS 37.340 [32].

If the Masked IMEISV IE is contained in the SGNB ADDITION REQUEST message the en-gNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

The en-gNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested E-RABs in the following way:

-	a list of E-RABs which are successfully established shall be included in the E-RABs Admitted To Be Added List IE;

-	a list of E-RABs which failed to be established shall be included in the E-RABs Not Admitted List IE.

NOTE:	The MeNB may trigger the SgNB Addition Preparation procedure in the course of the Inter-MeNB handover without SgNB change procedure as described in TS 37.340 [32]. The deleted E-RABs are not included in the E-RABs To Be Added List IE in the SGNB ADDITION REQUEST message, from MeNB point of view. If the en-gNB reports a certain E-RAB to be successfully established, respective SCG resources, from an en-gNB point of view, may be actually successfully established or modified or kept; if a certain E-RAB is reported to be failed to be established, respective SCG resources, from an en-gNB point of view, may be actually failed to be established or modified or kept.

For each E-RAB successfully established in the en-gNB, the en-gNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the same value in the EN-DC Resource Configuration IE as received in the SGNB ADDITION REQUEST message.

For each E-RAB for which allocation of the PDCP entity is requested at the en-gNB:

-	the MeNB may propose to apply forwarding of downlink data by including the DL Forwarding IE within the E-RABs To be Added Item IE of the SGNB ADDITION REQUEST message. For each E-RAB that it has decided to admit, the en-gNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs Admitted To Be Added Item IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. This GTP tunnel endpoint may be different from the corresponding GTP tunnel endpoint, i.e the information contained in the Transport Layer Address IE and the DL GTP TEID IE in the E-RAB To Be Modified List IE of the E-RAB MODIFICATION INDICATION message (see TS 36.413 [4]) depending on implementation choice;

-	the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE the UL Forwarding GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

-	the en-gNB shall use the S1 UL GTP Tunnel Endpoint IE of the SGNB ADDITION REQUEST message as the UL S1-U address.

-	the MeNB shall use the SgNB UL GTP Tunnel Endpoint at PDCP IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message as the UL X2-U address.

-	if the SGNB ADDITION REQUEST message contains for an E-RAB to be added which is requested to be configured with MCG resources the MeNB DL GTP Tunnel Endpoint at MCG IE the en-gNB shall use it as the DL X2-U address for delivery of DL PDCP PDUs.

-	the en-gNB shall include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the S1 DL GTP Tunnel Endpoint at the SgNB IE.

-	the en-gNB shall include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the RLC Mode IE.

-	the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE in the SGNB ADDITION REQUEST ACKNOWLEDGE the PDCP SN Length IE to indicate the PDCP SN length for that bearer.

-	If the RLC Mode IE is included for an E-RAB within the E-RABs To be Added List IE in the SGNB ADDITION REQUEST message, it indicates the mode that the MeNB used for the E-RAB when it was hosted at the MeNB.

-	If the Bearer Type IE for the concerned E-RAB is received by the en-gNB and is set to "non IP", the en-gNB shall, if supported,  not perform header compression for the concerned E-RAB.

Upon reception of the SGNB ADDITION REQUEST ACKNOWLEDGE message the MeNB shall stop the timer TDCprep.

If the SGNB ADDITION ACKNOWLEDGE message contains the SgNB Resource Coordination Information IE, the MeNB may use it for the purpose of resource coordination with the en-gNB. The MeNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The MeNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the SgNB Coordination Assistance Information IE is contained in the SgNB Resource Coordination Information IE, the MeNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the SgNB UE X2AP ID IE is contained in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, store this information and use it as defined in TS 37.340 [32].

If the SGNB ADDITION REQUEST message contains the SGNB Addition Trigger Indication, the en-gNB shall include the RRC config indication IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message to inform the MeNB if the en-gNB applied full or delta configuration, as specified in TS 37.340 [32].

If the en-gNB receives for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB ADDITION REQUEST message, it may provide the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message if PDCP duplication is configured at the en-gNB.

If the SGNB ADDITION REQUEST message contains the UL PDCP SN Length IE and the DL PDCP SN Length IE, the en-gNB shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

The SgNB may include the Location Information at SgNB IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message, if respective information is available at the SgNB.

If the Location Information at SgNB Reporting IE set to "pscell" is included in the SGNB ADDITION REQUEST, the SgNB shall start providing information about the current location of the UE. If the Location Information at SgNB IE is included in the SGNB ADDITION REQUEST ACKNOWLEDGE, the MeNB shall store the included information so that it may be transferred towards the MME.

If Trace Activation IE has previously been received for this UE, it shall be included in the SGNB ADDITION REQUEST message. If the Trace Activation IE is included in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, initiate the requested trace function as described in TS 32.422 [6].

If the SGNB ADDITION REQUEST message contains the Requested Fast MCG recovery via SRB3 IE, it indicates the MeNB needs fast MCG recovery via SRB3. If the SGNB ADDITION REQUEST ACKNOWLEDGE message contains the Fast MCG recovery via SRB3 enabled IE, it indicates the fast link recovery via SRB3 has been enabled.

Interactions with the MeNB initiated SgNB Modification procedure:

If the en-gNB provides for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message and the MeNB has not provided the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE in the SGNB ADDITION REQUEST message, the MeNB shall trigger the MeNB initiated SgNB Modification procedure to provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE to the SgNB.

Interactions with the SgNB Reconfiguration Completion procedure:

If the en-gNB admits at least one E-RAB, the en-gNB shall start the timer TDCoverall when sending the SGNB ADDITION REQUEST ACKNOWLEDGE message to the MeNB. The reception of the SGNB RECONFIGURATION COMPLETE message shall stop the timer TDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an SGNB ADDITION REQUEST message containing the Desired Activity Notification Level IE, the en-gNB shall, if supported, use this information to decide whether to trigger subsequent SgNB Activitity Notification procedures according to the requested notification level.

*** Next change, skipped text not changed ***

#### 8.7.6.2	Successful Operation

Figure 8.7.6.2-1: MeNB initiated SgNB Modification Preparation, successful operation

The MeNB initiates the procedure by sending the SGNB MODIFICATION REQUEST message to the en-gNB. When the MeNB sends the SGNB MODIFICATION REQUEST message, it shall start the timer TDCprep.

The SGNB MODIFICATION REQUEST message may contain:

-	within the UE Context Information IE (if the modification of the UE context at the en-gNB is requested);

-	E-RABs to be added within the E-RABs To Be Added Item IE;

-	E-RABs to be modified within the E-RABs To Be Modified Item IE;

-	E-RABs to be released within the E-RABs To Be Released Item IE;

-	the SgNB UE Aggregate Maximum Bit Rate IE;

-	the MeNB to SgNB Container IE;

-	the SCG Configuration Query IE;

-	the MeNB Resource Coordination Information IE;

-	the Requested split SRBs IE;

-	the Requested split SRBs release IE;

-	the Requested fast MCG recovery via SRB3 IE;

-	the Requested fast MCG recovery via SRB3 Release IE.

If the SGNB MODIFICATION REQUEST message contains the Serving PLMN IE, the en-gNB may use it for RRM purposes.

If the SGNB MODIFICATION REQUEST message contains the Handover Restriction List IE, the en-gNB shall

-	replace the previously provided Handover Restriction List by the received Handover Restriction List in the UE context;

-	use this information to select an appropriate NR cell.

If the SgNB UE Aggregate Maximum Bit Rate IE is included in the SGNB MODIFICATION REQUEST message, the en-gNB shall:

-	replace the previously provided SgNB UE Aggregate Maximum Bit Rate by the received SgNB UE Aggregate Maximum Bit Rate in the UE context;

-	use the received SgNB UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE as defined in TS 37.340 [32].

The allocation of resources according to the values of the QCI IE, Allocation and Retention Priority IE or GBR QoS Information IE included in the Full E-RAB Level QoS Parameters IE or in the Requested SCG E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If the SGNB MODIFICATION REQUEST message contains the MeNB Resource Coordination Information IE, the en-gNB should forward it to lower layers and it may use it for the purpose of resource coordination with the MeNB. The en-gNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The en-gNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the MeNB Coordination Assistance Information IE is contained in the MeNB Resource Coordination Information IE, the en-gNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If at least one of the requested modifications is admitted by the en-gNB, the en-gNB shall modify the related part of the UE context accordingly and send the SGNB MODIFICATION REQUEST ACKNOWLEDGE message back to the MeNB.

The en-gNB shall include the E-RABs for which resources have been either added or modified or released at the en-gNB either in the E-RABs Admitted To Be Added List IE or the E-RABs Admitted To Be Modified List IE or the E-RABs Admitted To Be Released List IE. The en-gNB shall include the E-RABs that have not been admitted in the E-RABs Not Admitted List IE with an appropriate cause value.

For each E-RAB successfully established or modified or released in the en-gNB, the en-gNB shall report to the MeNB, in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message, the same value in the EN-DC Resource Configuration IE as received in the SGNB MODIFICATION REQUEST message.

The en-gNB shall, if included, choose the ciphering algorithm based on the information in the NR UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the SgNB Security Key IE as specified in the TS 33.401 [18].

For each E-RAB for which allocation of the PDCP entity is requested at the en-gNB:

-	if applicable, the MeNB may propose to apply forwarding of downlink data by including the DL Forwarding IE within the E-RABs To Be Added Item IE of the SGNB MODIFICATION REQUEST message. For each E-RAB that it has decided to admit, the en-gNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs Admitted To Be Added Item IE of the SGNB MODIFICATION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. The MeNB may also provide for an applicable E-RAB to be released the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE of the SGNB MODIFICATION REQUEST message.

-	if applicable, the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the UL Forwarding GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

-	if applicable, the en-gNB may include for each bearer in the E-RABs Admitted To Be Modified List IE which is configured with the SN terminated split bearer option in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the UL Configuration IE to indicate that the MCG UL configuration of the UE has changed.

-	if applicable, the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the UL PDCP SN Length IE and the DL PDCP SN Length IE to indicate the PDCP SN length for that bearer.

-	If the Bearer Type IE for the concerned E-RAB is received by the en-gNB and is set to"non IP", then the en-gNB shall, if supported,  not perform header compression for the concerned E-RAB.

For each E-RAB configured with SCG resources and the PDCP entity is hosted by the MeNB and

-	requested to be modified,

-	if the SGNB MODIFICATION REQUEST message includes the MeNB UL GTP Tunnel Endpoint at PDCP IE in the E-RABs To Be Modified Item IE, the en-gNB shall act as specified in TS 37.340 [32].

-	if the SGNB MODIFICATION REQUEST message contains the MeNB UL GTP Tunnel Endpoint at PDCP IE the en-gNB shall use it as the new UL X2-U address.

-	the en-gNB may include in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the SgNB DL GTP Tunnel Endpoint at SCG IE.

If, dependent on the configured bearer type, the Full E-RAB Level QoS Parameters IE or the Maximum MCG admittable E-RAB Level QoS Parameters IE or the Requested SCG E-RAB level QoS Parameters IE are included in the SGNB MODIFICATION REQUEST message for an E-RAB to be modified the en-gNB shall allocate respective resources and provide corresponding radio configuration information within the SgNB to MeNB Container IE as described in TS 37.340 [32].

If the SGNB MODIFICATION REQUEST message contains, for an E-RAB to be modified which is configured with the PDCP entity in the en-gNB, the S1 UL GTP Tunnel Endpoint IE, the en-gNB shall use it as the new UL S1-U address.

If the SGNB MODIFICATION REQUEST message contains an E-RAB to be modified which is configured with the MN terminated split bearer option, the MeNB may include the UL Configuration IE to indicate that the SCG UL configuration of the UE has changed.

If the SGNB MODIFICATION REQUEST message contains for an E-RAB to be modified which is configured with the PDCP enitiy in the en-gNB and MCG resources the MeNB DL GTP Tunnel Endpoint at MCG IE the en-gNB shall use it as the DL X2-U address.

If the SGNB MODIFICATION REQUEST message contains the Subscriber Profile ID for RAT/Frequency Priority IE, the en-gNB may use it for RRM purposes.

If the SGNB MODIFICATION REQUEST message contains the Additional RRM Policy Index IE, the en-gNB may use it for RRM purposes.

For an E-RAB to be modified which is configured with the PDCP entity in the en-gNB the en-gNB may include in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the S1 DL GTP Tunnel Endpoint at the SgNB IE.

If the SGNB MODIFICATION REQUEST ACKNOWLEDGE message contains the SgNB Resource Coordination Information IE, the MeNB may use it for the purpose of resource coordination with the en-gNB. The MeNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The MeNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the SgNB Coordination Assistance Information IE is contained in the SgNB Resource Coordination Information IE, the MeNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

Upon reception of the SGNB MODIFICATION REQUEST ACKNOWLEDGE message the MeNB shall stop the timer TDCprep. If the SGNB MODIFICATION REQUEST ACKNOWLEDGE message has included the SgNB to MeNB Container IE the MeNB is then defined to have a Prepared SgNB Modification for that X2 UE-associated signalling.

If the SCG Configuration Query IE is included in the SGNB MODIFICATION REQUEST message, the en-gNB shall provide corresponding radio configuration information within the SgNB to MeNB Container IE as described in TS 37.340 [32].

If the SGNB MODIFICATION REQUEST message contains the Requested split SRBs IE, the en-gNB may use it to add split SRBs. If the SGNB MODIFICATION REQUEST message contains the Requested split SRBs release IE, the en-gNB may use it to release split SRBs.

If the SGNB MODIFICATION REQUEST message contains the Requested Fast MCG Recovery via SRB3 IE, it indicates the MeNB needs fast MCG recovery via SRB3. If the SGNB MODIFICATION REQUEST message contains the Requested Fast MCG Recovery via SRB3 Release IE, it indicates the MeNB does not need fast MCG recovery via SRB3. If the SGNB MODIFICATION REQUEST ACKNOWLEDGE message contains the Fast MCG recovery via SRB3 enabled IE or the Fast MCG recovery via SRB3 enabled IE, it indicates the fast link recovery via SRB3 has been enabled or disabled, correspondingly.

If the en-gNB receives for an E-RAB to be setup for which the PDCP entiy is allocated at the MeNB the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB MODIFICATION REQUEST message, it may provide the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message if PDCP duplication is configured at the en-gNB.

If the SGNB MODIFICATION REQUEST message contains the RLC Status IE, the en-gNB shall assume that RLC has been reestablished at the MeNB and may trigger PDCP data recovery.

If the en-gNB applied a full configuration or delta configuration, e.g. as part of a mobility procedure involving a change of DU, the en-gNB shall inform the MeNB by including the RRC config indication IE in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message.

If SGNB MODIFICATION REQUEST message contains the UL PDCP SN Length IE and the DL PDCP SN Length IE, the en-gNB shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

If the RLC Mode IE is included for an E-RAB within the E-RABs To be Added List IE in the SGNB MODIFICATION REQUEST message, it indicates the mode that the MeNB used for the E-RAB when it was hosted at the MeNB.

If the SGNB MODIFICATION REQUEST message contains the MeNB Cell ID IE, the en-gNB may search for the target NR cell among the NR neighbour cells of the E-UTRAN cell indicated in MeNB Cell ID IE, as specified in the TS 37.340 [32].

If the SGNB MODIFICATION REQUEST ACKNOWLEDGE message contains the RLC Status IE, the MeNB shall assume that RLC has been reestablished at the en-gNB and may trigger PDCP data recovery.

The SgNB may include the Location Information at SgNB IE in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message, if respective information is available at the SgNB.

If the Location Information at SgNB Reporting IE set to "pscell" is included in the SGNB MODIFICATION REQUEST, the SgNB shall start providing information about the current location of the UE. If the Location Information at SgNB IE is included in the SGNB MODIFICATION REQUEST ACKNOWLEDGE, the MeNB shall store the included information so that it may be transferred towards the MME.

If the Lower Layer presence status change IE set to "release lower layers" is included in the SGNB MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [32].

If the Lower Layer presence status change IE set to "re-establish lower layers" is included in the SGNB MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [32].

If the Lower Layer presence status change IE set to "suspend lower layers" is included in the SGNB MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [32].

If the Lower Layer presence status change IE set to "resume lower layers" is included in the SGNB MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [32].

Interactions with the MeNB initiated SgNB Modification procedure:

If the en-gNB provides for an E-RAB to be setup for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB MODIFICATION REQUEST ACKNOWLEDGE message and the MeNB has not provided the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB MODIFICATION REQUEST message, the MeNB shall trigger the MeNB initiated SgNB Modification procedure to provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE to the SgNB.

Interactions with the SgNB Reconfiguration Completion procedure:

If the en-gNB admits a modification of the UE context requiring the MeNB to report about the success of the RRC connection reconfiguration procedure, the en-gNB shall start the timer TDCoverall when sending the SGNB MODIFICATION REQUEST ACKNOWLEDGE message to the MeNB. The reception of the SGNB RECONFIGURATION COMPLETE message shall stop the timer TDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an SGNB MODIFICATION REQUEST message containing the Desired Activity Notification Level IE, the en-gNB shall, if supported, use this information to decide whether to trigger subsequent SgNB Activity Notification procedures, or stop or modify ongoing triggering of these procedures due to a previous request.

*** Next change, skipped text not changed ***

#### 8.7.7.2	Successful Operation

Figure 8.7.7.2-1: SgNB initiated SgNB Modification, successful operation.

The en-gNB initiates the procedure by sending the SGNB MODIFICATION REQUIRED message to the MeNB. When the en-gNB sends the SGNB MODIFICATION REQUIRED message, it shall start the timer TDCoverall.

The SGNB MODIFICATION REQUIRED message may contain

-	the PDCP Change Indication IE;

-	the SgNB to MeNB Container IE.

-	E-RABs to be modified within the E-RABs To Be Modified Item IE;

-	E-RABs to be released within the E-RABs To Be Released Item IE;

-	the SgNB Resource Coordination Information IE.

For the SN terminated split bearers, the en-gNB may include in the SGNB MODIFICATION REQUIRED message the UL Configuration IE to indicate that the MCG UL configuration of the UE has changed.

The en-gNB may include for each bearer in the E-RABs to Be Modified List IE in the SGNB MODIFICATION REQUIRED message the New DRB ID Request IE to request the MeNB to assign a new DRB ID for that bearer.

If the MeNB is able to perform the change requested by the en-gNB, the MeNB shall send the SGNB MODIFICATION CONFIRM message to the en-gNB. The SGNB MODIFICATION CONFIRM message may contain the MeNB to SgNB Container IE.

If the SGNB MODIFICATION REQUIRED message contains the SgNB Resource Coordination Information IE, the MeNB may use it for the purpose of resource coordination with the en-gNB. The MeNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The MeNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the SgNB Coordination Assistance Information IE is contained in the SgNB Resource Coordination Information IE, the MeNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the en-gNB applied a full configuration or delta configuration, e.g. as part of a mobility procedure involving a change of DU, the en-gNB shall inform the MeNB by including the RRC config indication IE in the SGNB MODIFICATION REQUIRED message.

For each E-RAB successfully modified or released as requested by the en-gNB, the MeNB shall inform the en-gNB, in the SGNB MODIFICATION CONFIRM message, the same value in the EN-DC Resource Configuration IE as received in the SGNB MODIFICATION REQUIRED message.

Upon reception of the SGNB MODIFICATION CONFIRM message the en-gNB shall stop the timer TDCoverall.

If the SGNB MODIFICATION CONFIRM message contains the MeNB Resource Coordination Information IE, the en-gNB should forward it to lower layers and it may use it for the purpose of resource coordination with the MeNB. The en-gNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The en-gNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the MeNB Coordination Assistance Information IE is contained in the MeNB Resource Coordination Information IE, the en-gNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the MeNB receives for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE in the SGNB MODIFICATION REQUIRED message, it shall provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE to the en-gNB in the SGNB MODIFICATION CONFIRM message. If the LCID IE is included in the SGNB MODIFICATION REQUIRED message, the MeNB should take it into account.

If the SGNB MODIFICATION REQUIRED message contains the RLC Status IE, the MeNB shall assume that RLC has been reestablished at the en-gNB and may trigger PDCP data recovery.

If the RLC Mode IE is included for an E-RAB within the E-RABs To Be Released List IE (for E-RABs hosted at the en-gNB) in the SGNB MODIFICATION REQUIRED message, it indicates the mode that the en-gNB used for the E-RAB when it was hosted at the en-gNB.

The MeNB shall include only E-RABs with the following IE in E-RABs Admitted To Be Modified List IE:

-	the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE.

If the Location Information at SgNB IE is included in the SGNB MODIFICATION REQUIRED, the MeNB shall store the included information so that it may be transferred towards the MME.

If the SGNB MODIFICATION REQUIRED message contains the Fast MCG recovery via SRB3 enabled IE or the Fast MCG recovery via SRB3 enabled IE, it indicates the fast link recovery via SRB3 has been enabled or disabled, correspondingly.

Interaction with the MeNB initiated SgNB Modification Preparation procedure:

If applicable, as specified in TS 37.340 [32], the en-gNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released List IE.

If applicable, as specified in TS 37.340 [32], the en-gNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the SgNB Security Key IE within the UE Context Information IE.

If applicable, as specified in TS 37.340 [32], the en-gNB may receive, after having initiated the SgNB initiated SgNB Modification procedure, the SGNB MODIFICATION REQUEST message including the measGapConfig IE as defined in TS 38.331 [31] within the MeNB to SgNB Container IE.

The en-gNB may receive, after having initiated the SgNB initiated SgNB modification procedure including the New DRB ID Request IE for an SN terminated bearer within the E-RABs To Be Modified List IE, the SGNB MODIFICATION REQUEST message to release and add the same bearer with a new DRB ID or with the same DRB ID but together with the SgNB Security Key IE within the UE Context Information IE.

*** Next change, skipped text not changed ***

#### 8.7.12.2	Successful Operation

Figure 8.7.12.2-1: RRC Transfer procedure, successful operation.

Either the MeNB initiates the procedure by sending the RRC TRANSFER message to the en-gNB or the en-gNB initiates the procedure by sending the RRC TRANSFER message to the MeNB.

If the en-gNB receives an RRC TRANSFER message which does not include the RRC Container IE in the Split SRB IE or the RRC container IE in NR UE Report IE, or the the RRC Container IE in the Fast MCG Recovery via SRB3 from MN to SN IE, or the the RRC Container IE in the Fast MCG Recovery via SRB3 from SN to MN IE, it shall ignore the message. If the en-gNB receives an RRC TRANSFER message with the Delivery Status IE, it shall ignore the message. If the en-gNB receives the RRC Container IE in the Split SRB IE, it shall deliver the contained PDCP-C PDU encapsulating an RRC message to the UE. If the en-gNB receives the RRC Container IE in the Fast MCG Recovery from MeNB to SgNB IE, the en-gNB shall, if supported, deliver the contained RRC Container encapsulating an RRC message to the UE.

If the MeNB receives the Delivery Status IE in the split SRB IE the MeNB shall consider RRC messages up to the indicated NR PDCP SN as having been successfully delivered (as defined in TS 36.322 [40]) to the UE by the en-gNB. If the MeNB receives the RRC Container IE in the Fast MCG Recovery from SgNB to MeNB IE, the MeNB shall, if supported, consider MCG link failure detected at the UE as specified in TS 37.340 [32].

*** Next change, skipped text not changed ***

#### 9.1.4.2	SGNB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to confirm the MeNB about the SgNB addition preparation.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | M |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>RLC Mode | C-ifMCGpresent |  | RLC Mode 9.2.119 | Indicates the RLC mode. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBRpresent |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication | – |  |
| >>>>LCID | O |  | 9.2.138 | LCID for the primary path in case of PDCP duplication | YES | ignore |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331[31]. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| Admitted split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between en-gNB and MeNB. | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Fast MCG recovery via SRB3 enabled | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifMCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| C-ifMCGandSCGpresent_GBRpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE. |

*** Next change, skipped text not changed ***

#### 9.1.4.6	SGNB MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to confirm the MeNB’s request to modify the en-gNB resources for a specific UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| E-RABs Admitted To Be Added List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>RLC Mode | C-ifMCGpresent |  | RLC Mode 9.2.119 | Indicates the RLC mode to be used at the assisting node. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBRpresent |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | M |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>LCID | O |  | 9.2.138 | LCID for the primary path in case of PDCP duplication configured. | YES | ignore |
| E-RABs Admitted To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | O |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | Endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication. | YES | ignore |
| >>>>RLC Status | O |  | 9.2.131 | Indicates the RLC has been re-established. | YES | ignore |
| E-RABs Admitted To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  | Note: no further information contained in the IE container |  |  |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the NR CG-Config message as defined in TS 38.331 [31]. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | ignore |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between en-gNB and MeNB. | YES | ignore |
| Admitted split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | ignore |
| Admitted split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs release | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Fast MCG recovery via SRB3 enabled | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Fast MCG recovery via SRB3 disabled | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is disabled. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| ifMCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| C-ifMCGandSCGpresent_GBRpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE. |

*** Next change, skipped text not changed ***

#### 9.1.4.8	SGNB MODIFICATION REQUIRED

This message is sent by the en-gNB to the MeNB to request the modification of en-gNB resources for a specific UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.109 |  | YES | ignore |
| E-RABs To Be Released List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Released Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>Cause | M |  | 9.2.6 |  | – |  |
| >>RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode at the en-gNB for PDCP transfer to MeNB. | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the NR CG-Config message as defined in TS 38.331 [31]. | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| E-RABs To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >E-RABs To Be Modified Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | O |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | O |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Shall be ignored by the MeNB if received. | YES | ignore |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | O |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>New DRB ID Request | O |  | ENUMERATED (True, …,) |  | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs for PDCP duplication. | – |  |
| >>>>RLC Status | O |  | 9.2.131 | Indicates the RLC has been re-established.. |  |  |
| >>>>LCID | O |  | 9.2.138 | Indicate the LCID of the primary path in case of PDCP duplication | YES | ignore |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between the en-gNB and the MeNB. | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Fast MCG recovery via SRB3 enabled | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is enabled. | YES | ignore |
| Fast MCG recovery via SRB3 disabled | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 is disabled. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

*** Next change, skipped text not changed ***

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

Cause,

CompositeAvailableCapacityGroup,

Correlation-ID,

COUNTvalue,

CellReportingIndicator,

AerialUEsubscriptionInformation,

CriticalityDiagnostics,

CRNTI,

CSGMembershipStatus,

CSG-Id,

DeactivationIndication,

DL-Forwarding,

DynamicDLTransmissionInformation,

ECGI,

E-RAB-ID,

E-RAB-Level-QoS-Parameters,

E-RAB-List,

EUTRANTraceID,

GlobalENB-ID,

GTPtunnelEndpoint,

GUGroupIDList,

GUMMEI,

HandoverReportType,

HandoverRestrictionList,

Masked-IMEISV,

InvokeIndication,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

MDT-Configuration,

ManagementBasedMDTallowed,

MDTPLMNList,

Neighbour-Information,

PCI,

PDCP-SN,

PLMN-Identity,

ReceiveStatusofULPDCPSDUs,

Registration-Request,

RelativeNarrowbandTxPower,

RadioResourceStatus,

RLC-Status,

RRCConnReestabIndicator,

RRCConnSetupIndicator,

UE-RLF-Report-Container,

UEAppLayerMeasConfig,

RRC-Context,

ServedCell-Information,

ServedCells,

ShortMAC-I,

SRVCCOperationPossible,

SubscriberProfileIDforRFP,

TargetCellInUTRAN,

TargeteNBtoSource-eNBTransparentContainer,

TimeToWait,

TraceActivation,

TraceDepth,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-HistoryInformation,

UE-HistoryInformationFromTheUE,

UE-S1AP-ID,

UESecurityCapabilities,

UEsToBeResetList,

UE-X2AP-ID,

UL-HighInterferenceIndicationInfo,

UL-InterferenceOverloadIndication,

HWLoadIndicator,

S1TNLLoadIndicator,

Measurement-ID,

ReportCharacteristics,

MobilityParametersInformation,

MobilityParametersModificationRange,

ReceiveStatusOfULPDCPSDUsExtended,

COUNTValueExtended,

SubframeAssignment,

ExtendedULInterferenceOverloadInfo,

ExpectedUEBehaviour,

SeNBSecurityKey,

MeNBtoSeNBContainer,

SeNBtoMeNBContainer,

SCGChangeIndication,

CoMPInformation,

ReportingPeriodicityRSRPMR,

RSRPMRList,

UE-RLF-Report-Container-for-extended-bands,

ProSeAuthorized,

CoverageModificationList,

ReportingPeriodicityCSIR,

CSIReportList,

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

COUNTvaluePDCP-SNlength18,

LHN-ID,

UE-ContextKeptIndicator,

UE-X2AP-ID-Extension,

SIPTOBearerDeactivationIndication,

TunnelInformation,

V2XServicesAuthorized,

X2BenefitValue,

ResumeID,

EUTRANCellIdentifier,

MakeBeforeBreakIndicator,

WTID,

WT-UE-XwAP-ID,

UESidelinkAggregateMaximumBitRate,

SgNBSecurityKey,

MeNBtoSgNBContainer,

SgNBtoMeNBContainer,

SplitSRBs,

RRCContainer,

SRBType,

GlobalGNB-ID,

GNB-ID,

SCGConfigurationQuery,

SplitSRB,

NRUeReport,

EN-DC-ResourceConfiguration,

TAC,

NRFreqInfo,

NRCGI,

NRPCI,

NRUESecurityCapabilities,

PDCPChangeIndication,

ULConfiguration,

SgNB-UE-X2AP-ID,

SecondaryRATUsageReportList,

ActivationID,

MeNBResourceCoordinationInformation,

SgNBResourceCoordinationInformation,

NR-TxBW,

BroadcastPLMNs-Item,

AdditionalPLMNs-Item,

RLCMode,

GBR-QosInformation,

DRB-ID,

FiveGS-TAC,

SULInformation,

Packet-LossRate,

ResourceType,

DataTrafficResourceIndication,

SpectrumSharingGroupID,

RRC-Config-Ind,

SGNB-Addition-Trigger-Ind,

UserPlaneTrafficActivityReport,

ERABActivityNotifyItemList,

PDCPSnLength,

Subscription-Based-UE-DifferentiationInfo,

LCID,

DuplicationActivation,

GNBOverloadInformation,

NewDRBIDrequest,

DesiredActNotificationLevel,

LocationInformationSgNB,

LocationInformationSgNBReporting,

EndcSONConfigurationTransfer,

NRNeighbour-Information,

InterfaceInstanceIndication,

BPLMN-ID-Info-NR,

AdditionalRRMPriorityIndex,

RequestedFastMCGRecoveryViaSRB3,

FastMCGRecoveryViaSRB3Enabled,

RequestedFastMCGRecoveryViaSRB3Release,

FastMCGRecoveryViaSRB3Disabled,

FastMCGRecovery,

PartialListIndicator,

MaximumCellListSize,

MessageOversizeNotification,

TNLConfigurationInfo

FROM X2AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

X2AP-PRIVATE-IES,

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES,

X2AP-PROTOCOL-IES-PAIR

FROM X2AP-Containers

id-ABSInformation,

id-ActivatedCellList,

id-BearerType,

id-Cause,

id-CellInformation,

id-CellInformation-Item,

id-CellMeasurementResult,

id-CellMeasurementResult-Item,

id-CellToReport,

id-CellToReport-Item,

id-CompositeAvailableCapacityGroup,

id-AerialUEsubscriptionInformation,

id-CriticalityDiagnostics,

id-DeactivationIndication,

id-DynamicDLTransmissionInformation,

id-E-RABs-Admitted-Item,

id-E-RABs-Admitted-List,

id-E-RABs-NotAdmitted-List,

id-E-RABs-SubjectToStatusTransfer-List,

id-E-RABs-SubjectToStatusTransfer-Item,

id-E-RABs-ToBeSetup-Item,

id-GlobalENB-ID,

id-GUGroupIDList,

id-GUGroupIDToAddList,

id-GUGroupIDToDeleteList,

id-GUMMEI-ID,

id-Masked-IMEISV,

id-InvokeIndication,

id-New-eNB-UE-X2AP-ID,

id-Old-eNB-UE-X2AP-ID,

id-Registration-Request,

id-ReportingPeriodicity,

id-RLC-Status,

id-ServedCells,

id-ServedCellsToActivate,

id-ServedCellsToAdd,

id-ServedCellsToModify,

id-ServedCellsToDelete,

id-SRVCCOperationPossible,

id-TargetCell-ID,

id-TargeteNBtoSource-eNBTransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-UE-ContextInformation,

id-UE-HistoryInformation,

id-UE-X2AP-ID,

id-Measurement-ID,

id-ReportCharacteristics,

id-ENB1-Measurement-ID,

id-ENB2-Measurement-ID,

id-ENB1-Cell-ID,

id-ENB2-Cell-ID,

id-ENB2-Proposed-Mobility-Parameters,

id-ENB1-Mobility-Parameters,

id-ENB2-Mobility-Parameters-Modification-Range,

id-FailureCellPCI,

id-Re-establishmentCellECGI,

id-FailureCellCRNTI,

id-ShortMAC-I,

id-SourceCellECGI,

id-FailureCellECGI,

id-HandoverReportType,

id-UE-RLF-Report-Container,

id-PartialSuccessIndicator,

id-MeasurementInitiationResult-List,

id-MeasurementInitiationResult-Item,

id-MeasurementFailureCause-Item,

id-CompleteFailureCauseInformation-List,

id-CompleteFailureCauseInformation-Item,

id-CSGMembershipStatus,

id-CSG-Id,

id-MDTConfiguration,

id-ManagementBasedMDTallowed,

id-ABS-Status,

id-RRCConnSetupIndicator,

id-RRCConnReestabIndicator,

id-TargetCellInUTRAN,

id-MobilityInformation,

id-SourceCellCRNTI,

id-ManagementBasedMDTPLMNList,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-IntendedULDLConfiguration,

id-ExtendedULInterferenceOverloadInfo,

id-RNL-Header,

id-x2APMessage,

id-UE-HistoryInformationFromTheUE,

id-ExpectedUEBehaviour,

id-MeNB-UE-X2AP-ID,

id-SeNB-UE-X2AP-ID,

id-UE-SecurityCapabilities,

id-SeNBSecurityKey,

id-SeNBUEAggregateMaximumBitRate,

id-ServingPLMN,

id-E-RABs-ToBeAdded-List,

id-E-RABs-ToBeAdded-Item,

id-MeNBtoSeNBContainer,

id-E-RABs-Admitted-ToBeAdded-List,

id-E-RABs-Admitted-ToBeAdded-Item,

id-SeNBtoMeNBContainer,

id-ResponseInformationSeNBReconfComp,

id-UE-ContextInformationSeNBModReq,

id-E-RABs-ToBeAdded-ModReqItem,

id-E-RABs-ToBeModified-ModReqItem,

id-E-RABs-ToBeReleased-ModReqItem,

id-E-RABs-Admitted-ToBeAdded-ModAckList,

id-E-RABs-Admitted-ToBeModified-ModAckList,

id-E-RABs-Admitted-ToBeReleased-ModAckList,

id-E-RABs-Admitted-ToBeAdded-ModAckItem,

id-E-RABs-Admitted-ToBeModified-ModAckItem,

id-E-RABs-Admitted-ToBeReleased-ModAckItem,

id-SCGChangeIndication,

id-E-RABs-ToBeReleased-ModReqd,

id-E-RABs-ToBeReleased-ModReqdItem,

id-E-RABs-ToBeReleased-List-RelReq,

id-E-RABs-ToBeReleased-RelReqItem,

id-E-RABs-ToBeReleased-List-RelConf,

id-E-RABs-ToBeReleased-RelConfItem,

id-E-RABs-SubjectToCounterCheck-List,

id-E-RABs-SubjectToCounterCheckItem,

id-CoMPInformation,

id-ReportingPeriodicityRSRPMR,

id-RSRPMRList,

id-UE-RLF-Report-Container-for-extended-bands,

id-ProSeAuthorized,

id-CoverageModificationList,

id-ReportingPeriodicityCSIR,

id-CSIReportList,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-LHN-ID,

id-Correlation-ID,

id-SIPTO-Correlation-ID,

id-UE-ContextReferenceAtSeNB,

id-UE-ContextReferenceAtWT,

id-UE-ContextKeptIndicator,

id-UEs-ToBeReset,

id-UEs-Admitted-ToBeReset,

id-WT-UE-ContextKeptIndicator,

id-New-eNB-UE-X2AP-ID-Extension,

id-Old-eNB-UE-X2AP-ID-Extension,

id-MeNB-UE-X2AP-ID-Extension,

id-SeNB-UE-X2AP-ID-Extension,

id-SIPTO-BearerDeactivationIndication,

id-Tunnel-Information-for-BBF,

id-SIPTO-L-GW-TransportLayerAddress,

id-GW-TransportLayerAddress,

id-X2RemovalThreshold,

id-CellReportingIndicator,

id-V2XServicesAuthorized,

id-resumeID,

id-UE-ContextInformationRetrieve,

id-E-RABs-ToBeSetupRetrieve-Item,

id-NewEUTRANCellIdentifier,

id-MakeBeforeBreakIndicator,

id-UESidelinkAggregateMaximumBitRate,

id-uL-GTPtunnelEndpoint,

id-SgNBSecurityKey,

id-SgNBUEAggregateMaximumBitRate,

id-E-RABs-ToBeAdded-SgNBAddReqList,

id-MeNBtoSgNBContainer,

id-SgNB-UE-X2AP-ID,

id-RequestedSplitSRBs,

id-E-RABs-ToBeAdded-SgNBAddReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList,

id-SgNBtoMeNBContainer,

id-AdmittedSplitSRBs,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item,

id-ResponseInformationSgNBReconfComp,

id-UE-ContextInformation-SgNBModReq,

id-E-RABs-ToBeAdded-SgNBModReq-Item,

id-E-RABs-ToBeModified-SgNBModReq-Item,

id-E-RABs-ToBeReleased-SgNBModReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList,

id-E-RABs-Admitted-ToBeModified-SgNBModAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList,

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item,

id-E-RABs-ToBeReleased-SgNBModReqdList,

id-E-RABs-ToBeModified-SgNBModReqdList,

id-E-RABs-ToBeReleased-SgNBModReqd-Item,

id-E-RABs-ToBeModified-SgNBModReqd-Item,

id-E-RABs-ToBeReleased-SgNBChaConfList,

id-E-RABs-ToBeReleased-SgNBChaConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqList,

id-E-RABs-ToBeReleased-SgNBRelReq-Item,

id-E-RABs-ToBeReleased-SgNBRelConfList,

id-E-RABs-ToBeReleased-SgNBRelConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqdList,

id-E-RABs-ToBeReleased-SgNBRelReqd-Item,

id-E-RABs-SubjectToSgNBCounterCheck-List,

id-E-RABs-SubjectToSgNBCounterCheck-Item,

id-Target-SgNB-ID,

id-RRCContainer,

id-SRBType,

id-HandoverRestrictionList,

id-SCGConfigurationQuery,

id-SplitSRB,

id-NRUeReport,

id-InitiatingNodeType-EndcX2Setup,

id-InitiatingNodeType-EndcConfigUpdate,

id-RespondingNodeType-EndcX2Setup,

id-RespondingNodeType-EndcConfigUpdate,

id-NRUESecurityCapabilities,

id-PDCPChangeIndication,

id-ServedEUTRAcellsENDCX2ManagementList,

id-ServedEUTRAcellsToModifyListENDCConfUpd,

id-ServedEUTRAcellsToDeleteListENDCConfUpd,

id-ServedNRcellsToModifyListENDCConfUpd,

id-ServedNRcellsToDeleteListENDCConfUpd,

id-CellAssistanceInformation,

id-Globalen-gNB-ID,

id-ServedNRcellsENDCX2ManagementList,

id-Old-SgNB-UE-X2AP-ID,

id-UE-ContextReferenceAtSgNB,

id-SecondaryRATUsageReportList,

id-ActivationID,

id-ServedNRCellsToActivate,

id-ActivatedNRCellList,

id-MeNBResourceCoordinationInformation,

id-SgNBResourceCoordinationInformation,

id-UEAppLayerMeasConfig,

id-SelectedPLMN,

id-SubscriberProfileIDforRFP,

id-InitiatingNodeType-EutranrCellResourceCoordination,

id-RespondingNodeType-EutranrCellResourceCoordination,

id-DataTrafficResourceIndication,

id-SpectrumSharingGroupID,

id-ListofEUTRACellsinEUTRACoordinationReq,

id-ListofEUTRACellsinEUTRACoordinationResp,

id-ListofEUTRACellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationResp,

id-RRCConfigIndication,

id-SGNB-Addition-Trigger-Ind,

id-RequestedSplitSRBsrelease,

id-AdmittedSplitSRBsrelease,

id-E-RABs-AdmittedToBeModified-SgNBModConfList,

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item,

id-UEContextLevelUserPlaneActivity,

id-ERABActivityNotifyItemList,

id-MeNBCell-ID,

id-InitiatingNodeType-EndcX2Removal,

id-RespondingNodeType-EndcX2Removal,

id-uLpDCPSnLength,

id-dL-Forwarding,

id-E-RABs-DataForwardingAddress-List,

id-E-RABs-DataForwardingAddress-Item,

id-Subscription-Based-UE-DifferentiationInfo,

id-RLCMode-transferred,

id-dLPDCPSnLength,

id-secondarysgNBDLGTPTEIDatPDCP,

id-secondarymeNBULGTPTEIDatPDCP,

id-lCID,

id-duplicationActivation,

id-GNBOverloadInformation,

id-new-drb-ID-req,

id-NRNeighbourInfoToModify,

id-DesiredActNotificationLevel,

id-LocationInformationSgNB,

id-LocationInformationSgNBReporting,

id-endcSONConfigurationTransfer,

id-EUTRANTraceID,

id-additionalPLMNs-Item,

id-InterfaceInstanceIndication,

id-BPLMN-ID-Info-NR,

id-ERABs-transferred-to-MeNB,

id-AdditionalRRMPriorityIndex,

id-LowerLayerPresenceStatusChange,

id-FastMCGRecovery-SN-to-MN,

id-FastMCGRecovery-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-FastMCGRecoveryViaSRB3Enabled,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-FastMCGRecoveryViaSRB3Disabled,

id-PartialListIndicator,

id-MaximumCellListSize,

id-MessageOversizeNotification,

id-CellandCapacityAssistInfo,

id-TNLConfigurationInfo,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands

FROM X2AP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory}|

{ ID id-TargetCell-ID								CRITICALITY reject	TYPE ECGI													PRESENCE mandatory}|

{ ID id-GUMMEI-ID									CRITICALITY reject	TYPE GUMMEI													PRESENCE mandatory}|

{ ID id-UE-ContextInformation						CRITICALITY reject	TYPE UE-ContextInformation								PRESENCE mandatory}|

{ ID id-UE-HistoryInformation						CRITICALITY ignore	TYPE UE-HistoryInformation								PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible								PRESENCE optional}|

{ ID id-CSGMembershipStatus							CRITICALITY reject	TYPE CSGMembershipStatus									PRESENCE optional}|

{ ID id-MobilityInformation							CRITICALITY ignore	TYPE MobilityInformation									PRESENCE optional}|

{ ID id-Masked-IMEISV								CRITICALITY ignore	TYPE Masked-IMEISV											PRESENCE optional}|

{ ID id-UE-HistoryInformationFromTheUE				CRITICALITY ignore	TYPE UE-HistoryInformationFromTheUE					PRESENCE optional}|

{ ID id-ExpectedUEBehaviour							CRITICALITY ignore	TYPE ExpectedUEBehaviour									PRESENCE optional}|

{ ID id-ProSeAuthorized								CRITICALITY ignore	TYPE ProSeAuthorized										PRESENCE optional}|

{ ID id-UE-ContextReferenceAtSeNB					CRITICALITY ignore	TYPE UE-ContextReferenceAtSeNB							PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-V2XServicesAuthorized						CRITICALITY ignore	TYPE V2XServicesAuthorized								PRESENCE optional}|

{ ID id-UE-ContextReferenceAtWT						CRITICALITY ignore	TYPE UE-ContextReferenceAtWT							PRESENCE optional}|

{ ID id-NRUESecurityCapabilities						CRITICALITY ignore	TYPE NRUESecurityCapabilities							PRESENCE optional}|

{ ID id-UE-ContextReferenceAtSgNB					CRITICALITY ignore	TYPE UE-ContextReferenceAtSgNB							PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation				CRITICALITY ignore	TYPE AerialUEsubscriptionInformation					PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo	CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo	PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex					CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional},

...

}

UE-ContextInformation ::= SEQUENCE {

mME-UE-S1AP-ID						UE-S1AP-ID,

uESecurityCapabilities				UESecurityCapabilities,

aS-SecurityInformation				AS-SecurityInformation,

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

e-RABs-ToBeSetup-List				E-RABs-ToBeSetup-List,

rRC-Context							RRC-Context,

handoverRestrictionList				HandoverRestrictionList		OPTIONAL,

locationReportingInformation		LocationReportingInformation	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-ContextInformation-ExtIEs} } OPTIONAL,

...

}

UE-ContextInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ManagementBasedMDTallowed			CRITICALITY ignore	EXTENSION ManagementBasedMDTallowed					PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore	EXTENSION MDTPLMNList									PRESENCE optional }|

{ ID id-UESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION UESidelinkAggregateMaximumBitRate		PRESENCE optional},

...

}

E-RABs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeSetup-ItemIEs} }

E-RABs-ToBeSetup-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeSetup-Item	 CRITICALITY ignore 	TYPE E-RABs-ToBeSetup-Item 	PRESENCE mandatory },

...

}

E-RABs-ToBeSetup-Item ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

dL-Forwarding					DL-Forwarding													OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeSetup-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeSetup-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-BearerType		CRITICALITY reject	EXTENSION BearerType		PRESENCE optional},

...

}

MobilityInformation ::= BIT STRING (SIZE(32))

UE-ContextReferenceAtSeNB ::= SEQUENCE {

source-GlobalSeNB-ID		GlobalENB-ID,

seNB-UE-X2AP-ID				UE-X2AP-ID,

seNB-UE-X2AP-ID-Extension	UE-X2AP-ID-Extension,

iE-Extensions				ProtocolExtensionContainer { {UE-ContextReferenceAtSeNB-ItemExtIEs} } OPTIONAL,

...

}

UE-ContextReferenceAtSeNB-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UE-ContextReferenceAtWT ::= SEQUENCE {

wTID					WTID,

wT-UE-XwAP-ID			WT-UE-XwAP-ID,

iE-Extensions			ProtocolExtensionContainer { {UE-ContextReferenceAtWT-ItemExtIEs} } OPTIONAL,

...

}

UE-ContextReferenceAtWT-ItemExtIEs	X2AP-PROTOCOL-EXTENSION ::= {

...

}

UE-ContextReferenceAtSgNB ::= SEQUENCE {

source-GlobalSgNB-ID			GlobalGNB-ID,

sgNB-UE-X2AP-ID				SgNB-UE-X2AP-ID,

iE-Extensions				ProtocolExtensionContainer { {UE-ContextReferenceAtSgNB-ItemExtIEs} } OPTIONAL,

...

}

UE-ContextReferenceAtSgNB-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverRequestAcknowledge-IEs}},

...

}

HandoverRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID		PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID		PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-List						CRITICALITY ignore	TYPE E-RABs-Admitted-List	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List		PRESENCE optional}|

{ ID id-TargeteNBtoSource-eNBTransparentContainer	CRITICALITY ignore	TYPE TargeteNBtoSource-eNBTransparentContainer	PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator						CRITICALITY ignore	TYPE UE-ContextKeptIndicator	PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension	PRESENCE optional}-- The id-SeNB-UE-X2AP-ID-Extension shall not be sent shall not be sent and shall be ignored, if received.--|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-WT-UE-ContextKeptIndicator					CRITICALITY ignore	TYPE UE-ContextKeptIndicator	PRESENCE optional}|

{ ID id-ERABs-transferred-to-MeNB					CRITICALITY ignore	TYPE E-RAB-List			PRESENCE optional},

...

}

E-RABs-Admitted-List 		::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ItemIEs} }

E-RABs-Admitted-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-Item 	PRESENCE mandatory	}

}

E-RABs-Admitted-Item ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

uL-GTP-TunnelEndpoint			GTPtunnelEndpoint	OPTIONAL,

dL-GTP-TunnelEndpoint			GTPtunnelEndpoint	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-Item-ExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverPreparationFailure-IEs}},

...

}

HandoverPreparationFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER REPORT

--

-- **************************************************************

HandoverReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverReport-IEs}},

...

}

HandoverReport-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-HandoverReportType							CRITICALITY ignore	TYPE HandoverReportType									PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory}|

{ ID id-SourceCellECGI								CRITICALITY ignore	TYPE ECGI													PRESENCE mandatory}|

{ ID id-FailureCellECGI								CRITICALITY ignore	TYPE ECGI													PRESENCE mandatory}|

{ ID id-Re-establishmentCellECGI					CRITICALITY ignore	TYPE ECGI													PRESENCE conditional} -- The IE shall be present if the Handover Report Type IE is set to “HO to Wrong Cell” -- |

{ ID id-TargetCellInUTRAN							CRITICALITY ignore	TYPE TargetCellInUTRAN									PRESENCE conditional} -- The IE shall be present if the Handover Report Type IE is set to "InterRAT ping-pong" --|

{ ID id-SourceCellCRNTI								CRITICALITY ignore	TYPE CRNTI													PRESENCE optional}|

{ ID id-MobilityInformation							CRITICALITY ignore	TYPE MobilityInformation									PRESENCE optional}|

{ ID id-UE-RLF-Report-Container						CRITICALITY ignore	TYPE UE-RLF-Report-Container							PRESENCE optional}|

{ ID id-UE-RLF-Report-Container-for-extended-bands	CRITICALITY ignore	TYPE UE-RLF-Report-Container-for-extended-bands	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-E-RABs-SubjectToStatusTransfer-List		CRITICALITY ignore	TYPE E-RABs-SubjectToStatusTransfer-List		PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE optional},

...

}

E-RABs-SubjectToStatusTransfer-List	::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-SubjectToStatusTransfer-ItemIEs} }

E-RABs-SubjectToStatusTransfer-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-SubjectToStatusTransfer-Item	CRITICALITY ignore	TYPE E-RABs-SubjectToStatusTransfer-Item 	PRESENCE mandatory	}

}

E-RABs-SubjectToStatusTransfer-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

receiveStatusofULPDCPSDUs				ReceiveStatusofULPDCPSDUs			OPTIONAL,

uL-COUNTvalue					COUNTvalue,

dL-COUNTvalue					COUNTvalue,

iE-Extensions							ProtocolExtensionContainer { {E-RABs-SubjectToStatusTransfer-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-SubjectToStatusTransfer-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ReceiveStatusOfULPDCPSDUsExtended			CRITICALITY ignore	EXTENSION ReceiveStatusOfULPDCPSDUsExtended			PRESENCE optional}|

{ ID id-ULCOUNTValueExtended						CRITICALITY ignore	EXTENSION COUNTValueExtended								PRESENCE optional}|

{ ID id-DLCOUNTValueExtended						CRITICALITY ignore	EXTENSION COUNTValueExtended								PRESENCE optional}|

{ ID id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18	CRITICALITY ignore	EXTENSION ReceiveStatusOfULPDCPSDUsPDCP-SNlength18	PRESENCE optional}|

{ ID id-ULCOUNTValuePDCP-SNlength18					CRITICALITY ignore	EXTENSION COUNTvaluePDCP-SNlength18						PRESENCE optional}|

{ ID id-DLCOUNTValuePDCP-SNlength18					CRITICALITY ignore	EXTENSION COUNTvaluePDCP-SNlength18						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UE CONTEXT RELEASE

--

-- **************************************************************

UEContextRelease ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{UEContextRelease-IEs}},

...

}

UEContextRelease-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SIPTO-BearerDeactivationIndication	CRITICALITY ignore	TYPE SIPTOBearerDeactivationIndication		PRESENCE optional}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{HandoverCancel-IEs}},

...

}

HandoverCancel-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID							PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID							PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause								PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-Old-SgNB-UE-X2AP-ID					CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID					PRESENCE optional}|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESET REQUEST

--

-- **************************************************************

ResetRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResetRequest-IEs}},

...

}

ResetRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESET RESPONSE

--

-- **************************************************************

ResetResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResetResponse-IEs}},

...

}

ResetResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 SETUP REQUEST

--

-- **************************************************************

X2SetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2SetupRequest-IEs}},

...

}

X2SetupRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID			CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory}|

{ ID id-ServedCells				CRITICALITY reject	TYPE ServedCells			PRESENCE mandatory}|

{ ID id-GUGroupIDList			CRITICALITY reject	TYPE GUGroupIDList			PRESENCE optional}|

{ ID id-LHN-ID					CRITICALITY ignore	TYPE LHN-ID					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 SETUP RESPONSE

--

-- **************************************************************

X2SetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2SetupResponse-IEs}},

...

}

X2SetupResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID				CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory}|

{ ID id-ServedCells					CRITICALITY reject	TYPE ServedCells			PRESENCE mandatory}|

{ ID id-GUGroupIDList				CRITICALITY reject	TYPE GUGroupIDList			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional}|

{ ID id-LHN-ID						CRITICALITY ignore	TYPE LHN-ID					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 SETUP FAILURE

--

-- **************************************************************

X2SetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2SetupFailure-IEs}},

...

}

X2SetupFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause					CRITICALITY ignore		TYPE Cause								PRESENCE mandatory} |

{ ID id-TimeToWait				CRITICALITY ignore		TYPE TimeToWait							PRESENCE optional} |

{ ID id-CriticalityDiagnostics	CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- LOAD INFORMATION

--

-- **************************************************************

LoadInformation ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{LoadInformation-IEs}},

...

}

LoadInformation-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellInformation				CRITICALITY ignore	TYPE CellInformation-List		PRESENCE mandatory} ,

...

}

CellInformation-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CellInformation-ItemIEs} }

CellInformation-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellInformation-Item	CRITICALITY ignore	TYPE CellInformation-Item 	PRESENCE mandatory	}

}

CellInformation-Item ::= SEQUENCE {

cell-ID							ECGI,

ul-InterferenceOverloadIndication		UL-InterferenceOverloadIndication		OPTIONAL,

ul-HighInterferenceIndicationInfo		UL-HighInterferenceIndicationInfo		OPTIONAL,

relativeNarrowbandTxPower				RelativeNarrowbandTxPower				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {CellInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CellInformation-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ABSInformation						CRITICALITY ignore	EXTENSION ABSInformation							PRESENCE optional }|

{ ID id-InvokeIndication					CRITICALITY ignore	EXTENSION InvokeIndication							PRESENCE optional }|

{ ID id-IntendedULDLConfiguration			CRITICALITY ignore	EXTENSION SubframeAssignment						PRESENCE optional }|

{ ID id-ExtendedULInterferenceOverloadInfo	CRITICALITY ignore	EXTENSION ExtendedULInterferenceOverloadInfo	PRESENCE optional }|

{ ID id-CoMPInformation						CRITICALITY ignore	EXTENSION CoMPInformation							PRESENCE optional }|

{ ID id-DynamicDLTransmissionInformation	CRITICALITY ignore	EXTENSION DynamicDLTransmissionInformation		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- ENB CONFIGURATION UPDATE

--

-- **************************************************************

ENBConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENBConfigurationUpdate-IEs}},

...

}

ENBConfigurationUpdate-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToAdd			CRITICALITY reject	TYPE ServedCells					PRESENCE optional}|

{ ID id-ServedCellsToModify			CRITICALITY reject	TYPE ServedCellsToModify			PRESENCE optional}|

{ ID id-ServedCellsToDelete			CRITICALITY reject	TYPE Old-ECGIs						PRESENCE optional}|

{ ID id-GUGroupIDToAddList			CRITICALITY reject	TYPE GUGroupIDList					PRESENCE optional}|

{ ID id-GUGroupIDToDeleteList		CRITICALITY reject	TYPE GUGroupIDList					PRESENCE optional}|

{ ID id-CoverageModificationList	CRITICALITY reject	TYPE CoverageModificationList		PRESENCE optional},

...

}

ServedCellsToModify::= SEQUENCE (SIZE (1..maxCellineNB)) OF ServedCellsToModify-Item

ServedCellsToModify-Item::= SEQUENCE {

old-ecgi						ECGI,

servedCellInfo					ServedCell-Information,

neighbour-Info					Neighbour-Information			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToModify-Item-ExtIEs} } OPTIONAL,

...

}

ServedCellsToModify-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-DeactivationIndication			CRITICALITY ignore	EXTENSION DeactivationIndication				PRESENCE optional }|

{ ID id-NRNeighbourInfoToModify			CRITICALITY ignore	EXTENSION NRNeighbour-Information				PRESENCE optional },

...

}

Old-ECGIs::= SEQUENCE (SIZE (1..maxCellineNB)) OF ECGI

-- **************************************************************

--

#### -- ENB CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

ENBConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENBConfigurationUpdateAcknowledge-IEs}},

...

}

ENBConfigurationUpdateAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ENB CONFIGURATION UPDATE FAIURE

--

-- **************************************************************

ENBConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENBConfigurationUpdateFailure-IEs}},

...

}

ENBConfigurationUpdateFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusRequest-IEs}},

...

}

ResourceStatusRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID			CRITICALITY ignore	TYPE Measurement-ID					PRESENCE conditional}|-- The IE shall be present if the Registration Request IE is set to “Stop”, “Partial stop” or to “Add”--

{ ID id-Registration-Request		CRITICALITY reject	TYPE Registration-Request			PRESENCE mandatory}|

{ ID id-ReportCharacteristics		CRITICALITY reject	TYPE ReportCharacteristics			PRESENCE optional}|

{ ID id-CellToReport				CRITICALITY ignore	TYPE CellToReport-List				PRESENCE mandatory}|

{ ID id-ReportingPeriodicity		CRITICALITY ignore	TYPE ReportingPeriodicity			PRESENCE optional}|

{ ID id-PartialSuccessIndicator		CRITICALITY ignore	TYPE PartialSuccessIndicator		PRESENCE optional}|

{ ID id-ReportingPeriodicityRSRPMR	CRITICALITY ignore	TYPE ReportingPeriodicityRSRPMR		PRESENCE optional}|

{ ID id-ReportingPeriodicityCSIR	CRITICALITY ignore	TYPE ReportingPeriodicityCSIR		PRESENCE optional},

...

}

CellToReport-List 		::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CellToReport-ItemIEs} }

CellToReport-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellToReport-Item	CRITICALITY ignore	TYPE CellToReport-Item 	PRESENCE mandatory}

}

CellToReport-Item ::= SEQUENCE {

cell-ID									ECGI,

iE-Extensions							ProtocolExtensionContainer { {CellToReport-Item-ExtIEs} } OPTIONAL,

...

}

CellToReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ReportingPeriodicity ::= ENUMERATED {

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

PartialSuccessIndicator ::= ENUMERATED {

partial-success-allowed,

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS RESPONSE

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusResponse-IEs}},

...

}

ResourceStatusResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional}|

{ ID id-MeasurementInitiationResult-List	CRITICALITY ignore	TYPE MeasurementInitiationResult-List	PRESENCE optional},

...

}

MeasurementInitiationResult-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {MeasurementInitiationResult-ItemIEs} }

MeasurementInitiationResult-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeasurementInitiationResult-Item	CRITICALITY ignore	TYPE MeasurementInitiationResult-Item	PRESENCE mandatory}

}

MeasurementInitiationResult-Item ::= SEQUENCE {

cell-ID											ECGI,

measurementFailureCause-List					MeasurementFailureCause-List	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {MeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

MeasurementInitiationResult-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementFailureCause-List ::= SEQUENCE (SIZE (1..maxFailedMeasObjects)) OF ProtocolIE-Single-Container { {MeasurementFailureCause-ItemIEs} }

MeasurementFailureCause-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeasurementFailureCause-Item	CRITICALITY ignore	TYPE MeasurementFailureCause-Item		PRESENCE mandatory}

}

MeasurementFailureCause-Item ::= SEQUENCE {

measurementFailedReportCharacteristics			ReportCharacteristics,

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { {MeasurementFailureCause-Item-ExtIEs} }	OPTIONAL,

...

}

MeasurementFailureCause-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS FAILURE

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusFailure-IEs}},

...

}

ResourceStatusFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID						CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID						CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-CompleteFailureCauseInformation-List	CRITICALITY ignore	TYPE CompleteFailureCauseInformation-List	PRESENCE optional},

...

}

CompleteFailureCauseInformation-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CompleteFailureCauseInformation-ItemIEs} }

CompleteFailureCauseInformation-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CompleteFailureCauseInformation-Item	CRITICALITY ignore	TYPE CompleteFailureCauseInformation-Item	PRESENCE mandatory}

}

CompleteFailureCauseInformation-Item ::= SEQUENCE {

cell-ID											ECGI,

measurementFailureCause-List					MeasurementFailureCause-List,

iE-Extensions									ProtocolExtensionContainer { {CompleteFailureCauseInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CompleteFailureCauseInformation-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ResourceStatusUpdate-IEs}},

...

}

ResourceStatusUpdate-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-ENB2-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-CellMeasurementResult	CRITICALITY ignore	TYPE CellMeasurementResult-List		PRESENCE mandatory},

...

}

CellMeasurementResult-List ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ProtocolIE-Single-Container { {CellMeasurementResult-ItemIEs} }

CellMeasurementResult-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellMeasurementResult-Item	CRITICALITY ignore	TYPE CellMeasurementResult-Item	PRESENCE mandatory}

}

CellMeasurementResult-Item ::= SEQUENCE {

cell-ID						ECGI,

hWLoadIndicator				HWLoadIndicator		OPTIONAL,

s1TNLLoadIndicator			S1TNLLoadIndicator	OPTIONAL,

radioResourceStatus			RadioResourceStatus	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {CellMeasurementResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementResult-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-CompositeAvailableCapacityGroup	CRITICALITY ignore	EXTENSION CompositeAvailableCapacityGroup		PRESENCE optional}|

{ ID id-ABS-Status						CRITICALITY ignore	EXTENSION ABS-Status								PRESENCE optional}|

{ ID id-RSRPMRList						CRITICALITY ignore	EXTENSION RSRPMRList								PRESENCE optional}|

{ ID id-CSIReportList					CRITICALITY ignore	EXTENSION CSIReportList								PRESENCE optional}|

{ ID id-CellReportingIndicator			CRITICALITY ignore	EXTENSION CellReportingIndicator					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs X2AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE REQUEST

--

-- **************************************************************

MobilityChangeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeRequest-IEs}},

...

}

MobilityChangeRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Cell-ID						CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-ENB2-Cell-ID						CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-ENB1-Mobility-Parameters			CRITICALITY ignore	TYPE MobilityParametersInformation				PRESENCE optional}|

{ ID id-ENB2-Proposed-Mobility-Parameters	CRITICALITY reject	TYPE MobilityParametersInformation				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause											PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE ACKNOWLEDGE

--

-- **************************************************************

MobilityChangeAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeAcknowledge-IEs}},

...

}

MobilityChangeAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Cell-ID			CRITICALITY reject	TYPE ECGI							PRESENCE mandatory}|

{ ID id-ENB2-Cell-ID			CRITICALITY reject	TYPE ECGI							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MOBILITY CHANGE FAILURE

--

-- **************************************************************

MobilityChangeFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MobilityChangeFailure-IEs}},

...

}

MobilityChangeFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ENB1-Cell-ID									CRITICALITY ignore	TYPE ECGI												PRESENCE mandatory}|

{ ID id-ENB2-Cell-ID									CRITICALITY ignore	TYPE ECGI												PRESENCE mandatory}|

{ ID id-Cause											CRITICALITY ignore	TYPE Cause												PRESENCE mandatory}|

{ ID id-ENB2-Mobility-Parameters-Modification-Range		CRITICALITY ignore	TYPE MobilityParametersModificationRange		PRESENCE optional}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RADIO LINK FAILURE INDICATION

--

-- **************************************************************

RLFIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{RLFIndication-IEs}},

...

}

RLFIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-FailureCellPCI								CRITICALITY ignore	TYPE PCI														PRESENCE mandatory}|

{ ID id-Re-establishmentCellECGI					CRITICALITY ignore	TYPE ECGI														PRESENCE mandatory}|

{ ID id-FailureCellCRNTI							CRITICALITY ignore	TYPE CRNTI														PRESENCE mandatory}|

{ ID id-ShortMAC-I									CRITICALITY ignore	TYPE ShortMAC-I													PRESENCE optional}|

{ ID id-UE-RLF-Report-Container						CRITICALITY ignore	TYPE UE-RLF-Report-Container								PRESENCE optional}|

{ ID id-RRCConnSetupIndicator						CRITICALITY reject	TYPE RRCConnSetupIndicator									PRESENCE optional}|

{ ID id-RRCConnReestabIndicator						CRITICALITY ignore	TYPE RRCConnReestabIndicator								PRESENCE optional}|

{ ID id-UE-RLF-Report-Container-for-extended-bands	CRITICALITY ignore	TYPE UE-RLF-Report-Container-for-extended-bands		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION REQUEST

--

-- **************************************************************

CellActivationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CellActivationRequest-IEs}},

...

}

CellActivationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedCellsToActivate 	CRITICALITY reject	TYPE ServedCellsToActivate		PRESENCE mandatory},

...

}

ServedCellsToActivate::= SEQUENCE (SIZE (1..maxCellineNB)) OF ServedCellsToActivate-Item

ServedCellsToActivate-Item::= SEQUENCE {

ecgi						ECGI,

iE-Extensions					ProtocolExtensionContainer { {ServedCellsToActivate-Item-ExtIEs} } OPTIONAL,

...

}

ServedCellsToActivate-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- CELL ACTIVATION RESPONSE

--

-- **************************************************************

CellActivationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CellActivationResponse-IEs}},

...

}

CellActivationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ActivatedCellList		CRITICALITY ignore	TYPE ActivatedCellList				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

ActivatedCellList ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ActivatedCellList-Item

ActivatedCellList-Item::= SEQUENCE {

ecgi								ECGI,

iE-Extensions						ProtocolExtensionContainer { {ActivatedCellList-Item-ExtIEs} } OPTIONAL,

...

}

ActivatedCellList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

--**************************************************************

--

#### -- CELL ACTIVATION FAILURE

--

-- **************************************************************

CellActivationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{CellActivationFailure-IEs}},

...

}

CellActivationFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- X2 RELEASE

--

-- **************************************************************

X2Release ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2Release-IEs}},

...

}

X2Release-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID			CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- X2AP MESSAGE TRANSFER

--

-- **************************************************************

X2APMessageTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2APMessageTransfer-IEs}},

...

}

X2APMessageTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RNL-Header	CRITICALITY reject	TYPE RNL-Header				PRESENCE mandatory}|

{ ID id-x2APMessage	CRITICALITY reject	TYPE X2AP-Message			PRESENCE optional},

...

}

RNL-Header ::= SEQUENCE {

source-GlobalENB-ID	GlobalENB-ID,

target-GlobalENB-ID	GlobalENB-ID	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {RNL-Header-Item-ExtIEs} } OPTIONAL,

...

}

RNL-Header-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

X2AP-Message ::= OCTET STRING

-- **************************************************************

--

#### -- SENB ADDITION REQUEST

--

-- **************************************************************

SeNBAdditionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequest-IEs}},

...

}

SeNBAdditionRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-UE-SecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE conditional}|

-- This IE shall be present if the Bearer Option IE is set to the value “SCG bearer” --

{ ID id-SeNBSecurityKey					CRITICALITY reject	TYPE SeNBSecurityKey					PRESENCE conditional}|

-- This IE shall be present if the Bearer Option IE is set to the value “SCG bearer” --

{ ID id-SeNBUEAggregateMaximumBitRate	CRITICALITY reject	TYPE UEAggregateMaximumBitRate			PRESENCE mandatory}|

{ ID id-ServingPLMN						CRITICALITY ignore	TYPE PLMN-Identity						PRESENCE optional}|

{ ID id-E-RABs-ToBeAdded-List			CRITICALITY reject	TYPE E-RABs-ToBeAdded-List				PRESENCE mandatory}|

{ ID id-MeNBtoSeNBContainer				CRITICALITY reject	TYPE MeNBtoSeNBContainer				PRESENCE mandatory}|

{ ID id-CSGMembershipStatus				CRITICALITY reject	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-ExpectedUEBehaviour				CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional},

...

}

E-RABs-ToBeAdded-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-ItemIEs} }

E-RABs-ToBeAdded-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-Item	CRITICALITY reject	TYPE E-RABs-ToBeAdded-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-Item ::= CHOICE {

sCG-Bearer		E-RABs-ToBeAdded-Item-SCG-Bearer,

split-Bearer	E-RABs-ToBeAdded-Item-Split-Bearer,

...

}

E-RABs-ToBeAdded-Item-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

dL-Forwarding					DL-Forwarding																	OPTIONAL,

s1-UL-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-Item-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-Item-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID				CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID		CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-BearerType					CRITICALITY ignore	EXTENSION BearerType			PRESENCE optional},

...

}

E-RABs-ToBeAdded-Item-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-Item-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeAdded-Item-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SeNBAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequestAcknowledge-IEs}},

...

}

SeNBAdditionRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-List		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-List	PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List				CRITICALITY ignore	TYPE E-RAB-List							PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer					CRITICALITY reject	TYPE SeNBtoMeNBContainer				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

{ ID id-GW-TransportLayerAddress			CRITICALITY ignore	TYPE TransportLayerAddress				PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-Tunnel-Information-for-BBF			CRITICALITY ignore	TYPE TunnelInformation					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-List ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-ItemIEs} }

E-RABs-Admitted-ToBeAdded-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-Item 		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-Item ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeAdded-Item-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeAdded-Item-Split-Bearer,

...

}

E-RABs-Admitted-ToBeAdded-Item-SCG-Bearer ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-Item-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-Item-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-Item-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-Item-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-Item-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB ADDITION REQUEST REJECT

--

-- **************************************************************

SeNBAdditionRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBAdditionRequestReject-IEs}},

...

}

SeNBAdditionRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB RECONFIGURATION COMPLETE

--

-- **************************************************************

SeNBReconfigurationComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBReconfigurationComplete-IEs}},

...

}

SeNBReconfigurationComplete-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-ResponseInformationSeNBReconfComp		CRITICALITY ignore	TYPE ResponseInformationSeNBReconfComp	PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional},

...

}

ResponseInformationSeNBReconfComp ::= CHOICE {

success				ResponseInformationSeNBReconfComp-SuccessItem,

reject-by-MeNB		ResponseInformationSeNBReconfComp-RejectByMeNBItem,

...

}

ResponseInformationSeNBReconfComp-SuccessItem ::= SEQUENCE {

meNBtoSeNBContainer				MeNBtoSeNBContainer OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSeNBReconfComp-SuccessItemExtIEs} }	OPTIONAL,

...

}

ResponseInformationSeNBReconfComp-SuccessItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ResponseInformationSeNBReconfComp-RejectByMeNBItem ::= SEQUENCE {

cause							Cause,

meNBtoSeNBContainer				MeNBtoSeNBContainer																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSeNBReconfComp-RejectByMeNBItemExtIEs} }	OPTIONAL,

...

}

ResponseInformationSeNBReconfComp-RejectByMeNBItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUEST

--

-- **************************************************************

SeNBModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ SeNBModificationRequest-IEs}},

...

}

SeNBModificationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause									PRESENCE mandatory}|

{ ID id-SCGChangeIndication					CRITICALITY ignore	TYPE SCGChangeIndication					PRESENCE optional}|

{ ID id-ServingPLMN							CRITICALITY ignore	TYPE PLMN-Identity							PRESENCE optional}|

{ ID id-UE-ContextInformationSeNBModReq		CRITICALITY reject	TYPE UE-ContextInformationSeNBModReq	PRESENCE optional}|

{ ID id-MeNBtoSeNBContainer					CRITICALITY ignore	TYPE MeNBtoSeNBContainer					PRESENCE optional}|

{ ID id-CSGMembershipStatus					CRITICALITY reject	TYPE CSGMembershipStatus					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional},

...

}

UE-ContextInformationSeNBModReq ::= SEQUENCE {

uE-SecurityCapabilities			UESecurityCapabilities															OPTIONAL,

seNB-SecurityKey				SeNBSecurityKey 																OPTIONAL,

seNBUEAggregateMaximumBitRate	UEAggregateMaximumBitRate														OPTIONAL,

e-RABs-ToBeAdded				E-RABs-ToBeAdded-List-ModReq													OPTIONAL,

e-RABs-ToBeModified				E-RABs-ToBeModified-List-ModReq													OPTIONAL,

e-RABs-ToBeReleased				E-RABs-ToBeReleased-List-ModReq													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UE-ContextInformationSeNBModReqExtIEs} }		OPTIONAL,

...

}

UE-ContextInformationSeNBModReqExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-List-ModReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-ModReqItemIEs} }

E-RABs-ToBeAdded-ModReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-ModReqItem	CRITICALITY ignore	TYPE E-RABs-ToBeAdded-ModReqItem	PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-ModReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeAdded-ModReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeAdded-ModReqItem-Split-Bearer,

...

}

E-RABs-ToBeAdded-ModReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

dL-Forwarding					DL-Forwarding																	OPTIONAL,

s1-UL-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-ModReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeAdded-ModReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Correlation-ID				CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-SIPTO-Correlation-ID		CRITICALITY ignore	EXTENSION Correlation-ID		PRESENCE optional}|

{ ID id-BearerType					CRITICALITY ignore	EXTENSION BearerType			PRESENCE optional},

...

}

E-RABs-ToBeAdded-ModReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-ModReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeAdded-ModReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-List-ModReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-ModReqItemIEs} }

E-RABs-ToBeModified-ModReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-ModReqItem	CRITICALITY ignore	TYPE E-RABs-ToBeModified-ModReqItem		PRESENCE mandatory},

...

}

E-RABs-ToBeModified-ModReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeModified-ModReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeModified-ModReqItem-Split-Bearer,

...

}

E-RABs-ToBeModified-ModReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters														OPTIONAL,

s1-UL-GTPtunnelEndpoint			GTPtunnelEndpoint																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-ModReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeModified-ModReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-ModReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters																OPTIONAL,

meNB-GTPtunnelEndpoint			GTPtunnelEndpoint																		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-ModReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeModified-ModReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-List-ModReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-ModReqItemIEs} }

E-RABs-ToBeReleased-ModReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-ModReqItem	CRITICALITY ignore	TYPE E-RABs-ToBeReleased-ModReqItem		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-ModReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeReleased-ModReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeReleased-ModReqItem-Split-Bearer,

...

}

E-RABs-ToBeReleased-ModReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-ModReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-ModReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-ModReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-ModReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-ModReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SeNBModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBModificationRequestAcknowledge-IEs}},

...

}

SeNBModificationRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-ModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-ModAckList		PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeModified-ModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-ModAckList	PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeReleased-ModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-ModAckList	PRESENCE optional}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List											PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer							CRITICALITY ignore	TYPE SeNBtoMeNBContainer								PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-ModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-ModAckItemIEs} }

E-RABs-Admitted-ToBeAdded-ModAckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-ModAckItem	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-ModAckItem		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-ModAckItem ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeAdded-ModAckItem-Split-Bearer,

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-ModAckItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-ModAckItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeModified-ModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeModified-ModAckItemIEs} }

E-RABs-Admitted-ToBeModified-ModAckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeModified-ModAckItem		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-ModAckItem	PRESENCE mandatory}

}

E-RABs-Admitted-ToBeModified-ModAckItem ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeModified-ModAckItem-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeModified-ModAckItem-Split-Bearer,

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

s1-DL-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-ModAckItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

seNB-GTPtunnelEndpoint			GTPtunnelEndpoint																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-ModAckItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-ModAckItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-ModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeReleased-ModAckItemIEs} }

E-RABs-Admitted-ToBeReleased-ModAckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeReleased-ModAckItem		CRITICALITY ignore	TYPE E-RABs-Admitted-ToReleased-ModAckItem		PRESENCE mandatory}

}

E-RABs-Admitted-ToReleased-ModAckItem ::= CHOICE {

sCG-Bearer		E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-Bearer,

split-Bearer	E-RABs-Admitted-ToBeReleased-ModAckItem-Split-Bearer,

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-BearerExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID					E-RAB-ID,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-ModAckItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-ModAckItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUEST REJECT

--

-- **************************************************************

SeNBModificationRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBModificationRequestReject-IEs}},

...

}

SeNBModificationRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REQUIRED

--

-- **************************************************************

SeNBModificationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SeNBModificationRequired-IEs}},

...

}

SeNBModificationRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-SCGChangeIndication					CRITICALITY ignore	TYPE SCGChangeIndication				PRESENCE optional}|

{ ID id-E-RABs-ToBeReleased-ModReqd			CRITICALITY ignore	TYPE E-RABs-ToBeReleased-ModReqd		PRESENCE optional}|

{ ID id-SeNBtoMeNBContainer					CRITICALITY ignore	TYPE SeNBtoMeNBContainer				PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension				PRESENCE optional},

...

}

E-RABs-ToBeReleased-ModReqd ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-ModReqdItemIEs} }

E-RABs-ToBeReleased-ModReqdItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-ModReqdItem	 CRITICALITY ignore		TYPE E-RABs-ToBeReleased-ModReqdItem	PRESENCE mandatory },

...

}

E-RABs-ToBeReleased-ModReqdItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

cause							Cause,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-ModReqdItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-ModReqdItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION CONFIRM

--

-- **************************************************************

SeNBModificationConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBModificationConfirm-IEs}},

...

}

SeNBModificationConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-MeNBtoSeNBContainer					CRITICALITY ignore	TYPE MeNBtoSeNBContainer			PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension			PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB MODIFICATION REFUSE

--

-- **************************************************************

SeNBModificationRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SeNBModificationRefuse-IEs}},

...

}

SeNBModificationRefuse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-MeNBtoSeNBContainer					CRITICALITY ignore	TYPE MeNBtoSeNBContainer		PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB RELEASE REQUEST

--

-- **************************************************************

SeNBReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SeNBReleaseRequest-IEs}},

...

}

SeNBReleaseRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE optional}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE optional}|

{ ID id-E-RABs-ToBeReleased-List-RelReq		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-List-RelReq		PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator				CRITICALITY ignore	TYPE UE-ContextKeptIndicator					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-MakeBeforeBreakIndicator			CRITICALITY ignore	TYPE MakeBeforeBreakIndicator					PRESENCE optional},

...

}

E-RABs-ToBeReleased-List-RelReq ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-RelReqItemIEs} }

E-RABs-ToBeReleased-RelReqItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-RelReqItem		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-RelReqItem		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-RelReqItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeReleased-RelReqItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeReleased-RelReqItem-Split-Bearer,

...

}

E-RABs-ToBeReleased-RelReqItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelReqItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelReqItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-RelReqItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelReqItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelReqItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB RELEASE REQUIRED

--

-- **************************************************************

SeNBReleaseRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBReleaseRequired-IEs}},

...

}

SeNBReleaseRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SENB RELEASE CONFIRM

--

-- **************************************************************

SeNBReleaseConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBReleaseConfirm-IEs}},

...

}

SeNBReleaseConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-List-RelConf		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-List-RelConf		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional},

...

}

E-RABs-ToBeReleased-List-RelConf ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-RelConfItemIEs} }

E-RABs-ToBeReleased-RelConfItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-RelConfItem		CRITICALITY ignore		TYPE E-RABs-ToBeReleased-RelConfItem		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-RelConfItem ::= CHOICE {

sCG-Bearer		E-RABs-ToBeReleased-RelConfItem-SCG-Bearer,

split-Bearer	E-RABs-ToBeReleased-RelConfItem-Split-Bearer,

...

}

E-RABs-ToBeReleased-RelConfItem-SCG-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelConfItem-SCG-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelConfItem-SCG-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-RelConfItem-Split-Bearer ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-RelConfItem-Split-BearerExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-RelConfItem-Split-BearerExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SENB COUNTER CHECK REQUEST

--

-- **************************************************************

SeNBCounterCheckRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SeNBCounterCheckRequest-IEs}},

...

}

SeNBCounterCheckRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-SubjectToCounterCheck-List	CRITICALITY ignore	TYPE E-RABs-SubjectToCounterCheck-List		PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional},

...

}

E-RABs-SubjectToCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-SubjectToCounterCheckItemIEs} }

E-RABs-SubjectToCounterCheckItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-SubjectToCounterCheckItem	CRITICALITY ignore	TYPE E-RABs-SubjectToCounterCheckItem		PRESENCE mandatory},

...

}

E-RABs-SubjectToCounterCheckItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Count						INTEGER (0..4294967295),

dL-Count						INTEGER (0..4294967295),

iE-Extensions					ProtocolExtensionContainer { {E-RABs-SubjectToCounterCheckItemExtIEs} } OPTIONAL,

...

}

E-RABs-SubjectToCounterCheckItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- X2 REMOVAL REQUEST

--

-- **************************************************************

X2RemovalRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2RemovalRequest-IEs}},

...

}

X2RemovalRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID				CRITICALITY reject	TYPE GlobalENB-ID			PRESENCE mandatory}|

{ ID id-X2RemovalThreshold			CRITICALITY reject	TYPE X2BenefitValue			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 REMOVAL RESPONSE

--

-- **************************************************************

X2RemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2RemovalResponse-IEs}},

...

}

X2RemovalResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID				CRITICALITY reject	TYPE GlobalENB-ID				PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- X2 REMOVAL FAILURE

--

-- **************************************************************

X2RemovalFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{X2RemovalFailure-IEs}},

...

}

X2RemovalFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT REQUEST

--

-- **************************************************************

RetrieveUEContextRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ RetrieveUEContextRequest-IEs}},

...

}

RetrieveUEContextRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-SeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension	PRESENCE optional}|

{ ID id-resumeID							CRITICALITY reject	TYPE ResumeID				PRESENCE mandatory}|

{ ID id-ShortMAC-I							CRITICALITY reject	TYPE ShortMAC-I				PRESENCE mandatory}|

{ ID id-NewEUTRANCellIdentifier				CRITICALITY reject	TYPE EUTRANCellIdentifier	PRESENCE mandatory}|

{ID id-FailureCellCRNTI						CRITICALITY reject	TYPE CRNTI					PRESENCE optional}|

{ID id-FailureCellPCI						CRITICALITY reject	TYPE PCI					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-GUMMEI-ID									CRITICALITY reject	TYPE GUMMEI													PRESENCE mandatory}|

{ ID id-UE-ContextInformationRetrieve				CRITICALITY reject	TYPE UE-ContextInformationRetrieve					PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible								PRESENCE optional}|

{ ID id-Masked-IMEISV								CRITICALITY ignore	TYPE Masked-IMEISV											PRESENCE optional}|

{ ID id-ExpectedUEBehaviour							CRITICALITY ignore	TYPE ExpectedUEBehaviour									PRESENCE optional}|

{ ID id-ProSeAuthorized								CRITICALITY ignore	TYPE ProSeAuthorized										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional}|

{ ID id-V2XServicesAuthorized						CRITICALITY ignore	TYPE V2XServicesAuthorized								PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation				CRITICALITY ignore	TYPE AerialUEsubscriptionInformation					PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo	CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo	PRESENCE optional},

...

}

UE-ContextInformationRetrieve ::= SEQUENCE {

mME-UE-S1AP-ID						UE-S1AP-ID,

uESecurityCapabilities				UESecurityCapabilities,

aS-SecurityInformation				AS-SecurityInformation,

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

e-RABs-ToBeSetup-ListRetrieve		E-RABs-ToBeSetup-ListRetrieve,

rRC-Context							RRC-Context,

handoverRestrictionList				HandoverRestrictionList			OPTIONAL,

locationReportingInformation		LocationReportingInformation	OPTIONAL,

managBasedMDTallowed				ManagementBasedMDTallowed		OPTIONAL,

managBasedMDTPLMNList				MDTPLMNList						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-ContextInformationRetrieve-ExtIEs} } OPTIONAL,

...

}

UE-ContextInformationRetrieve-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION UESidelinkAggregateMaximumBitRate		PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional},

...

}

E-RABs-ToBeSetup-ListRetrieve ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeSetupRetrieve-ItemIEs} }

E-RABs-ToBeSetupRetrieve-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeSetupRetrieve-Item	CRITICALITY ignore	TYPE E-RABs-ToBeSetupRetrieve-Item	PRESENCE mandatory},

...

}

E-RABs-ToBeSetupRetrieve-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

e-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

bearerType						BearerType	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeSetupRetrieve-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeSetupRetrieve-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uL-GTPtunnelEndpoint		CRITICALITY reject	EXTENSION GTPtunnelEndpoint		PRESENCE mandatory}|

{ ID id-dL-Forwarding				CRITICALITY ignore	EXTENSION DL-Forwarding			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT FAILURE

--

-- **************************************************************

RetrieveUEContextFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ RetrieveUEContextFailure-IEs}},

...

}

RetrieveUEContextFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID				CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension	CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST

--

-- **************************************************************

SgNBAdditionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container {{SgNBAdditionRequest-IEs}},

...

}

SgNBAdditionRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-NRUESecurityCapabilities					CRITICALITY reject	TYPE NRUESecurityCapabilities					PRESENCE mandatory}|

{ ID id-SgNBSecurityKey							CRITICALITY reject	TYPE SgNBSecurityKey									PRESENCE mandatory}|

{ ID id-SgNBUEAggregateMaximumBitRate			CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-SelectedPLMN								CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList						PRESENCE optional}|

{ ID id-E-RABs-ToBeAdded-SgNBAddReqList			CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReqList			PRESENCE mandatory}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-RequestedSplitSRBs						CRITICALITY reject	TYPE SplitSRBs										PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation		CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation		PRESENCE optional}|

{ ID id-SGNB-Addition-Trigger-Ind				CRITICALITY reject	TYPE SGNB-Addition-Trigger-Ind					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP				CRITICALITY ignore	TYPE SubscriberProfileIDforRFP					PRESENCE optional}|

{ ID id-MeNBCell-ID								CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore	TYPE DesiredActNotificationLevel				PRESENCE optional}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional}|

{ ID id-LocationInformationSgNBReporting			CRITICALITY ignore	TYPE LocationInformationSgNBReporting			PRESENCE optional}|

{ ID id-Masked-IMEISV							CRITICALITY ignore	TYPE Masked-IMEISV									PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex					PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore	TYPE	 RequestedFastMCGRecoveryViaSRB3			PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBAddReqList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBAddReq-ItemIEs} }

E-RABs-ToBeAdded-SgNBAddReq-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-SgNBAddReq-Item		CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

drb-ID								DRB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters				E-RAB-Level-QoS-Parameters,

max-MCG-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation															OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE --

dL-Forwarding								DL-Forwarding																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG						GTPtunnelEndpoint																	OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

s1-UL-GTPtunnelEndpoint						GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresentExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLCMode-transferred					CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional}|

{ ID id-BearerType							CRITICALITY ignore	EXTENSION BearerType				PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-UL-GTP-TEIDatPDCP							GTPtunnelEndpoint,

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint	OPTIONAL,

rlc-Mode										RLCMode,

uL-Configuration								ULConfiguration	OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresentExtIEs} }		 OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-duplicationActivation					CRITICALITY ignore	EXTENSION DuplicationActivation		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SgNBAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBAdditionRequestAcknowledge-IEs}},

...

}

SgNBAdditionRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY reject	TYPE UE-X2AP-ID													PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY reject	TYPE SgNB-UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList		PRESENCE mandatory}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List													PRESENCE optional}|

{ ID id-SgNBtoMeNBContainer							CRITICALITY reject	TYPE SgNBtoMeNBContainer										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY reject	TYPE UE-X2AP-ID-Extension									PRESENCE optional}|

{ ID id-AdmittedSplitSRBs							CRITICALITY reject	TYPE SplitSRBs													PRESENCE optional}|

{ ID id-SgNBResourceCoordinationInformation			CRITICALITY ignore	TYPE SgNBResourceCoordinationInformation				PRESENCE optional}|

{ ID id-RRCConfigIndication							CRITICALITY reject	TYPE RRC-Config-Ind												PRESENCE optional}|

{ ID id-LocationInformationSgNB						CRITICALITY ignore	TYPE LocationInformationSgNB								PRESENCE optional}|

{ ID id-FastMCGRecoveryViaSRB3Enabled				CRITICALITY ignore	TYPE FastMCGRecoveryViaSRB3Enabled						PRESENCE optional },

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-ItemIEs} }

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item 		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBAddReqAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBAddReqAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresent ::= SEQUENCE {

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

sgNB-UL-GTP-TEIDatPDCP					GTPtunnelEndpoint																						OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

rlc-Mode								RLCMode																									OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																						OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																						OPTIONAL,

mCG-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																				OPTIONAL,

-- This IE shall be present if MCG resource and SCG resource IEs in the EN-DC Resource Configuration IE are set to “present” and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE --

uL-Configuration							ULConfiguration																							OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions							ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresentExtIEs} }			OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG				GTPtunnelEndpoint,

secondary-sgNB-DL-GTP-TEIDatSCG		GTPtunnelEndpoint		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresentExtIEs} } 		OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-lCID				CRITICALITY ignore	EXTENSION LCID			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ADDITION REQUEST REJECT

--

-- **************************************************************

SgNBAdditionRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBAdditionRequestReject-IEs}},

...

}

SgNBAdditionRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY reject	TYPE SgNB-UE-X2AP-ID			PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension		CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB RECONFIGURATION COMPLETE

--

-- **************************************************************

SgNBReconfigurationComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBReconfigurationComplete-IEs}},

...

}

SgNBReconfigurationComplete-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-ResponseInformationSgNBReconfComp		CRITICALITY ignore	TYPE ResponseInformationSgNBReconfComp			PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional},

...

}

ResponseInformationSgNBReconfComp ::= CHOICE {

success-SgNBReconfComp				ResponseInformationSgNBReconfComp-SuccessItem,

reject-by-MeNB-SgNBReconfComp		ResponseInformationSgNBReconfComp-RejectByMeNBItem,

...

}

ResponseInformationSgNBReconfComp-SuccessItem ::= SEQUENCE {

meNBtoSgNBContainer				MeNBtoSgNBContainer					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSgNBReconfComp-SuccessItemExtIEs} }	OPTIONAL,

...

}

ResponseInformationSgNBReconfComp-SuccessItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ResponseInformationSgNBReconfComp-RejectByMeNBItem ::= SEQUENCE {

cause							Cause,

iE-Extensions					ProtocolExtensionContainer { {ResponseInformationSgNBReconfComp-RejectByMeNBItemExtIEs} }		OPTIONAL,

...

}

ResponseInformationSgNBReconfComp-RejectByMeNBItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUEST

--

-- **************************************************************

SgNBModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ SgNBModificationRequest-IEs}},

...

}

SgNBModificationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause												PRESENCE mandatory}|

{ ID id-SelectedPLMN							CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList							PRESENCE optional}|

{ ID	id-SCGConfigurationQuery				CRITICALITY ignore	TYPE SCGConfigurationQuery							PRESENCE optional}|

{ ID id-UE-ContextInformation-SgNBModReq		CRITICALITY reject	TYPE UE-ContextInformation-SgNBModReq				PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation		CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation			PRESENCE optional}|

{ ID id-RequestedSplitSRBs						CRITICALITY ignore	TYPE SplitSRBs											PRESENCE optional}|

{ ID id-RequestedSplitSRBsrelease				CRITICALITY ignore	TYPE SplitSRBs											PRESENCE optional}|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore	TYPE DesiredActNotificationLevel					PRESENCE optional}|

{ ID id-LocationInformationSgNBReporting		CRITICALITY ignore	TYPE LocationInformationSgNBReporting				PRESENCE optional}|

{ ID id-MeNBCell-ID								CRITICALITY ignore	TYPE ECGI												PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore	TYPE RequestedFastMCGRecoveryViaSRB3				PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3Release	CRITICALITY ignore	TYPE RequestedFastMCGRecoveryViaSRB3Release		PRESENCE optional},

...

}

UE-ContextInformation-SgNBModReq ::= SEQUENCE {

nRUE-SecurityCapabilities		NRUESecurityCapabilities															OPTIONAL,

sgNB-SecurityKey				SgNBSecurityKey 																	OPTIONAL,

sgNBUEAggregateMaximumBitRate	UEAggregateMaximumBitRate															OPTIONAL,

e-RABs-ToBeAdded				E-RABs-ToBeAdded-SgNBModReq-List													OPTIONAL,

e-RABs-ToBeModified				E-RABs-ToBeModified-SgNBModReq-List													OPTIONAL,

e-RABs-ToBeReleased				E-RABs-ToBeReleased-SgNBModReq-List													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UE-ContextInformationSgNBModReqExtIEs} }			OPTIONAL,

...

}

UE-ContextInformationSgNBModReqExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{	ID id-SubscriberProfileIDforRFP					CRITICALITY ignore	EXTENSION SubscriberProfileIDforRFP									PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional}|

{ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	EXTENSION LowerLayerPresenceStatusChange				PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBModReq-ItemIEs} }

E-RABs-ToBeAdded-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeAdded-SgNBModReq-Item		CRITICALITY ignore	TYPE E-RABs-ToBeAdded-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeAdded-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

drb-ID									DRB-ID,

en-DC-ResourceConfiguration				EN-DC-ResourceConfiguration,

resource-configuration					CHOICE {

sgNBPDCPpresent						E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters,

max-MN-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation															OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE --

dL-Forwarding							DL-Forwarding																		OPTIONAL,

meNB-DL-GTP-TEIDatMCG					GTPtunnelEndpoint																	OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE is set to “present” --

s1-UL-GTPtunnelEndpoint					GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLCMode-transferred					CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional}|

{ ID id-BearerType							CRITICALITY ignore	EXTENSION BearerType				PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters,

meNB-UL-GTP-TEIDatPDCP							GTPtunnelEndpoint,

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint		OPTIONAL,

rlc-Mode										RLCMode,

uL-Configuration								ULConfiguration																OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 		OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-duplicationActivation					CRITICALITY ignore	EXTENSION DuplicationActivation		PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-SgNBModReq-ItemIEs} }

E-RABs-ToBeModified-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-SgNBModReq-Item	CRITICALITY ignore	TYPE E-RABs-ToBeModified-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeModified-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

full-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																		OPTIONAL,

max-MN-admit-E-RAB-Level-QoS-Parameters	GBR-QosInformation																				OPTIONAL,

meNB-DL-GTP-TEIDatMCG					GTPtunnelEndpoint																				OPTIONAL,

s1-UL-GTPtunnelEndpoint					GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional },

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

requested-SCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters																			OPTIONAL,

meNB-UL-GTP-TEIDatPDCP						GTPtunnelEndpoint																					OPTIONAL,

uL-Configuration								ULConfiguration																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-secondarymeNBULGTPTEIDatPDCP			CRITICALITY ignore	EXTENSION GTPtunnelEndpoint			PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBModReq-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBModReq-ItemIEs} }

E-RABs-ToBeReleased-SgNBModReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBModReq-Item	CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBModReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBModReq-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresent ::= SEQUENCE {

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SgNBModificationRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBModificationRequestAcknowledge-IEs}},

...

}

SgNBModificationRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID													PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBModAckList		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBModAckList			PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeModified-SgNBModAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-SgNBModAckList		PRESENCE optional}|

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBModAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-SgNBModAckList		PRESENCE optional}|

{ ID id-E-RABs-NotAdmitted-List						CRITICALITY ignore	TYPE E-RAB-List													PRESENCE optional}|

{ ID id-SgNBtoMeNBContainer							CRITICALITY ignore	TYPE SgNBtoMeNBContainer										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension									PRESENCE optional}|

{ ID id-SgNBResourceCoordinationInformation			CRITICALITY ignore	TYPE SgNBResourceCoordinationInformation				PRESENCE optional}|

{ ID id-AdmittedSplitSRBs							CRITICALITY ignore	TYPE SplitSRBs													PRESENCE optional}|

{ ID id-AdmittedSplitSRBsrelease						CRITICALITY ignore	TYPE SplitSRBs													PRESENCE optional}|

{ ID id-RRCConfigIndication							CRITICALITY reject	TYPE RRC-Config-Ind												PRESENCE optional}|

{ ID id-LocationInformationSgNB						CRITICALITY ignore	TYPE LocationInformationSgNB								PRESENCE optional}|

{ ID id-FastMCGRecoveryViaSRB3Enabled				CRITICALITY ignore	TYPE FastMCGRecoveryViaSRB3Enabled						PRESENCE optional}|

{ ID id-FastMCGRecoveryViaSRB3Disabled		CRITICALITY ignore	TYPE FastMCGRecoveryViaSRB3Disabled				PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemIEs} }

E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item 	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeAdded-SgNBModAck-Item		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresent ::= SEQUENCE {

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint,

sgNB-UL-GTP-TEIDatPDCP					GTPtunnelEndpoint																					OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE are set to “present” --

rlc-Mode									RLCMode																								OPTIONAL,

-- This IE shall be present if MCG resource IE in the EN-DC Resource Configuration IE are set to “present” --

dL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																					OPTIONAL,

uL-Forwarding-GTPtunnelEndpoint			GTPtunnelEndpoint																					OPTIONAL,

mCG-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																			OPTIONAL,

-- This IE shall be present if MCG resource and SCG resource IEs in the EN-DC Resource Configuration IE are set to “present” and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE --

uL-Configuration						ULConfiguration																							OPTIONAL,

-- This IE shall be present if MCG resource and SCG resources IEs in the EN-DC Resource Configuration IE are set to “present” --

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG					GTPtunnelEndpoint,

secondary-sgNB-DL-GTP-TEIDatSCG			GTPtunnelEndpoint			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeAdded-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-lCID				CRITICALITY ignore	EXTENSION LCID		PRESENCE optional},

...

}

E-RABs-Admitted-ToBeModified-SgNBModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeModified-SgNBModAck-ItemIEs} }

E-RABs-Admitted-ToBeModified-SgNBModAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item		CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeModified-SgNBModAck-Item		PRESENCE mandatory}

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeAdded-SgNBModAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeAdded-SgNBModAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresent ::= SEQUENCE {

s1-DL-GTPtunnelEndpoint					GTPtunnelEndpoint																						OPTIONAL,

sgNB-UL-GTP-TEIDatPDCP					GTPtunnelEndpoint																						OPTIONAL,

mCG-E-RAB-Level-QoS-Parameters			E-RAB-Level-QoS-Parameters																				OPTIONAL,

uL-Configuration							ULConfiguration																							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG					GTPtunnelEndpoint																								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-Admitted-ToBeModified-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-secondarysgNBDLGTPTEIDatPDCP			CRITICALITY ignore	EXTENSION GTPtunnelEndpoint				PRESENCE optional}|

{ ID id-RLC-Status								CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional },

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAckList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeReleased-SgNBModAck-ItemIEs} }

E-RABs-Admitted-ToBeReleased-SgNBModAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item		CRITICALITY ignore	TYPE E-RABs-Admitted-ToReleased-SgNBModAck-Item		PRESENCE mandatory}

}

E-RABs-Admitted-ToReleased-SgNBModAck-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresent ::= SEQUENCE {

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions				ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs} } OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-SgNBModAck-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUEST REJECT

--

-- **************************************************************

SgNBModificationRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBModificationRequestReject-IEs}},

...

}

SgNBModificationRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REQUIRED

--

-- **************************************************************

SgNBModificationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBModificationRequired-IEs}},

...

}

SgNBModificationRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-PDCPChangeIndication						CRITICALITY ignore	TYPE PDCPChangeIndication							PRESENCE optional}|

{ ID id-E-RABs-ToBeReleased-SgNBModReqdList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBModReqdList		PRESENCE optional}|

{ ID id-SgNBtoMeNBContainer						CRITICALITY ignore	TYPE SgNBtoMeNBContainer								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-E-RABs-ToBeModified-SgNBModReqdList		CRITICALITY ignore	TYPE E-RABs-ToBeModified-SgNBModReqdList		PRESENCE optional}|

{ ID id-SgNBResourceCoordinationInformation		CRITICALITY ignore	TYPE SgNBResourceCoordinationInformation		PRESENCE optional}|

{ ID id-RRCConfigIndication						CRITICALITY reject	TYPE RRC-Config-Ind									PRESENCE optional}|

{ ID id-LocationInformationSgNB					CRITICALITY ignore	TYPE LocationInformationSgNB						PRESENCE optional}|

{ ID id-FastMCGRecoveryViaSRB3Enabled			CRITICALITY ignore	TYPE FastMCGRecoveryViaSRB3Enabled				PRESENCE optional}|

{ ID id-FastMCGRecoveryViaSRB3Disabled			CRITICALITY ignore	TYPE FastMCGRecoveryViaSRB3Disabled			PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBModReqdList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBModReqd-ItemIEs} }

E-RABs-ToBeReleased-SgNBModReqd-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBModReqd-Item	 CRITICALITY ignore		TYPE E-RABs-ToBeReleased-SgNBModReqd-Item		PRESENCE mandatory },

...

}

E-RABs-ToBeReleased-SgNBModReqd-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

cause							Cause,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBModReqd-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBModReqd-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLCMode-transferred					CRITICALITY ignore	EXTENSION RLCMode					PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReqdList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeModified-SgNBModReqd-ItemIEs} }

E-RABs-ToBeModified-SgNBModReqd-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeModified-SgNBModReqd-Item	 CRITICALITY ignore		TYPE E-RABs-ToBeModified-SgNBModReqd-Item		PRESENCE mandatory },

...

}

E-RABs-ToBeModified-SgNBModReqd-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReqd-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReqd-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresent ::= SEQUENCE {

requested-MCG-E-RAB-Level-QoS-Parameters		E-RAB-Level-QoS-Parameters													OPTIONAL,

uL-Configuration								ULConfiguration																OPTIONAL,

sgNB-UL-GTP-TEIDatPDCP						GTPtunnelEndpoint															OPTIONAL,

s1-DL-GTP-TEIDatSgNB							GTPtunnelEndpoint															OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-new-drb-ID-req							CRITICALITY ignore	EXTENSION NewDRBIDrequest				PRESENCE optional},

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresent ::= SEQUENCE {

sgNB-DL-GTP-TEIDatSCG				GTPtunnelEndpoint				OPTIONAL,

secondary-sgNB-DL-GTP-TEIDatSCG		GTPtunnelEndpoint				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-ToBeModified-SgNBModReqd-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional}|

{ ID id-lCID				CRITICALITY ignore			EXTENSION LCID						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION CONFIRM

--

-- **************************************************************

SgNBModificationConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBModificationConfirm-IEs}},

...

}

SgNBModificationConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY ignore	TYPE UE-X2AP-ID													PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-E-RABs-AdmittedToBeModified-SgNBModConfList	CRITICALITY ignore	TYPE E-RABs-AdmittedToBeModified-SgNBModConfList		PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer							CRITICALITY ignore	TYPE MeNBtoSgNBContainer										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension									PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation			CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation				PRESENCE optional},

...

}

E-RABs-AdmittedToBeModified-SgNBModConfList ::= SEQUENCE (SIZE (1..maxnoofBearers)) OF ProtocolIE-Single-Container

{ {E-RABs-AdmittedToBeModified-SgNBModConf-ItemIEs} }

E-RABs-AdmittedToBeModified-SgNBModConf-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-AdmittedToBeModified-SgNBModConf-Item	 CRITICALITY ignore	TYPE E-RABs-AdmittedToBeModified-SgNBModConf-Item	 PRESENCE mandatory },

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item ::= SEQUENCE {

e-RAB-ID								E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-AdmittedToBeModified-SgNBModConf-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresent ::= SEQUENCE {

iE-Extensions				ProtocolExtensionContainer { {E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresent ::= SEQUENCE {

secondary-meNB-UL-GTP-TEIDatPDCP				GTPtunnelEndpoint				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresentExtIEs} }		OPTIONAL,

...

}

E-RABs-AdmittedToBeModified-SgNBModConf-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-uLpDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional}|

{ ID id-dLPDCPSnLength							CRITICALITY ignore	EXTENSION PDCPSnLength					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB MODIFICATION REFUSE

--

-- **************************************************************

SgNBModificationRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBModificationRefuse-IEs}},

...

}

SgNBModificationRefuse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-MeNBtoSgNBContainer					CRITICALITY ignore	TYPE MeNBtoSgNBContainer			PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY ignore	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST

--

-- **************************************************************

SgNBReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequest-IEs}},

...

}

SgNBReleaseRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE optional}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-SgNBRelReqList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReqList		PRESENCE optional}|

{ ID id-UE-ContextKeptIndicator					CRITICALITY ignore	TYPE UE-ContextKeptIndicator						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE optional}|

{ ID id-ERABs-transferred-to-MeNB				CRITICALITY ignore	TYPE E-RAB-List										PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBRelReqList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBRelReq-ItemIEs} }

E-RABs-ToBeReleased-SgNBRelReq-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBRelReq-Item		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReq-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReq-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReq-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresent ::= SEQUENCE {

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																	OPTIONAL,

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReq-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST ACKNOWLEDGE

--

-- **************************************************************

SgNBReleaseRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequestAcknowledge-IEs}},

...

}

SgNBReleaseRequestAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID									CRITICALITY ignore	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID									CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-MeNB-UE-X2AP-ID-Extension						CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional }|

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList	PRESENCE optional },

...

}

E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF

ProtocolIE-Single-Container { {E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemIEs} }

E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item	CRITICALITY ignore	TYPE E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item		PRESENCE mandatory},

...

}

E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

rlc-Mode-transferred			RLCMode,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUEST REJECT

--

-- **************************************************************

SgNBReleaseRequestReject ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBReleaseRequestReject-IEs}},

...

}

SgNBReleaseRequestReject-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB RELEASE REQUIRED

--

-- **************************************************************

SgNBReleaseRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBReleaseRequired-IEs}},

...

}

SgNBReleaseRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause												PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional }|

{ ID id-E-RABs-ToBeReleased-SgNBRelReqdList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReqdList			PRESENCE optional }|

{ ID id-SgNBtoMeNBContainer						CRITICALITY ignore	TYPE SgNBtoMeNBContainer									PRESENCE optional },

...

}

E-RABs-ToBeReleased-SgNBRelReqdList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBRelReqd-ItemIEs} }

E-RABs-ToBeReleased-SgNBRelReqd-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBRelReqd-Item		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelReqd-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBRelReqd-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

rlc-Mode-transferred			RLCMode,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelReqd-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelReqd-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB RELEASE CONFIRM

--

-- **************************************************************

SgNBReleaseConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBReleaseConfirm-IEs}},

...

}

SgNBReleaseConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-SgNBRelConfList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBRelConfList		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBRelConfList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBRelConf-ItemIEs} }

E-RABs-ToBeReleased-SgNBRelConf-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBRelConf-Item		CRITICALITY ignore		TYPE E-RABs-ToBeReleased-SgNBRelConf-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelConf-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelConf-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresent ::= SEQUENCE {

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresentExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBRelConf-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB COUNTER CHECK REQUEST

--

-- **************************************************************

SgNBCounterCheckRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBCounterCheckRequest-IEs}},

...

}

SgNBCounterCheckRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID								CRITICALITY reject	TYPE UE-X2AP-ID											PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY reject	TYPE SgNB-UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-E-RABs-SubjectToSgNBCounterCheck-List		CRITICALITY ignore	TYPE E-RABs-SubjectToSgNBCounterCheck-List		PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension					CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-SubjectToSgNBCounterCheck-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-SubjectToSgNBCounterCheck-ItemIEs} }

E-RABs-SubjectToSgNBCounterCheck-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-SubjectToSgNBCounterCheck-Item		CRITICALITY ignore	TYPE E-RABs-SubjectToSgNBCounterCheck-Item		PRESENCE mandatory},

...

}

E-RABs-SubjectToSgNBCounterCheck-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

uL-Count						INTEGER (0..4294967295),

dL-Count						INTEGER (0..4294967295),

iE-Extensions					ProtocolExtensionContainer { {E-RABs-SubjectToSgNBCounterCheck-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-SubjectToSgNBCounterCheck-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- SGNB CHANGE REQUIRED

--

-- **************************************************************

SgNBChangeRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBChangeRequired-IEs}},

...

}

SgNBChangeRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Target-SgNB-ID						CRITICALITY reject	TYPE GlobalGNB-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-SgNBtoMeNBContainer					CRITICALITY reject	TYPE SgNBtoMeNBContainer			PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB CHANGE CONFIRM

--

-- **************************************************************

SgNBChangeConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{SgNBChangeConfirm-IEs}},

...

}

SgNBChangeConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-E-RABs-ToBeReleased-SgNBChaConfList		CRITICALITY ignore	TYPE E-RABs-ToBeReleased-SgNBChaConfList		PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension							PRESENCE optional},

...

}

E-RABs-ToBeReleased-SgNBChaConfList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeReleased-SgNBChaConf-ItemIEs} }

E-RABs-ToBeReleased-SgNBChaConf-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-ToBeReleased-SgNBChaConf-Item		CRITICALITY ignore		TYPE E-RABs-ToBeReleased-SgNBChaConf-Item		PRESENCE mandatory},

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

en-DC-ResourceConfiguration			EN-DC-ResourceConfiguration,

resource-configuration				CHOICE {

sgNBPDCPpresent						E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresent,

sgNBPDCPnotpresent					E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresent,

...

},

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBChaConf-ItemExtIEs} }	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBChaConf-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresent ::= SEQUENCE {

uL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

dL-GTPtunnelEndpoint			GTPtunnelEndpoint																				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresentExtIEs} } OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresent ::= SEQUENCE {

iE-Extensions					ProtocolExtensionContainer { {E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresentExtIEs} } 	OPTIONAL,

...

}

E-RABs-ToBeReleased-SgNBChaConf-Item-SgNBPDCPnotpresentExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- RRC TRANSFER

--

-- **************************************************************

RRCTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{RRCTransfer-IEs}},

...

}

RRCTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-SplitSRB								CRITICALITY reject	TYPE SplitSRB					PRESENCE optional}|

{ ID id-NRUeReport							CRITICALITY reject	TYPE NRUeReport					PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional}|

{ ID id-FastMCGRecovery-SN-to-MN				CRITICALITY ignore 	TYPE FastMCGRecovery				PRESENCE optional}|

{ ID id-FastMCGRecovery-MN-to-SN				CRITICALITY ignore 	TYPE FastMCGRecovery				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB CHANGE REFUSE

--

-- **************************************************************

SgNBChangeRefuse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBChangeRefuse-IEs}},

...

}

SgNBChangeRefuse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY ignore	TYPE UE-X2AP-ID					PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY ignore	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC X2 SETUP REQUEST

--

-- **************************************************************

ENDCX2SetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupRequest-IEs}},

...

}

ENDCX2SetupRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EndcX2Setup			CRITICALITY reject	TYPE InitiatingNodeType-EndcX2Setup		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional}|

{ ID id-TNLConfigurationInfo					CRITICALITY ignore	TYPE TNLConfigurationInfo						PRESENCE optional},

...

}

InitiatingNodeType-EndcX2Setup ::= CHOICE {

init-eNB				ProtocolIE-Container	{{ENB-ENDCX2SetupReqIEs}},

init-en-gNB			ProtocolIE-Container	{{En-gNB-ENDCX2SetupReqIEs}},

...

}

ENB-ENDCX2SetupReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory}|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication					PRESENCE optional},

-- NOTE: In the current version of this specification the Interface Instance Indication IE is not included in the Initiating NodeType IE --

...

}

ServedEUTRAcellsENDCX2ManagementList ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

servedEUTRACellInfo					ServedCell-Information,

nrNeighbourInfo						NRNeighbour-Information		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ServedEUTRAcellsENDCX2Management-ExtIEs} } OPTIONAL,

...

}

ServedEUTRAcellsENDCX2Management-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-CellandCapacityAssistInfo		CRITICALITY ignore	EXTENSION CellandCapacityAssistInfo			PRESENCE optional},

...

}

En-gNB-ENDCX2SetupReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory}|

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE mandatory},

...

}

ServedNRcellsENDCX2ManagementList ::= SEQUENCE (SIZE (1.. maxCellinengNB)) OF SEQUENCE {

servedNRCellInfo						ServedNRCell-Information,

nRNeighbourInfo						NRNeighbour-Information OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {En-gNBServedCells-ExtIEs} } OPTIONAL,

...

}

En-gNBServedCells-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-PartialListIndicator		CRITICALITY ignore	EXTENSION PartialListIndicator		PRESENCE optional},

...

}

ServedNRCell-Information ::= SEQUENCE {

nrpCI				NRPCI,

nrCellID			NRCGI,

fiveGS-TAC			FiveGS-TAC	OPTIONAL,

configured-TAC		TAC			OPTIONAL,

broadcastPLMNs		BroadcastPLMNs-Item,

nrModeInfo			CHOICE {

fdd		FDD-InfoServedNRCell-Information,

tdd		TDD-InfoServedNRCell-Information,

...

},

measurementTimingConfiguration	OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { {ServedNRCell-Information-ExtIEs} } OPTIONAL,

...

}

ServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-additionalPLMNs-Item			CRITICALITY ignore	EXTENSION AdditionalPLMNs-Item			PRESENCE optional}|

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore 	EXTENSION BPLMN-ID-Info-NR				PRESENCE optional},

...

}

FDD-InfoServedNRCell-Information ::= SEQUENCE {

ul-NRFreqInfo				NRFreqInfo,

dl-NRFreqInfo				NRFreqInfo,

ul-NR-TxBW				NR-TxBW,

dl-NR-TxBW				NR-TxBW,

iE-Extensions			ProtocolExtensionContainer { {FDD-InfoServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

FDD-InfoServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-InfoServedNRCell-Information ::= SEQUENCE {

nRFreqInfo				NRFreqInfo,

nR-TxBW					NR-TxBW,

iE-Extensions			ProtocolExtensionContainer { {TDD-InfoServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellandCapacityAssistInfo::= SEQUENCE {

maximumCellListSize								MaximumCellListSize			OPTIONAL,

cellAssistanceInformation						CellAssistanceInformation	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {CellandCapacityAssistInfo-ExtIEs} } OPTIONAL,

...

}

CellandCapacityAssistInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellAssistanceInformation ::= CHOICE {

limited-list				Limited-list,

full-list				ENUMERATED {allServedNRcells, ...},

...

}

Limited-list 	::= SEQUENCE (SIZE (1..maxCellinengNB)) OF SEQUENCE {

nrCellID			NRCGI,

iE-Extensions						ProtocolExtensionContainer { {Limited-list-ExtIEs} } OPTIONAL,

...

}

Limited-list-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- EN-DC X2 SETUP RESPONSE

--

-- **************************************************************

ENDCX2SetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupResponse-IEs}},

...

}

ENDCX2SetupResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcX2Setup		CRITICALITY reject	TYPE RespondingNodeType-EndcX2Setup		PRESENCE mandatory},

...

}

RespondingNodeType-EndcX2Setup ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCX2SetupReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCX2SetupReqAckIEs}},

...

}

ENB-ENDCX2SetupReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory}|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication					PRESENCE optional },

...

}

En-gNB-ENDCX2SetupReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory}|

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- EN-DC X2 SETUP FAILURE

--

-- **************************************************************

ENDCX2SetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2SetupFailure-IEs}},

...

}

ENDCX2SetupFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore		TYPE Cause								PRESENCE mandatory} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional } |

{ ID id-TimeToWait						CRITICALITY ignore		TYPE TimeToWait							PRESENCE optional	} |

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 		TYPE InterfaceInstanceIndication		PRESENCE optional }|

{ ID id-MessageOversizeNotification		CRITICALITY ignore 		TYPE MessageOversizeNotification		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- EN-DC CONFIGURATION UPDATE

--

-- **************************************************************

ENDCConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationUpdate-IEs}},

...

}

ENDCConfigurationUpdate-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EndcConfigUpdate			CRITICALITY reject	TYPE InitiatingNodeType-EndcConfigUpdate			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject 	TYPE InterfaceInstanceIndication						PRESENCE optional}|

{ ID id-TNLConfigurationInfo						CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional	},

...

}

InitiatingNodeType-EndcConfigUpdate::= CHOICE {

init-eNB			ProtocolIE-Container	{{ENB-ENDCConfigUpdateIEs}},

init-en-gNB			ProtocolIE-Container	{{En-gNB-ENDCConfigUpdateIEs}},

...

}

ENB-ENDCConfigUpdateIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellAssistanceInformation				CRITICALITY reject	TYPE CellAssistanceInformation							PRESENCE optional }|

{ ID id-ServedEUTRAcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedEUTRAcellsENDCX2ManagementList			PRESENCE optional }|

{ ID id-ServedEUTRAcellsToModifyListENDCConfUpd	CRITICALITY reject	TYPE ServedEUTRAcellsToModifyListENDCConfUpd		PRESENCE optional }|

{ ID id-ServedEUTRAcellsToDeleteListENDCConfUpd	CRITICALITY reject	TYPE ServedEUTRAcellsToDeleteListENDCConfUpd		PRESENCE optional },

...

}

ServedEUTRAcellsToModifyListENDCConfUpd ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

old-ECGI								ECGI,

servedEUTRACellInfo					ServedCell-Information,

nrNeighbourInfo						NRNeighbour-Information		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ServedEUTRAcellsToModifyListENDCConfUpd-ExtIEs} } OPTIONAL,

...

}

ServedEUTRAcellsToModifyListENDCConfUpd-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ServedEUTRAcellsToDeleteListENDCConfUpd ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ECGI

En-gNB-ENDCConfigUpdateIEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList			PRESENCE optional }|

{ ID id-ServedNRcellsToModifyListENDCConfUpd		CRITICALITY reject	TYPE ServedNRcellsToModifyENDCConfUpdList		PRESENCE optional }|

{ ID id-ServedNRcellsToDeleteListENDCConfUpd		CRITICALITY reject	TYPE ServedNRcellsToDeleteENDCConfUpdList		PRESENCE optional },

...

}

ServedNRcellsToModifyENDCConfUpdList ::= SEQUENCE (SIZE (1..maxCellinengNB)) OF ServedNRCellsToModify-Item

ServedNRCellsToModify-Item::= SEQUENCE {

old-nrcgi						NRCGI,

servedNRCellInformation			ServedNRCell-Information,

nrNeighbourInformation			NRNeighbour-Information			OPTIONAL,

nrDeactivationIndication			DeactivationIndication			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedNRCellsToModify-Item-ExtIEs} } OPTIONAL,

...

}

ServedNRCellsToModify-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ServedNRcellsToDeleteENDCConfUpdList ::= SEQUENCE (SIZE (1..maxCellinengNB)) OF NRCGI

-- **************************************************************

--

#### -- EN-DC CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

ENDCConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationUpdateAcknowledge-IEs}},

...

}

ENDCConfigurationUpdateAcknowledge-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcConfigUpdate			CRITICALITY reject	TYPE RespondingNodeType-EndcConfigUpdate			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject 	TYPE InterfaceInstanceIndication						PRESENCE optional }|

{ ID id-TNLConfigurationInfo						CRITICALITY ignore	TYPE TNLConfigurationInfo								PRESENCE optional },

...

}

RespondingNodeType-EndcConfigUpdate::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCConfigUpdateAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCConfigUpdateAckIEs}},

...

}

ENB-ENDCConfigUpdateAckIEs X2AP-PROTOCOL-IES ::= {

...

}

En-gNB-ENDCConfigUpdateAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedNRcellsENDCX2ManagementList		CRITICALITY reject	TYPE ServedNRcellsENDCX2ManagementList		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

ENDCConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationUpdateFailure-IEs}},

...

}

ENDCConfigurationUpdateFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-TimeToWait						CRITICALITY ignore	TYPE TimeToWait						PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- EN-DC CELL ACTIVATION REQUEST

--

-- **************************************************************

ENDCCellActivationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCCellActivationRequest-IEs}},

...

}

ENDCCellActivationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ServedNRCellsToActivate 		CRITICALITY reject	TYPE ServedNRCellsToActivate				PRESENCE mandatory}|

{ ID id-ActivationID			 		CRITICALITY reject	TYPE ActivationID						PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ServedNRCellsToActivate::= SEQUENCE (SIZE (1.. maxCellinengNB)) OF ServedNRCellsToActivate-Item

ServedNRCellsToActivate-Item::= SEQUENCE {

nrCellID						NRCGI,

iE-Extensions					ProtocolExtensionContainer { {ServedNRCellsToActivate-Item-ExtIEs} } OPTIONAL,

...

}

ServedNRCellsToActivate-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- EN-DC CELL ACTIVATION RESPONSE

--

-- **************************************************************

ENDCCellActivationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCCellActivationResponse-IEs}},

...

}

ENDCCellActivationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ActivatedNRCellList			CRITICALITY ignore	TYPE ActivatedNRCellList				PRESENCE mandatory}|

{ ID id-ActivationID					CRITICALITY reject	TYPE ActivationID						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

ActivatedNRCellList ::= SEQUENCE (SIZE (1.. maxCellinengNB)) OF ActivatedNRCellList-Item

ActivatedNRCellList-Item::= SEQUENCE {

nrCellID						NRCGI,

iE-Extensions						ProtocolExtensionContainer { {ActivatedNRCellList-Item-ExtIEs} } OPTIONAL,

...

}

ActivatedNRCellList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

--**************************************************************

--

#### -- EN-DC CELL ACTIVATION FAILURE

--

-- **************************************************************

ENDCCellActivationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCCellActivationFailure-IEs}},

...

}

ENDCCellActivationFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-ActivationID						CRITICALITY reject	TYPE ActivationID					PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SecondaryRATDataUsageReport-IEs}},

...

}

SecondaryRATDataUsageReport-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-SecondaryRATUsageReportList			CRITICALITY reject	TYPE SecondaryRATUsageReportList			PRESENCE mandatory}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SGNB ACTIVITY NOTIFICATION

--

-- **************************************************************

SgNBActivityNotification ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{SgNBActivityNotification-IEs}},

...

}

SgNBActivityNotification-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID						CRITICALITY reject	TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID						CRITICALITY reject	TYPE SgNB-UE-X2AP-ID							PRESENCE mandatory}|

{ ID id-UEContextLevelUserPlaneActivity		CRITICALITY ignore	TYPE UserPlaneTrafficActivityReport		PRESENCE optional}|

{ ID id-ERABActivityNotifyItemList			CRITICALITY ignore	TYPE ERABActivityNotifyItemList			PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension			CRITICALITY reject	TYPE UE-X2AP-ID-Extension					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC PARTIAL RESET REQUIRED

--

-- **************************************************************

ENDCPartialResetRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCPartialResetRequired-IEs}},

...

}

ENDCPartialResetRequired-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-UEs-ToBeReset				CRITICALITY reject	TYPE UEsToBeResetList					PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause								PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- EN-DC PARTIAL RESET CONFIRM

--

-- **************************************************************

ENDCPartialResetConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCPartialResetConfirm-IEs}},

...

}

ENDCPartialResetConfirm-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-UEs-Admitted-ToBeReset				CRITICALITY reject	TYPE UEsToBeResetList					PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication			CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-UTRA - NR CELL RESOURCE COORDINATION REQUEST

--

-- **************************************************************

EUTRANRCellResourceCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{EUTRANRCellResourceCoordinationRequest-IEs}},

...

}

EUTRANRCellResourceCoordinationRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EutranrCellResourceCoordination	CRITICALITY reject	TYPE InitiatingNodeType-EutranrCellResourceCoordination			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication							CRITICALITY reject 	TYPE InterfaceInstanceIndication										PRESENCE optional },

...

}

InitiatingNodeType-EutranrCellResourceCoordination ::= CHOICE {

initiate-eNB			ProtocolIE-Container	{{ENB-EUTRA-NRCellResourceCoordinationReqIEs}},

initiate-en-gNB			ProtocolIE-Container	{{En-gNB-EUTRA-NRCellResourceCoordinationReqIEs}},

...

}

ENB-EUTRA-NRCellResourceCoordinationReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication			CRITICALITY reject	TYPE DataTrafficResourceIndication					PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID					CRITICALITY reject	TYPE SpectrumSharingGroupID							PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinEUTRACoordinationReq	CRITICALITY reject	TYPE ListofEUTRACellsinEUTRACoordinationReq		PRESENCE mandatory},

...

}

En-gNB-EUTRA-NRCellResourceCoordinationReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication			CRITICALITY reject	TYPE DataTrafficResourceIndication				PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinNRCoordinationReq		CRITICALITY reject	TYPE ListofEUTRACellsinNRCoordinationReq		PRESENCE mandatory }|

{ ID id-SpectrumSharingGroupID					CRITICALITY reject	TYPE SpectrumSharingGroupID						PRESENCE mandatory}|

{ ID id-ListofNRCellsinNRCoordinationReq		CRITICALITY reject	TYPE ListofNRCellsinNRCoordinationReq			PRESENCE mandatory},

...

}

ListofEUTRACellsinEUTRACoordinationReq ::= SEQUENCE (SIZE (0..maxCellineNB)) OF ECGI

ListofEUTRACellsinNRCoordinationReq ::= SEQUENCE (SIZE (1..maxCellineNB)) OF ECGI

ListofNRCellsinNRCoordinationReq ::= SEQUENCE (SIZE (0..maxnoNRcellsSpectrumSharingWithE-UTRA)) OF NRCGI

-- **************************************************************

--

#### -- E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

EUTRANRCellResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{EUTRANRCellResourceCoordinationResponse-IEs}},

...

}

EUTRANRCellResourceCoordinationResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EutranrCellResourceCoordination		CRITICALITY reject	TYPE RespondingNodeType-EutranrCellResourceCoordination			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication	CRITICALITY reject 	TYPE InterfaceInstanceIndication	PRESENCE optional },

...

}

RespondingNodeType-EutranrCellResourceCoordination ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-EUTRA-NRCellResourceCoordinationReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-EUTRA-NRCellResourceCoordinationReqAckIEs}},

...

}

ENB-EUTRA-NRCellResourceCoordinationReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication				CRITICALITY reject	TYPE DataTrafficResourceIndication				PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID						CRITICALITY reject	TYPE SpectrumSharingGroupID							PRESENCE mandatory}|

{ ID id-ListofEUTRACellsinEUTRACoordinationResp		CRITICALITY reject	TYPE ListofEUTRACellsinEUTRACoordinationResp	PRESENCE mandatory},

...

}

En-gNB-EUTRA-NRCellResourceCoordinationReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-DataTrafficResourceIndication		CRITICALITY reject	TYPE DataTrafficResourceIndication			PRESENCE mandatory}|

{ ID id-SpectrumSharingGroupID				CRITICALITY reject	TYPE SpectrumSharingGroupID						PRESENCE mandatory}|

{ ID id-ListofNRCellsinNRCoordinationResp	CRITICALITY reject	TYPE ListofNRCellsinNRCoordinationResp		PRESENCE mandatory},

...

}

ListofEUTRACellsinEUTRACoordinationResp ::= SEQUENCE (SIZE (0..maxCellineNB)) OF ECGI

ListofNRCellsinNRCoordinationResp ::= SEQUENCE (SIZE (0..maxnoNRcellsSpectrumSharingWithE-UTRA)) OF NRCGI

-- **************************************************************

--

#### -- EN-DC X2 REMOVAL REQUEST

--

-- **************************************************************

ENDCX2RemovalRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2RemovalRequest-IEs}},

...

}

ENDCX2RemovalRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-InitiatingNodeType-EndcX2Removal			CRITICALITY reject	TYPE InitiatingNodeType-EndcX2Removal		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication					CRITICALITY reject 	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

InitiatingNodeType-EndcX2Removal ::= CHOICE {

init-eNB				ProtocolIE-Container	{{ENB-ENDCX2RemovalReqIEs}},

init-en-gNB			ProtocolIE-Container	{{En-gNB-ENDCX2RemovalReqIEs}},

...

}

ENB-ENDCX2RemovalReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory},

...

}

En-gNB-ENDCX2RemovalReqIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- EN-DC X2 REMOVAL RESPONSE

--

-- **************************************************************

ENDCX2RemovalResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2RemovalResponse-IEs}},

...

}

ENDCX2RemovalResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RespondingNodeType-EndcX2Removal		CRITICALITY reject	TYPE RespondingNodeType-EndcX2Removal		PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject 	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

RespondingNodeType-EndcX2Removal ::= CHOICE {

respond-eNB			ProtocolIE-Container	{{ENB-ENDCX2RemovalReqAckIEs}},

respond-en-gNB		ProtocolIE-Container	{{En-gNB-ENDCX2RemovalReqAckIEs}},

...

}

ENB-ENDCX2RemovalReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GlobalENB-ID								CRITICALITY reject	TYPE GlobalENB-ID										PRESENCE mandatory},

...

}

En-gNB-ENDCX2RemovalReqAckIEs X2AP-PROTOCOL-IES ::= {

{ ID id-Globalen-gNB-ID							CRITICALITY reject	TYPE GlobalGNB-ID										PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- EN-DC X2 REMOVAL FAILURE

--

-- **************************************************************

ENDCX2RemovalFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCX2RemovalFailure-IEs}},

...

}

ENDCX2RemovalFailure-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore		TYPE Cause								PRESENCE mandatory} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore		TYPE CriticalityDiagnostics				PRESENCE optional }|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 		TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- DATA FORWARDING ADDRESS INDICATION

--

-- **************************************************************

DataForwardingAddressIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataForwardingAddressIndication-IEs}},

...

}

DataForwardingAddressIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional }|

{ ID id-Old-eNB-UE-X2AP-ID					CRITICALITY ignore	TYPE UE-X2AP-ID									PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension		CRITICALITY ignore	TYPE UE-X2AP-ID-Extension						PRESENCE optional }|

{ ID id-E-RABs-DataForwardingAddress-List	CRITICALITY ignore	TYPE E-RABs-DataForwardingAddress-List		PRESENCE mandatory},

...

}

E-RABs-DataForwardingAddress-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-DataForwardingAddress-ItemIEs} }

E-RABs-DataForwardingAddress-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABs-DataForwardingAddress-Item	CRITICALITY ignore	TYPE E-RABs-DataForwardingAddress-Item	PRESENCE mandatory},

...

}

E-RABs-DataForwardingAddress-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

dl-GTPtunnelEndpoint			GTPtunnelEndpoint,

iE-Extensions					ProtocolExtensionContainer { {E-RABs-DataForwardingAddress-ItemExtIEs} } OPTIONAL,

...

}

E-RABs-DataForwardingAddress-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- GNB STATUS INDICATION

--

-- **************************************************************

GNBStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ { GNBStatusIndicationIEs} },

...

}

GNBStatusIndicationIEs X2AP-PROTOCOL-IES ::= {

{ ID id-GNBOverloadInformation			CRITICALITY ignore	TYPE GNBOverloadInformation			PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication		PRESENCE optional },

...

}

-- **************************************************************

--

-- EN-DC CONFIGURATION TRANSFER

--

-- **************************************************************

ENDCConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ENDCConfigurationTransfer-IEs}},

...

}

ENDCConfigurationTransfer-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-endcSONConfigurationTransfer	CRITICALITY ignore	TYPE EndcSONConfigurationTransfer			PRESENCE optional}|

{ ID id-InterfaceInstanceIndication		CRITICALITY reject 	TYPE InterfaceInstanceIndication				PRESENCE optional },

...

}

-- **************************************************************

--

-- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID			CRITICALITY reject	TYPE UE-X2AP-ID						PRESENCE mandatory	}|

{ ID id-SgNB-UE-X2AP-ID			CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation			CRITICALITY ignore	TYPE TraceActivation				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID			CRITICALITY reject	TYPE UE-X2AP-ID						PRESENCE mandatory	}|

{ ID id-SgNB-UE-X2AP-ID			CRITICALITY reject	TYPE SgNB-UE-X2AP-ID				PRESENCE mandatory	}|

{ ID id-EUTRANTraceID			CRITICALITY ignore	TYPE EUTRANTraceID					PRESENCE mandatory	},

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RAB-Item,

id-Number-of-Antennaports,

id-MBSFN-Subframe-Info,

id-PRACH-Configuration,

id-CSG-Id,

id-MDTConfiguration,

id-SignallingBasedMDTPLMNList,

id-MultibandInfoList,

id-FreqBandIndicatorPriority,

id-NeighbourTAC,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-MBMS-Service-Area-List,

id-HO-cause,

id-eARFCNExtension,

id-DL-EARFCNExtension,

id-UL-EARFCNExtension,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-NRrestrictioninEPSasSecondaryRAT,

id-NRrestrictionin5GS,

id-AdditionalSpecialSubframe-Info,

id-UEID,

id-enhancedRNTP,

id-ProSeUEtoNetworkRelaying,

id-M6Configuration,

id-M7Configuration,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-AdditionalSpecialSubframeExtension-Info,

id-BandwidthReducedSI,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDownlink,

id-extended-uEaggregateMaximumBitRateUplink,

id-E-RABUsageReport-Item,

id-SecondaryRATUsageReport-Item,

id-UEAppLayerMeasConfig,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-DownlinkPacketLossRate,

id-UplinkPacketLossRate,

id-serviceType,

id-ProtectedEUTRAResourceIndication,

id-NRS-NSSS-PowerOffset,

id-NSSS-NumOccasionDifferentPrecoder,

id-CNTypeRestrictions,

id-BluetoothMeasurementConfiguration,

id-WLANMeasurementConfiguration,

id-ECGI,

id-NRCGI,

id-MeNBCoordinationAssistanceInformation,

id-SgNBCoordinationAssistanceInformation,

id-NRNeighbourInfoToAdd,

id-LastNG-RANPLMNIdentity,

id-BPLMN-ID-Info-EUTRA,

id-NBIoT-UL-DL-AlignmentOffset,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

maxnoofBands,

maxnoofBPLMNs,

maxnoofAdditionalPLMNs,

maxnoofCells,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofForbLACs,

maxnoofForbTACs,

maxnoofNeighbours,

maxnoofPRBs,

maxNrOfErrors,

maxPools,

maxnoofMBSFN,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofMBMSServiceAreaIdentities,

maxnoofMDTPLMNs,

maxnoofCoMPHypothesisSet,

maxnoofCoMPCells,

maxUEReport,

maxCellReport,

maxnoofPA,

maxCSIProcess,

maxCSIReport,

maxSubband,

maxnooftimeperiods,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxUEsinengNBDU,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands,

maxnoofBluetoothName,

maxnoofWLANName,

maxofNRNeighbours,

maxnoofextBPLMNs,

maxnoofextBPLMNsminus1,

maxnoofBPLMNsminus1,

maxnoofTLAs,

maxnoofGTPTLAs

FROM X2AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM X2AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES

FROM X2AP-Containers;

#### -- A

ABSInformation ::= CHOICE {

fdd					ABSInformationFDD,

tdd					ABSInformationTDD,

abs-inactive		NULL,

...

}

ABSInformationFDD ::= SEQUENCE {

abs-pattern-info					BIT STRING (SIZE(40)),

numberOfCellSpecificAntennaPorts	ENUMERATED {one, two, four, ...},

measurement-subset					BIT STRING (SIZE(40)),

iE-Extensions						ProtocolExtensionContainer { {ABSInformationFDD-ExtIEs} } OPTIONAL,

...

}

ABSInformationFDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ABSInformationTDD ::= SEQUENCE {

abs-pattern-info					BIT STRING (SIZE(1..70, ...)),

numberOfCellSpecificAntennaPorts	ENUMERATED {one, two, four, ...},

measurement-subset					BIT STRING (SIZE(1..70, ...)),

iE-Extensions						ProtocolExtensionContainer { {ABSInformationTDD-ExtIEs} } OPTIONAL,

...

}

ABSInformationTDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ABS-Status ::= SEQUENCE {

dL-ABS-status								DL-ABS-status,

usableABSInformation						UsableABSInformation,

iE-Extensions								ProtocolExtensionContainer { {ABS-Status-ExtIEs} } OPTIONAL,

...

}

ABS-Status-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ActivationID ::= INTEGER (0..255)

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AdditionalSpecialSubframe-Info ::=		SEQUENCE {

additionalspecialSubframePatterns		AdditionalSpecialSubframePatterns,

cyclicPrefixDL							CyclicPrefixDL,

cyclicPrefixUL							CyclicPrefixUL,

iE-Extensions							ProtocolExtensionContainer { {AdditionalSpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

AdditionalSpecialSubframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalSpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

...

}

AdditionalSpecialSubframeExtension-Info ::=		SEQUENCE {

additionalspecialSubframePatternsExtension	AdditionalSpecialSubframePatternsExtension,

cyclicPrefixDL								CyclicPrefixDL,

cyclicPrefixUL								CyclicPrefixUL,

iE-Extensions								ProtocolExtensionContainer { {AdditionalSpecialSubframeExtension-Info-ExtIEs} } OPTIONAL,

...

}

AdditionalSpecialSubframeExtension-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalSpecialSubframePatternsExtension ::= ENUMERATED {

ssp10,

...

}

AerialUEsubscriptionInformation ::= ENUMERATED {

allowed,

not-allowed,

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfMDT ::= CHOICE {

cellBased					CellBasedMDT,

tABased						TABasedMDT,

pLMNWide					NULL,

...,

tAIBased					TAIBasedMDT

}

AreaScopeOfQMC ::= CHOICE {

cellBased					CellBasedQMC,

tABased						TABasedQMC,

tAIBased					TAIBasedQMC,

pLMNAreaBased				PLMNAreaBasedQMC,

...

}

AS-SecurityInformation ::= SEQUENCE {

key-eNodeB-star		Key-eNodeB-Star,

nextHopChainingCount			NextHopChainingCount,

iE-Extensions						ProtocolExtensionContainer { {AS-SecurityInformation-ExtIEs} } OPTIONAL,

...

}

AS-SecurityInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalPLMNs-Item ::= SEQUENCE (SIZE(1..maxnoofAdditionalPLMNs)) OF PLMN-Identity

#### -- B

BandwidthReducedSI::= ENUMERATED {

scheduled,

...

}

BearerType ::= ENUMERATED {

non-IP,

...

}

BenefitMetric ::= INTEGER (-101..100, ...)

BitRate ::= INTEGER (0..10000000000)

BroadcastPLMNs-Item ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig				BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList			OPTIONAL,

bt-rssi							ENUMERATED {true, ...}				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BluetoothMeasurementConfiguration-ExtIEs} } OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothName

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))

BPLMN-ID-Info-EUTRA ::= SEQUENCE (SIZE(1..maxnoofBPLMNsminus1)) OF BPLMN-ID-Info-EUTRA-Item

BPLMN-ID-Info-EUTRA-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastPLMNs-Item,

tac								TAC,

e-utraCI						EUTRANCellIdentifier,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-EUTRA-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofextBPLMNsminus1)) OF BPLMN-ID-Info-NR-Item

BPLMN-ID-Info-NR-Item ::= SEQUENCE {

broadcastPLMNs					BroadcastextPLMNs,

fiveGStac						FiveGS-TAC		OPTIONAL,

nr-CI							NRCellIdentifier,

iE-Extension					ProtocolExtensionContainer { {BPLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-NR-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastextPLMNs ::= SEQUENCE (SIZE(1..maxnoofextBPLMNs)) OF PLMN-Identity

#### -- C

CapacityValue ::= INTEGER (0..100)

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

hardware-failure,

om-intervention,

not-enough-user-plane-processing-resources,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

handover-desirable-for-radio-reasons,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

partial-handover,

unknown-new-eNB-UE-X2AP-ID,

unknown-old-eNB-UE-X2AP-ID,

unknown-pair-of-UE-X2AP-ID,

ho-target-not-allowed,

tx2relocoverall-expiry,

trelocprep-expiry,

cell-not-available,

no-radio-resources-available-in-target-cell,

invalid-MME-GroupID,

unknown-MME-Code,

encryption-and-or-integrity-protection-algorithms-not-supported,

reportCharacteristicsEmpty,

noReportPeriodicity,

existingMeasurementID,

unknown-eNB-Measurement-ID,

measurement-temporarily-not-available,

unspecified,

...,

load-balancing,

handover-optimisation,

value-out-of-allowed-range,

multiple-E-RAB-ID-instances,

switch-off-ongoing,

not-supported-QCI-value,

measurement-not-supported-for-the-object,

tDCoverall-expiry,

tDCprep-expiry,

action-desirable-for-radio-reasons,

reduce-load,

resource-optimisation,

time-critical-action,

target-not-allowed,

no-radio-resources-available,

invalid-QoS-combination,

encryption-algorithms-not-supported,

procedure-cancelled,

rRM-purpose,

improve-user-bit-rate,

user-inactivity,

radio-connection-with-UE-lost,

failure-in-the-radio-interface-procedure,

bearer-option-not-supported,

mCG-Mobility,

sCG-Mobility,

count-reaches-max-value,

unknown-old-en-gNB-UE-X2AP-ID,

pDCP-Overload

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

CellBasedMDT::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellBasedQMC::= SEQUENCE {

cellIdListforQMC		CellIdListforQMC,

iE-Extensions		ProtocolExtensionContainer { {CellBasedQMC-ExtIEs} } OPTIONAL,

...

}

CellBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellCapacityClassValue ::= INTEGER (1..100, ...)

CellDeploymentStatusIndicator ::= ENUMERATED {pre-change-notification, ...}

CellIdListforMDT ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF ECGI

CellIdListforQMC ::= SEQUENCE (SIZE(1..maxnoofCellIDforQMC)) OF ECGI

CellReplacingInfo ::= SEQUENCE {

replacingCellsList				ReplacingCellsList,

iE-Extensions					ProtocolExtensionContainer { {CellReplacingInfo-ExtIEs}}	OPTIONAL,

...

}

CellReplacingInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CellReportingIndicator ::= ENUMERATED {stop-request, ... }

Cell-Size ::= ENUMERATED {verysmall, small, medium, large, ... }

CellType ::= SEQUENCE {

cell-Size						Cell-Size,

iE-Extensions					ProtocolExtensionContainer { {CellType-ExtIEs}}	OPTIONAL,

...

}

CellType-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CNTypeRestrictions ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF CNTypeRestrictionsItem

CNTypeRestrictionsItem ::= SEQUENCE {

plmn-Id				PLMN-Identity,

cn-type				ENUMERATED {fiveGC-forbidden, ... , epc-forbidden},

iE-Extensions		ProtocolExtensionContainer { {CNTypeRestrictionsItem-ExtIEs} } OPTIONAL,

...

}

CNTypeRestrictionsItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPHypothesisSet ::= SEQUENCE (SIZE(1..maxnoofCoMPCells)) OF CoMPHypothesisSetItem

CoMPHypothesisSetItem ::= SEQUENCE {

coMPCellID						ECGI,

coMPHypothesis					BIT STRING (SIZE(6..4400, ...)),

iE-Extensions					ProtocolExtensionContainer { {CoMPHypothesisSetItem-ExtIEs} } OPTIONAL,

...

}

CoMPHypothesisSetItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPInformation ::= SEQUENCE {

coMPInformationItem						CoMPInformationItem,

coMPInformationStartTime				CoMPInformationStartTime,

iE-Extensions							ProtocolExtensionContainer { {CoMPInformation-ExtIEs} } OPTIONAL,

...

}

CoMPInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPInformationItem ::= SEQUENCE (SIZE(1..maxnoofCoMPHypothesisSet)) OF

SEQUENCE {

coMPHypothesisSet					CoMPHypothesisSet,

benefitMetric						BenefitMetric,

iE-Extensions						ProtocolExtensionContainer { {CoMPInformationItem-ExtIEs} } OPTIONAL,

...

}

CoMPInformationItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoMPInformationStartTime ::= SEQUENCE (SIZE(0..1)) OF

SEQUENCE {

startSFN							INTEGER (0..1023, ...),

startSubframeNumber					INTEGER (0..9, ...),

iE-Extensions						ProtocolExtensionContainer { {CoMPInformationStartTime-ExtIEs} } OPTIONAL,

...

}

CoMPInformationStartTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue							CellCapacityClassValue				OPTIONAL,

capacityValue									CapacityValue,

iE-Extensions									ProtocolExtensionContainer { {CompositeAvailableCapacity-ExtIEs} } OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacityGroup	::= SEQUENCE {

dL-CompositeAvailableCapacity					CompositeAvailableCapacity,

uL-CompositeAvailableCapacity					CompositeAvailableCapacity,

iE-Extensions									ProtocolExtensionContainer { {CompositeAvailableCapacityGroup-ExtIEs} } OPTIONAL,

...

}

CompositeAvailableCapacityGroup-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Correlation-ID ::= OCTET STRING (SIZE (4))

COUNTvalue ::= SEQUENCE {

pDCP-SN					PDCP-SN,

hFN						HFN,

iE-Extensions			ProtocolExtensionContainer { {COUNTvalue-ExtIEs} } OPTIONAL,

...

}

COUNTvalue-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueExtended ::= SEQUENCE {

pDCP-SNExtended			PDCP-SNExtended,

hFNModified				HFNModified,

iE-Extensions			ProtocolExtensionContainer { {COUNTValueExtended-ExtIEs} } OPTIONAL,

...

}

COUNTValueExtended-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

COUNTvaluePDCP-SNlength18 ::= SEQUENCE {

pDCP-SNlength18			PDCP-SNlength18,

hFNforPDCP-SNlength18	HFNforPDCP-SNlength18,

iE-Extensions			ProtocolExtensionContainer { {COUNTvaluePDCP-SNlength18-ExtIEs} } OPTIONAL,

...

}

COUNTvaluePDCP-SNlength18-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CoverageModificationList ::= SEQUENCE (SIZE (1..maxCellineNB)) OF CoverageModification-Item

CoverageModification-Item ::= SEQUENCE {

eCGI							ECGI,

coverageState					INTEGER (0..15, ...),

cellDeploymentStatusIndicator	CellDeploymentStatusIndicator			OPTIONAL,

cellReplacingInfo				CellReplacingInfo						OPTIONAL,

-- Included in case the Cell Deployment Status Indicator IE is present

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CRNTI ::= BIT STRING (SIZE (16))

CSGMembershipStatus ::= ENUMERATED {

member,

not-member

}

CSG-Id ::= BIT STRING (SIZE (27))

CSIReportList ::= SEQUENCE (SIZE(1..maxUEReport)) OF

SEQUENCE {

uEID							UEID,

cSIReportPerCSIProcess			CSIReportPerCSIProcess,

iE-Extensions					ProtocolExtensionContainer { {CSIReportList-ExtIEs} } OPTIONAL,

...

}

CSIReportList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CSIReportPerCSIProcess ::= SEQUENCE (SIZE(1.. maxCSIProcess)) OF

SEQUENCE {

cSIProcessConfigurationIndex	INTEGER (1..7, ...),

cSIReportPerCSIProcessItem		CSIReportPerCSIProcessItem,

iE-Extensions					ProtocolExtensionContainer { {CSIReportPerCSIProcess-ExtIEs} } OPTIONAL,

...

}

CSIReportPerCSIProcess-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CSIReportPerCSIProcessItem ::= SEQUENCE (SIZE(1.. maxCSIReport)) OF

SEQUENCE {

rI								INTEGER (1..8, ...),

widebandCQI						WidebandCQI,

subbandSize						SubbandSize,

subbandCQIList					SubbandCQIList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CSIReportPerCSIProcessItem-ExtIEs} } OPTIONAL,

...

}

CSIReportPerCSIProcessItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CyclicPrefixDL ::= ENUMERATED {

normal,

extended,

...

}

CyclicPrefixUL ::= ENUMERATED {

normal,

extended,

...

}

#### -- D

DataTrafficResources ::= BIT STRING (SIZE(6..17600))

DataTrafficResourceIndication ::= SEQUENCE {

activationSFN					INTEGER (0..1023),

sharedResourceType				SharedResourceType,

reservedSubframePattern 		ReservedSubframePattern OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DataTrafficResourceIndication-ExtIEs} } OPTIONAL,

...

}

DataTrafficResourceIndication-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DeactivationIndication::= ENUMERATED {

deactivated,

...

}

DeliveryStatus ::= SEQUENCE {

highestSuccessDeliveredPDCPSN		INTEGER (0..4095),

iE-Extensions		ProtocolExtensionContainer { {DeliveryStatus-ExtIEs} } OPTIONAL,

...

}

DeliveryStatus-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

DesiredActNotificationLevel	::= ENUMERATED {none, e-rab, ue-level, ...}

DL-ABS-status::= INTEGER (0..100)

DL-Forwarding ::= ENUMERATED {

dL-forwardingProposed,

...

}

DL-GBR-PRB-usage::= INTEGER (0..100)

DL-non-GBR-PRB-usage::= INTEGER (0..100)

DLResourceBitmapULandDLSharing ::= DataTrafficResources

DLResourcesULandDLSharing ::= CHOICE {

unchanged			NULL,

changed				DLResourceBitmapULandDLSharing,

...

}

DL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

DL-Total-PRB-usage::= INTEGER (0..100)

DRB-ID ::= INTEGER (1..32)

DuplicationActivation::= ENUMERATED {active, inactive, ...}

DynamicDLTransmissionInformation ::= CHOICE {

naics-active			DynamicNAICSInformation,

naics-inactive			NULL,

...

}

DynamicNAICSInformation ::= SEQUENCE {

transmissionModes					BIT STRING (SIZE(8))												OPTIONAL,

pB-information						INTEGER(0..3)														OPTIONAL,

pA-list								SEQUENCE (SIZE(0..maxnoofPA)) OF PA-Values,

iE-Extensions						ProtocolExtensionContainer { {DynamicNAICSInformation-ExtIEs} } OPTIONAL,

...

}

DynamicNAICSInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

EARFCN ::= INTEGER (0..maxEARFCN)

EARFCNExtension ::= INTEGER(maxEARFCNPlusOne..newmaxEARFCN, ...)

ECGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRANcellIdentifier		EUTRANCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {ECGI-ExtIEs} } OPTIONAL,

...

}

ECGI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EndcSONConfigurationTransfer ::= OCTET STRING

EnhancedRNTP ::= SEQUENCE {

enhancedRNTPBitmap			BIT STRING (SIZE(12..8800, ...)),

rNTP-High-Power-Threshold	RNTP-Threshold,

enhancedRNTPStartTime		EnhancedRNTPStartTime OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {EnhancedRNTP-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTP-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EnhancedRNTPStartTime ::= SEQUENCE {

startSFN				INTEGER (0..1023, ...),

startSubframeNumber		INTEGER (0..9, ...),

iE-Extensions			ProtocolExtensionContainer { {EnhancedRNTPStartTime-ExtIEs} } OPTIONAL,

...

}

EnhancedRNTPStartTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ENB-ID ::= CHOICE {

macro-eNB-ID	BIT STRING (SIZE (20)),

home-eNB-ID		BIT STRING (SIZE (28)),

... ,

short-Macro-eNB-ID		BIT STRING (SIZE(18)),

long-Macro-eNB-ID		BIT STRING (SIZE(21))

}

EncryptionAlgorithms ::= BIT STRING (SIZE (16, ...))

EN-DC-ResourceConfiguration ::= SEQUENCE {

pDCPatSgNB		ENUMERATED {present, not-present, ...},

mCGresources	ENUMERATED {present, not-present, ...},

sCGresources	ENUMERATED {present, not-present, ...},

iE-Extensions					ProtocolExtensionContainer { {EN-DC-ResourceConfigurationExtIEs} }	OPTIONAL,

...

}

EN-DC-ResourceConfigurationExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMN-Identity

ERABActivityNotifyItemList ::= SEQUENCE (SIZE (0..maxnoofBearers)) OF ERABActivityNotifyItem

ERABActivityNotifyItem ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

activityReport					UserPlaneTrafficActivityReport,

iE-Extensions					ProtocolExtensionContainer { {ERABActivityNotifyItem-ExtIEs} }	OPTIONAL,

...

}

ERABActivityNotifyItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-ID ::= INTEGER (0..15, ...)

E-RAB-Level-QoS-Parameters ::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {E-RAB-Level-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

E-RAB-Level-QoS-Parameters-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extended for introduction of downlink and uplink packet loss rate for enhanced Voice performance –

{ ID id-DownlinkPacketLossRate			CRITICALITY ignore	EXTENSION Packet-LossRate		PRESENCE optional}|

{ ID id-UplinkPacketLossRate			CRITICALITY ignore	EXTENSION Packet-LossRate		PRESENCE optional},

...

}

E-RAB-List ::= SEQUENCE (SIZE(1.. maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RAB-ItemIEs} }

E-RAB-ItemIEs 	X2AP-PROTOCOL-IES ::= {

{ ID id-E-RAB-Item	 CRITICALITY ignore 	TYPE E-RAB-Item 	PRESENCE mandatory },

...

}

E-RAB-Item ::= SEQUENCE {

e-RAB-ID				E-RAB-ID,

cause						Cause,

iE-Extensions				ProtocolExtensionContainer { {E-RAB-Item-ExtIEs} } OPTIONAL,

...

}

E-RAB-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

E-RABUsageReportList ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF ProtocolIE-Single-Container { {E-RABUsageReport-ItemIEs} }

E-RABUsageReport-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-E-RABUsageReport-Item	 CRITICALITY ignore 	TYPE E-RABUsageReport-Item 	PRESENCE mandatory },

...

}

E-RABUsageReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { {E-RABUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

E-RABUsageReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

...

}

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

EUTRANTraceID		::= OCTET STRING (SIZE (8))

EventType ::= ENUMERATED{

change-of-serving-cell,

...

}

ExpectedUEBehaviour ::= SEQUENCE {

expectedActivity		ExpectedUEActivityBehaviour OPTIONAL,

expectedHOInterval		ExpectedHOInterval			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod					ExpectedActivityPeriod					OPTIONAL,

expectedIdlePeriod						ExpectedIdlePeriod						OPTIONAL,

sourceofUEActivityBehaviourInformation	SourceOfUEActivityBehaviourInformation	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} } OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181,...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExtendedULInterferenceOverloadInfo ::= SEQUENCE {

associatedSubframes							BIT STRING (SIZE (5)),

extended-ul-InterferenceOverloadIndication	UL-InterferenceOverloadIndication,

iE-Extensions								ProtocolExtensionContainer { {ExtendedULInterferenceOverloadInfo-ExtIEs} } OPTIONAL,

...

}

ExtendedULInterferenceOverloadInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedBitRate	::= INTEGER (10000000001..4000000000000,...)

#### -- F

FastMCGRecovery	::= SEQUENCE {

rrcContainer 				RRCContainer			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {FastMCGRecovery-ExtIEs} } OPTIONAL,

...

}

FastMCGRecovery-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

FastMCGRecoveryViaSRB3Enabled ::= ENUMERATED {true,...}

FastMCGRecoveryViaSRB3Disabled ::= ENUMERATED {true,...}

FDD-Info ::= SEQUENCE {

uL-EARFCN						EARFCN,

dL-EARFCN						EARFCN,

uL-Transmission-Bandwidth		Transmission-Bandwidth,

dL-Transmission-Bandwidth		Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {FDD-Info-ExtIEs} } OPTIONAL,

...

}

FDD-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UL-EARFCNExtension						CRITICALITY reject	EXTENSION EARFCNExtension									PRESENCE optional}|

{ ID id-DL-EARFCNExtension						CRITICALITY reject	EXTENSION EARFCNExtension									PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToUL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NRS-NSSS-PowerOffset					CRITICALITY ignore	EXTENSION NRS-NSSS-PowerOffset							PRESENCE optional}|

{ ID id-NSSS-NumOccasionDifferentPrecoder		CRITICALITY ignore	EXTENSION NSSS-NumOccasionDifferentPrecoder			PRESENCE optional},

...

}

FDD-InfoNeighbourServedNRCell-Information ::= SEQUENCE {

ul-NRFreqInfo			NRFreqInfo,

dl-NRFreqInfo			NRFreqInfo,

iE-Extensions			ProtocolExtensionContainer { {FDD-InfoNeighbourServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

FDD-InfoNeighbourServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenInterRATs ::= ENUMERATED {

all,

geran,

utran,

cdma2000,

...,

geranandutran,

cdma2000andutran

}

ForbiddenTAs ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenTAs-Item

ForbiddenTAs-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

forbiddenTACs		ForbiddenTACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenTAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenTAs-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF TAC

ForbiddenLAs ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF ForbiddenLAs-Item

ForbiddenLAs-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

forbiddenLACs		ForbiddenLACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenLAs-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenLAs-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenLACs ::= SEQUENCE (SIZE(1..maxnoofForbLACs)) OF LAC

Fourframes ::= BIT STRING (SIZE (24))

FreqBandIndicator ::= INTEGER (1..256, ...)

FreqBandIndicatorPriority ::= ENUMERATED {

not-broadcasted,

broadcasted,

...

}

FreqBandNrItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

supportedSULBandList	SEQUENCE (SIZE(0..maxnoofNrCellBands)) OF SupportedSULFreqBandItem,

iE-Extensions				ProtocolExtensionContainer { {FreqBandNrItem-ExtIEs} } OPTIONAL,

...

}

FreqBandNrItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { {GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extension for maximum bitrate > 10Gbps --

{ ID id-extended-e-RAB-MaximumBitrateDL		CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-MaximumBitrateUL		CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateDL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-e-RAB-GuaranteedBitrateUL	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

GlobalENB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalENB-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GlobalGNB-ID ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

gNB-ID					GNB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalGNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalGNB-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GNBOverloadInformation ::= ENUMERATED {overloaded, not-overloaded, ...}

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses			TransportLayerAddress,

iE-Extensions						ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } } OPTIONAL,

...

}

GTPTLA-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTPtunnelEndpoint ::= SEQUENCE {

transportLayerAddress			TransportLayerAddress,

gTP-TEID						GTP-TEI,

iE-Extensions					ProtocolExtensionContainer { {GTPtunnelEndpoint-ExtIEs} } OPTIONAL,

...

}

GTPtunnelEndpoint-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-TEI					::= OCTET STRING (SIZE (4))

GUGroupIDList		::= SEQUENCE (SIZE (1..maxPools)) OF GU-Group-ID

GU-Group-ID			::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

mME-Group-ID		MME-Group-ID,

iE-Extensions		ProtocolExtensionContainer { {GU-Group-ID-ExtIEs} } OPTIONAL,

...

}

GU-Group-ID-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GUMMEI			::= SEQUENCE {

gU-Group-ID		GU-Group-ID,

mME-Code			MME-Code,

iE-Extensions					ProtocolExtensionContainer { {GUMMEI-ExtIEs} } OPTIONAL,

...

}

GUMMEI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID ::= CHOICE {

gNB-ID	BIT STRING (SIZE (22..32)),

...

}

#### -- H

HandoverReportType ::= ENUMERATED {

hoTooEarly,

hoToWrongCell,

...,

interRATpingpong

}

HandoverRestrictionList ::= SEQUENCE {

servingPLMN					PLMN-Identity,

equivalentPLMNs				EPLMNs					OPTIONAL,

forbiddenTAs				ForbiddenTAs			OPTIONAL,

forbiddenLAs				ForbiddenLAs			OPTIONAL,

forbiddenInterRATs			ForbiddenInterRATs		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {HandoverRestrictionList-ExtIEs} }	OPTIONAL,

...

}

HandoverRestrictionList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRrestrictioninEPSasSecondaryRAT			CRITICALITY ignore	EXTENSION NRrestrictioninEPSasSecondaryRAT			PRESENCE optional}|

{ ID id-CNTypeRestrictions						CRITICALITY ignore	EXTENSION CNTypeRestrictions								PRESENCE optional}|

{ ID id-NRrestrictionin5GS						CRITICALITY ignore	EXTENSION NRrestrictionin5GS								PRESENCE optional}|

{ ID id-LastNG-RANPLMNIdentity					CRITICALITY ignore	EXTENSION PLMN-Identity										PRESENCE optional},

...

}

HFN ::= INTEGER (0..1048575)

HFNModified ::= INTEGER (0..131071)

HFNforPDCP-SNlength18 ::= INTEGER (0..16383)

HWLoadIndicator ::= SEQUENCE {

dLHWLoadIndicator			LoadIndicator,

uLHWLoadIndicator			LoadIndicator,

iE-Extensions				ProtocolExtensionContainer { {HWLoadIndicator-ExtIEs} } OPTIONAL,

...

}

HWLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- I

IntegrityProtectionAlgorithms ::= BIT STRING (SIZE (16, ...))

InterfaceInstanceIndication ::= INTEGER (0..255, ...)

InterfacesToTrace ::= BIT STRING (SIZE (8))

InvokeIndication ::= ENUMERATED{

abs-information,

...,

naics-information-start,

naics-information-stop

}

#### -- J

#### -- K

Key-eNodeB-Star ::= BIT STRING (SIZE(256))

#### -- L

LAC					::= OCTET STRING (SIZE (2)) --(EXCEPT ('0000'H|'FFFE'H))

LastVisitedCell-Item ::= CHOICE {

e-UTRAN-Cell					LastVisitedEUTRANCellInformation,

uTRAN-Cell						LastVisitedUTRANCellInformation,

gERAN-Cell						LastVisitedGERANCellInformation,

...,

nG-RAN-Cell						LastVisitedNGRANCellInformation

}

LastVisitedEUTRANCellInformation ::= SEQUENCE {

global-Cell-ID					ECGI,

cellType						CellType,

time-UE-StayedInCell			Time-UE-StayedInCell,

iE-Extensions					ProtocolExtensionContainer { {LastVisitedEUTRANCellInformation-ExtIEs} } OPTIONAL,

...

}

LastVisitedEUTRANCellInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-11 to support enhanced granularity for time UE stayed in cell --

{ ID id-Time-UE-StayedInCell-EnhancedGranularity	CRITICALITY ignore	EXTENSION Time-UE-StayedInCell-EnhancedGranularity	PRESENCE optional}|

{ ID id-HO-cause									CRITICALITY ignore	EXTENSION Cause													PRESENCE optional},

...

}

LastVisitedGERANCellInformation ::= CHOICE {

undefined						NULL,

...

}

LastVisitedNGRANCellInformation	::= OCTET STRING

LastVisitedUTRANCellInformation	::= OCTET STRING

LCID ::= INTEGER(1..32, ...)

LHN-ID ::= OCTET STRING(SIZE (32..256))

Links-to-log ::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

LoadIndicator ::= ENUMERATED {

lowLoad,

mediumLoad,

highLoad,

overLoad,

...

}

LocationInformationSgNB ::= SEQUENCE {

pSCell-id			NRCGI,

iE-Extensions		ProtocolExtensionContainer { {LocationInformationSgNB-ExtIEs} } OPTIONAL,

...

}

LocationInformationSgNB-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

LocationInformationSgNBReporting ::= ENUMERATED {

pSCell,

...

}

LocationReportingInformation ::= SEQUENCE {

eventType		EventType,

reportArea		ReportArea,

iE-Extensions		ProtocolExtensionContainer { {LocationReportingInformation-ExtIEs} } OPTIONAL,

...

}

LocationReportingInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

suspend-lower-layers,

resume-lower-layers,

...

}

#### -- M

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions				ProtocolExtensionContainer { {M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M1ReportingTrigger::= ENUMERATED{

periodic,

a2eventtriggered,

...,

a2eventtriggered-periodic

}

M1ThresholdEventA2 ::= SEQUENCE {

measurementThreshold		MeasurementThresholdA2,

iE-Extensions				ProtocolExtensionContainer { {M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M3Configuration ::= SEQUENCE {

m3period			M3period,

iE-Extensions		ProtocolExtensionContainer { {M3Configuration-ExtIEs} } OPTIONAL,

...

}

M3Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M3period ::= ENUMERATED {ms100, ms1000, ms10000, ... }

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-interval	M6report-interval,

m6delay-threshold	M6delay-threshold	OPTIONAL,

-- This IE shall be present if the M6 Links to log IE is set to “uplink” or to “both-uplink-and-downlink” --

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M6report-interval ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, ... }

M6delay-threshold ::= ENUMERATED { ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, ... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { {M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

MakeBeforeBreakIndicator::= ENUMERATED {true, ...}

ManagementBasedMDTallowed ::= ENUMERATED {allowed, ...}

Masked-IMEISV ::= BIT STRING (SIZE (64))

MDT-Activation		::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

...

}

MDT-Configuration ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT				AreaScopeOfMDT,

measurementsToActivate		MeasurementsToActivate,

m1reportingTrigger			M1ReportingTrigger,

m1thresholdeventA2			M1ThresholdEventA2			OPTIONAL,

-- Included in case of event-triggered, or event-triggered periodic reporting for measurement M1

m1periodicReporting			M1PeriodicReporting			OPTIONAL,

-- Included in case of periodic, or event-triggered periodic reporting for measurement M1

iE-Extensions				ProtocolExtensionContainer { {MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-M3Configuration							CRITICALITY ignore	EXTENSION M3Configuration								PRESENCE conditional}|

{ID id-M4Configuration							CRITICALITY ignore	EXTENSION M4Configuration								PRESENCE conditional}|

{ID id-M5Configuration							CRITICALITY ignore	EXTENSION M5Configuration								PRESENCE conditional}|

{ID id-MDT-Location-Info						CRITICALITY ignore	EXTENSION MDT-Location-Info							PRESENCE optional}|

{ID id-SignallingBasedMDTPLMNList				CRITICALITY ignore	EXTENSION MDTPLMNList									PRESENCE optional}|

{ID id-M6Configuration							CRITICALITY ignore	EXTENSION M6Configuration								PRESENCE conditional}|

{ID id-M7Configuration							CRITICALITY ignore	EXTENSION M7Configuration								PRESENCE conditional}|

{ ID id-BluetoothMeasurementConfiguration		CRITICALITY ignore	EXTENSION BluetoothMeasurementConfiguration		PRESENCE optional}|

{ ID id-WLANMeasurementConfiguration			CRITICALITY ignore	EXTENSION WLANMeasurementConfiguration			PRESENCE optional},

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDT-Location-Info ::= BIT STRING (SIZE (8))

Measurement-ID	::= INTEGER (1..4095, ...)

MeasurementsToActivate::= BIT STRING (SIZE (8))

MeasurementThresholdA2 ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

...

}

MeNBCoordinationAssistanceInformation ::= ENUMERATED{

coordination-not-required,

...

}

MeNBResourceCoordinationInformation ::= SEQUENCE {

eUTRA-Cell-ID					ECGI,

uLCoordinationInformation		BIT STRING (SIZE(6..4400, ...)),

dLCoordinationInformation		BIT STRING (SIZE(6..4400, ...))		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MeNBResourceCoordinationInformationExtIEs} }		 OPTIONAL,

...

}

MeNBResourceCoordinationInformationExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI									CRITICALITY ignore	EXTENSION NRCGI												PRESENCE optional}|

{ ID id-MeNBCoordinationAssistanceInformation		CRITICALITY reject	EXTENSION MeNBCoordinationAssistanceInformation	PRESENCE optional},

...

}

MeNBtoSeNBContainer ::= OCTET STRING

MME-Group-ID	::= OCTET STRING (SIZE (2))

MME-Code		::= OCTET STRING (SIZE (1))

MBMS-Service-Area-Identity-List ::= SEQUENCE (SIZE(1.. maxnoofMBMSServiceAreaIdentities)) OF MBMS-Service-Area-Identity

MBMS-Service-Area-Identity ::= OCTET STRING (SIZE (2))

MBSFN-Subframe-Infolist::= SEQUENCE (SIZE(1.. maxnoofMBSFN)) OF MBSFN-Subframe-Info

MBSFN-Subframe-Info	::= SEQUENCE {

radioframeAllocationPeriod		RadioframeAllocationPeriod,

radioframeAllocationOffset		RadioframeAllocationOffset,

subframeAllocation				SubframeAllocation,

iE-Extensions					ProtocolExtensionContainer { {MBSFN-Subframe-Info-ExtIEs} } 	OPTIONAL,

...

}

MBSFN-Subframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MobilityParametersModificationRange ::= SEQUENCE {

handoverTriggerChangeLowerLimit		INTEGER (-20..20),

handoverTriggerChangeUpperLimit		INTEGER (-20..20),

...

}

MobilityParametersInformation ::= SEQUENCE {

handoverTriggerChange			INTEGER (-20..20),

...

}

MultibandInfoList ::= SEQUENCE (SIZE(1..maxnoofBands)) OF BandInfo

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extensions					ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs} } 	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

BandInfo	::= SEQUENCE {

freqBandIndicator		FreqBandIndicator,

iE-Extensions			ProtocolExtensionContainer { {BandInfo-ExtIEs} } 	OPTIONAL,

...

}

BandInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

MeNBtoSgNBContainer ::= OCTET STRING

SplitSRBs ::= ENUMERATED {srb1, srb2, srb1and2, ...}

SplitSRB ::= SEQUENCE {

rrcContainer 				RRCContainer			OPTIONAL,

srbType						SRBType,

deliveryStatus				DeliveryStatus			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SplitSRB-ExtIEs} } OPTIONAL,

...

}

SplitSRB-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- N

NBIoT-UL-DL-AlignmentOffset ::= ENUMERATED {

khz-7dot5,

khz0,

khz7dot5,

...

}

Neighbour-Information ::= SEQUENCE (SIZE (0..maxnoofNeighbours)) OF SEQUENCE {

eCGI						ECGI,

pCI							PCI,

eARFCN						EARFCN,

iE-Extensions				ProtocolExtensionContainer { {Neighbour-Information-ExtIEs} } OPTIONAL,

...

}

Neighbour-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NeighbourTAC		CRITICALITY ignore	EXTENSION TAC				PRESENCE optional}|

{ ID id-eARFCNExtension		CRITICALITY reject	EXTENSION EARFCNExtension	PRESENCE optional},

...

}

NextHopChainingCount ::= INTEGER (0..7)

NewDRBIDrequest::= ENUMERATED {true, ...}

Number-of-Antennaports ::= ENUMERATED {

an1,

an2,

an4,

...

}

NRFreqInfo ::= SEQUENCE{

nRARFCN			INTEGER (0.. 3279165),

freqBandListNr	SEQUENCE (SIZE(1..maxnoofNrCellBands)) OF FreqBandNrItem,

sULInformation	SULInformation		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { {NRFreqInfo-ExtIEs} } OPTIONAL,

...

}

NRFreqInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRCellIdentifier ::= BIT STRING (SIZE (36))

NRCGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nRcellIdentifier				NRCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {NRCGI-ExtIEs} } OPTIONAL,

...

}

NRCGI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRNeighbour-Information ::= SEQUENCE (SIZE (1.. maxofNRNeighbours))OF SEQUENCE {

nrpCI							NRPCI,

nrCellID						NRCGI,

fiveGS-TAC						FiveGS-TAC	OPTIONAL,

configured-TAC					TAC			OPTIONAL,

measurementTimingConfiguration	OCTET STRING,

nRNeighbourModeInfo				CHOICE {

fdd		FDD-InfoNeighbourServedNRCell-Information,

tdd		TDD-InfoNeighbourServedNRCell-Information,

...

},

iE-Extensions						ProtocolExtensionContainer { {NRNeighbour-Information-ExtIEs} } OPTIONAL,

...

}

NRNeighbour-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRPCI ::= INTEGER (0..1007)

NRrestrictioninEPSasSecondaryRAT ::= ENUMERATED {

nRrestrictedinEPSasSecondaryRAT,

...

}

NRrestrictionin5GS ::= ENUMERATED {

nRrestrictedin5GS,

...

}

NRencryptionAlgorithms ::= BIT STRING (SIZE (16,...))

NRintegrityProtectionAlgorithms ::= BIT STRING (SIZE (16,...))

NR-TxBW	::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { {NR-TxBW-ExtIEs} } OPTIONAL,

...

}

NR-TxBW-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ...}

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ...}

NRS-NSSS-PowerOffset ::= ENUMERATED { minusThree, zero, three, ...}

FiveGS-TAC ::= OCTET STRING (SIZE (3))

NRUeReport ::= SEQUENCE {

uENRMeasurements			RRCContainer,

iE-Extensions			ProtocolExtensionContainer { { NRUeReport-ExtIEs} } OPTIONAL,

...

}

NRUeReport-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NRUESecurityCapabilities ::= SEQUENCE {

nRencryptionAlgorithms				NRencryptionAlgorithms,

nRintegrityProtectionAlgorithms		NRintegrityProtectionAlgorithms,

iE-Extensions						ProtocolExtensionContainer { {NRUESecurityCapabilities-ExtIEs} }	OPTIONAL,

...

}

NRUESecurityCapabilities-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

NSSS-NumOccasionDifferentPrecoder ::= ENUMERATED { two, four, eight, ...}

#### -- O

OffsetOfNbiotChannelNumberToEARFCN ::= ENUMERATED {

minusTen,

minusNine,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFour,

minusThree,

minusTwo,

minusOne,

minusZeroDotFive,

zero,

one,

two,

three,

four,

five,

six,

seven,

eight,

nine,

...,

minusEightDotFive,

minusFourDotFive,

threeDotFive,

sevenDotFive

}

Oneframe ::= BIT STRING (SIZE (6))

#### -- P

Packet-LossRate	::= INTEGER(0..1000)

PA-Values ::= ENUMERATED {

dB-6,

dB-4dot77,

dB-3,

dB-1dot77,

dB0,

dB1,

dB2,

dB3,

...

}

PDCPChangeIndication ::= ENUMERATED {s-KgNB-update-required, pDCP-data-recovery-required,...}

PDCP-SN ::= INTEGER (0..4095)

PDCP-SNExtended ::= INTEGER (0..32767)

PDCP-SNlength18 ::= INTEGER (0..262143)

PDCPSnLength	::=	ENUMERATED {twelve-bits,eighteen-bits,...}

PCI ::= INTEGER (0..503, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

Port-Number ::= OCTET STRING (SIZE (2))

PRACH-Configuration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							BOOLEAN,

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)		OPTIONAL, -- present for TDD --

iE-Extensions							ProtocolExtensionContainer { {PRACH-Configuration-ExtIEs} }	OPTIONAL,

...

}

PLMNAreaBasedQMC ::= SEQUENCE {

plmnListforQMC		PLMNListforQMC,

iE-Extensions		ProtocolExtensionContainer { {PLMNAreaBasedQMC-ExtIEs} } OPTIONAL,

...

}

PLMNAreaBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

PLMNListforQMC ::= SEQUENCE (SIZE(1..maxnoofPLMNforQMC)) OF PLMN-Identity

PRACH-Configuration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel				::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

ProSeAuthorized ::= SEQUENCE {

proSeDirectDiscovery		ProSeDirectDiscovery			OPTIONAL,

proSeDirectCommunication	ProSeDirectCommunication		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

ProSeAuthorized-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ProSeUEtoNetworkRelaying	CRITICALITY ignore	EXTENSION ProSeUEtoNetworkRelaying		PRESENCE optional},

...

}

ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProSeUEtoNetworkRelaying ::= ENUMERATED {

authorized,

not-authorized,

...

}

ProtectedEUTRAResourceIndication::= SEQUENCE {

activationSFN							INTEGER (0..1023),

protectedResourceList					ProtectedResourceList,

mBSFNControlRegionLength				INTEGER (0..3) OPTIONAL,

pDCCHRegionLength						INTEGER (1..3) OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {ProtectedEUTRAResourceIndication-ExtIEs} }	OPTIONAL,

...

}

ProtectedEUTRAResourceIndication-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

} -- Rapporteur: missing extension --

ProtectedFootprintTimePattern ::= SEQUENCE {

protectedFootprintTimePeriodicity				INTEGER (1..320, ...),

protectedFootprintStartTime						INTEGER (1..20, ...),

iE-Extensions									ProtocolExtensionContainer { {ProtectedFootprintTimePattern-ExtIEs} }	OPTIONAL,

...

}

ProtectedFootprintTimePattern-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ProtectedResourceList ::= SEQUENCE (SIZE(1.. maxnoofProtectedResourcePatterns)) OF ProtectedResourceList-Item

ProtectedResourceList-Item ::= SEQUENCE {

resourceType					ResourceType,

intraPRBProtectedResourceFootprint 		BIT STRING (SIZE(84, ...)),

protectedFootprintFrequencyPattern 		BIT STRING (SIZE(6..110, ...)),

protectedFootprintTimePattern			ProtectedFootprintTimePattern,

iE-Extensions							ProtocolExtensionContainer { {ProtectedResourceList-Item-ExtIEs} }	OPTIONAL,

...

}

ProtectedResourceList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

PartialListIndicator ::= ENUMERATED {partial, ...}

#### -- Q

QCI ::= INTEGER (0..255)

#### -- R

RadioframeAllocationOffset ::= INTEGER (0..7, ...)

RadioframeAllocationPeriod ::= ENUMERATED{

n1,

n2,

n4,

n8,

n16,

n32,

...

}

RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage							DL-GBR-PRB-usage,

uL-GBR-PRB-usage							UL-GBR-PRB-usage,

dL-non-GBR-PRB-usage						DL-non-GBR-PRB-usage,

uL-non-GBR-PRB-usage						UL-non-GBR-PRB-usage,

dL-Total-PRB-usage							DL-Total-PRB-usage,

uL-Total-PRB-usage							UL-Total-PRB-usage,

iE-Extensions								ProtocolExtensionContainer { {RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

RadioResourceStatus-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-DL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION DL-scheduling-PDCCH-CCE-usage		PRESENCE optional}|

{ID id-UL-scheduling-PDCCH-CCE-usage		CRITICALITY ignore	EXTENSION UL-scheduling-PDCCH-CCE-usage		PRESENCE optional},

...

}

ReceiveStatusofULPDCPSDUs ::= BIT STRING (SIZE(4096))

ReceiveStatusOfULPDCPSDUsExtended ::= BIT STRING (SIZE(1..16384))

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18 ::= BIT STRING (SIZE(1..131072))

Reestablishment-Indication ::= ENUMERATED {

reestablished,

...

}

Registration-Request	::= ENUMERATED {

start,

stop,

...,

partial-stop,

add

}

RelativeNarrowbandTxPower ::= SEQUENCE {

rNTP-PerPRB							BIT STRING (SIZE(6..110, ...)),

rNTP-Threshold						RNTP-Threshold,

numberOfCellSpecificAntennaPorts	ENUMERATED {one, two, four, ...},

p-B									INTEGER (0..3,...),

pDCCH-InterferenceImpact			INTEGER (0..4,...),

iE-Extensions						ProtocolExtensionContainer { {RelativeNarrowbandTxPower-ExtIEs} } OPTIONAL,

...

}

RelativeNarrowbandTxPower-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-enhancedRNTP	CRITICALITY ignore	EXTENSION EnhancedRNTP		PRESENCE optional },

...

}

ReplacingCellsList ::= SEQUENCE (SIZE(0.. maxCellineNB)) OF ReplacingCellsList-Item

ReplacingCellsList-Item ::= SEQUENCE {

eCGI			ECGI,

...

}

ReportAmountMDT ::= ENUMERATED{r1, r2, r4, r8, r16, r32, r64, rinfinity}

ReportArea ::= ENUMERATED{

ecgi,

...

}

ReportCharacteristics	::= BIT STRING (SIZE (32))

ReportingPeriodicityCSIR ::= ENUMERATED {

ms5,

ms10,

ms20,

ms40,

ms80,

...

}

ReportingPeriodicityRSRPMR ::= ENUMERATED {

one-hundred-20-ms,

two-hundred-40-ms,

four-hundred-80-ms,

six-hundred-40-ms,

...

}

ReportIntervalMDT ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true,...}

RequestedFastMCGRecoveryViaSRB3Release ::= ENUMERATED {true,...}

ReservedSubframePattern ::= SEQUENCE{

subframeType					SubframeType,

reservedSubframePattern			BIT STRING (SIZE(10..160)),

mBSFNControlRegionLength		INTEGER (0..3),

iE-Extensions					ProtocolExtensionContainer { {ReservedSubframePattern-ExtIEs} }	OPTIONAL,

...

}

ReservedSubframePattern-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceType ::= ENUMERATED {

downlinknonCRS,

cRS,

uplink,

...

}

ResumeID	::= CHOICE {

non-truncated	BIT STRING(SIZE(40)),

truncated		BIT STRING(SIZE(24)),

...

}

RLCMode ::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions				ProtocolExtensionContainer { {RLC-Status-ExtIEs} } OPTIONAL,

...

}

RLC-Status-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

RNTP-Threshold ::= ENUMERATED {

minusInfinity,

minusEleven,

minusTen,

minusNine,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFour,

minusThree,

minusTwo,

minusOne,

zero,

one,

two,

three,

...

}

RRC-Config-Ind ::= ENUMERATED {

full-config,

delta-config,

...

}

RRC-Context ::= OCTET STRING

RRCConnReestabIndicator ::= ENUMERATED {

reconfigurationFailure, handoverFailure, otherFailure, ...

}

-- The values correspond to the values of ReestablishmentCause reported from the UE in the RRCConnectionReestablishmentRequest, as defined in TS 36.331 [9]

RRCConnSetupIndicator::= ENUMERATED {

rrcConnSetup,

...

}

RSRPMeasurementResult ::= SEQUENCE (SIZE(1..maxCellReport)) OF

SEQUENCE {

rSRPCellID						ECGI,

rSRPMeasured					INTEGER (0..97, ...),

iE-Extensions					ProtocolExtensionContainer { {RSRPMeasurementResult-ExtIEs} } OPTIONAL,

...

}

RSRPMeasurementResult-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

RSRPMRList ::= SEQUENCE (SIZE(1..maxUEReport)) OF

SEQUENCE {

rSRPMeasurementResult			RSRPMeasurementResult,

iE-Extensions					ProtocolExtensionContainer { {RSRPMRList-ExtIEs} } OPTIONAL,

...

}

RSRPMRList-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UEID	CRITICALITY ignore	EXTENSION UEID		PRESENCE optional},

...

}

RRCContainer ::= OCTET STRING

#### -- S

S1TNLLoadIndicator ::= SEQUENCE {

dLS1TNLLoadIndicator			LoadIndicator,

uLS1TNLLoadIndicator			LoadIndicator,

iE-Extensions					ProtocolExtensionContainer { {S1TNLLoadIndicator-ExtIEs} } OPTIONAL,

...

}

S1TNLLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SCGChangeIndication ::= ENUMERATED {pDCPCountWrapAround, pSCellChange, other, ...}

SecondaryRATUsageReportList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container {{SecondaryRATUsageReport-ItemIEs}}

SecondaryRATUsageReport-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-SecondaryRATUsageReport-Item		CRITICALITY reject	TYPE SecondaryRATUsageReport-Item		PRESENCE mandatory},

...

}

SecondaryRATUsageReport-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

secondaryRATType				ENUMERATED {nr, ...},

e-RABUsageReportList			E-RABUsageReportList,

iE-Extensions				ProtocolExtensionContainer { {SecondaryRATUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

SecondaryRATUsageReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SeNBSecurityKey ::= BIT STRING (SIZE(256))

SeNBtoMeNBContainer ::= OCTET STRING

ServedCells ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

servedCellInfo					ServedCell-Information,

neighbour-Info					Neighbour-Information			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCell-ExtIEs} } OPTIONAL,

...

}

ServedCell-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRNeighbourInfoToAdd			CRITICALITY ignore	EXTENSION NRNeighbour-Information				PRESENCE optional },

...

}

ServedCell-Information ::= SEQUENCE {

pCI					PCI,

cellId				ECGI,

tAC					TAC,

broadcastPLMNs		BroadcastPLMNs-Item,

eUTRA-Mode-Info		EUTRA-Mode-Info,

iE-Extensions		ProtocolExtensionContainer { {ServedCell-Information-ExtIEs} } OPTIONAL,

...

}

ServedCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Number-of-Antennaports				CRITICALITY ignore	EXTENSION Number-of-Antennaports					PRESENCE optional}|

{ ID id-PRACH-Configuration					CRITICALITY ignore	EXTENSION PRACH-Configuration						PRESENCE optional}|

{ ID id-MBSFN-Subframe-Info					CRITICALITY ignore	EXTENSION MBSFN-Subframe-Infolist				PRESENCE optional}|

{ ID id-CSG-Id								CRITICALITY ignore	EXTENSION CSG-Id									PRESENCE optional}|

{ ID id-MBMS-Service-Area-List				CRITICALITY ignore	EXTENSION MBMS-Service-Area-Identity-List		PRESENCE optional}|

{ ID id-MultibandInfoList					CRITICALITY ignore	EXTENSION MultibandInfoList							PRESENCE optional}|

{ ID id-FreqBandIndicatorPriority			CRITICALITY ignore	EXTENSION FreqBandIndicatorPriority				PRESENCE optional}|

{ ID id-BandwidthReducedSI					CRITICALITY ignore	EXTENSION BandwidthReducedSI						PRESENCE optional}|

{ ID id-ProtectedEUTRAResourceIndication	CRITICALITY ignore	EXTENSION ProtectedEUTRAResourceIndication	PRESENCE optional}|

{ ID id-BPLMN-ID-Info-EUTRA					CRITICALITY ignore 	EXTENSION BPLMN-ID-Info-EUTRA						PRESENCE optional},

...

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MTSI-service,

...

}

SgNBCoordinationAssistanceInformation ::= ENUMERATED{

coordination-not-required,

...

}

SgNBResourceCoordinationInformation ::= SEQUENCE {

nR-CGI							NRCGI,

uLCoordinationInformation		BIT STRING (SIZE(6..4400, ...)),

dLCoordinationInformation		BIT STRING (SIZE(6..4400, ...))	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {SgNBResourceCoordinationInformationExtIEs} }		 OPTIONAL,

...

}

SgNBResourceCoordinationInformationExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ECGI										CRITICALITY ignore	EXTENSION ECGI												PRESENCE optional}|

{ ID id-SgNBCoordinationAssistanceInformation		CRITICALITY reject	EXTENSION SgNBCoordinationAssistanceInformation	PRESENCE optional},

...

}

SgNB-UE-X2AP-ID ::= INTEGER (0..4294967295)

SIPTOBearerDeactivationIndication ::= ENUMERATED {

true,

...

}

SharedResourceType ::= CHOICE{

uLOnlySharing		ULOnlySharing,

uLandDLSharing		ULandDLSharing,

...

}

ShortMAC-I ::= BIT STRING (SIZE(16))

SGNB-Addition-Trigger-Ind	 ::= ENUMERATED {

sn-change,

inter-eNB-HO,

intra-eNB-HO,

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

SpecialSubframe-Info ::= SEQUENCE {

specialSubframePatterns		SpecialSubframePatterns,

cyclicPrefixDL				CyclicPrefixDL,

cyclicPrefixUL				CyclicPrefixUL,

iE-Extensions				ProtocolExtensionContainer { {SpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

SpecialSubframe-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxCellineNB)

SubbandCQI ::= SEQUENCE {

subbandCQICodeword0			SubbandCQICodeword0,

subbandCQICodeword1			SubbandCQICodeword1		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SubbandCQI-ExtIEs} } OPTIONAL,

...

}

Subscription-Based-UE-DifferentiationInfo ::= SEQUENCE {

periodicCommunicationIndicator	ENUMERATED {periodically, ondemand, ...}		OPTIONAL,

periodicTime					INTEGER (1..3600, ...) 							OPTIONAL,

scheduledCommunicationTime		ScheduledCommunicationTime 						OPTIONAL,

stationaryIndication			ENUMERATED {stationary, mobile, ...}			OPTIONAL,

trafficProfile 					ENUMERATED {single-packet, dual-packets, multiple-packets, ...} 			OPTIONAL,

batteryIndication				ENUMERATED {battery-powered, battery-powered-not-rechargeable-or-replaceable, not-battery-powered, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Subscription-Based-UE-DifferentiationInfo-ExtIEs} } OPTIONAL,

...

}

Subscription-Based-UE-DifferentiationInfo-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ScheduledCommunicationTime ::= SEQUENCE {

dayofWeek				BIT STRING (SIZE(7))									OPTIONAL,

timeofDayStart			INTEGER (0..86399, ...) 								OPTIONAL,

timeofDayEnd			INTEGER (0..86399, ...) 								OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { ScheduledCommunicationTime-ExtIEs}}	OPTIONAL,

...

}

ScheduledCommunicationTime-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SRVCCOperationPossible ::= ENUMERATED {

possible,

...

}

SubbandCQI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SubbandCQICodeword0 ::= CHOICE {

four-bitCQI						INTEGER (0..15, ...),

two-bitSubbandDifferentialCQI	INTEGER (0..3, ...),

two-bitDifferentialCQI			INTEGER (0..3, ...),

...

}

SubbandCQICodeword1 ::= CHOICE {

four-bitCQI							INTEGER (0..15, ...),

three-bitSpatialDifferentialCQI		INTEGER (0..7, ...),

two-bitSubbandDifferentialCQI		INTEGER (0..3, ...),

two-bitDifferentialCQI				INTEGER (0..3, ...),

...

}

SubbandCQIList ::= SEQUENCE (SIZE(1.. maxSubband)) OF SubbandCQIItem

SubbandCQIItem ::= SEQUENCE {

subbandCQI			SubbandCQI,

subbandIndex		INTEGER (0..27,...),

iE-Extensions		ProtocolExtensionContainer { {SubbandCQIItem-ExtIEs} } OPTIONAL,

...

}

SubbandCQIItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SubbandSize ::= ENUMERATED {

size2,

size3,

size4,

size6,

size8,

...

}

SubscriberProfileIDforRFP ::= INTEGER (1..256)

SubframeAllocation ::= CHOICE {

oneframe						Oneframe,

fourframes						Fourframes,

...

}

SubframeAssignment ::= ENUMERATED {

sa0,

sa1,

sa2,

sa3,

sa4,

sa5,

sa6,

...

}

SubframeType ::= ENUMERATED{mbsfn,nonmbsfn,...}

SgNBSecurityKey ::= BIT STRING (SIZE(256))

SgNBtoMeNBContainer ::= OCTET STRING

SRBType ::= ENUMERATED {srb1, srb2, ...}

SCGConfigurationQuery ::= ENUMERATED {true,...}

SULInformation ::= SEQUENCE {

sUL-ARFCN				INTEGER (0.. 3279165),

sUL-TxBW				NR-TxBW,

iE-Extensions			ProtocolExtensionContainer { {SULInformation-ExtIEs} }		OPTIONAL,

...

}

SupportedSULFreqBandItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

iE-Extensions			ProtocolExtensionContainer { {SupportedSULFreqBandItem-ExtIEs} }		OPTIONAL,

...

}

SupportedSULFreqBandItem-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SULInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TABasedMDT::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAC ::= OCTET STRING (SIZE (2))

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAI-Item

TAI-Item ::= SEQUENCE {

tAC					TAC,

pLMN-Identity		PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { {TAI-Item-ExtIEs} } OPTIONAL,

...

}

TAI-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

TABasedQMC ::= SEQUENCE {

tAListforQMC		TAListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,

...

}

TABasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC

TAIBasedQMC ::= SEQUENCE {

tAIListforQMC		TAIListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,

...

}

TAIBasedQMC-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI-Item

TargetCellInUTRAN ::= OCTET STRING -- This IE is to be encoded according to the UTRAN Cell ID in the Last Visited UTRAN Cell Information IE in TS 25.413 [24]

TargeteNBtoSource-eNBTransparentContainer	::= OCTET STRING

TDD-Info ::= SEQUENCE {

eARFCN							EARFCN,

transmission-Bandwidth			Transmission-Bandwidth,

subframeAssignment				SubframeAssignment,

specialSubframe-Info			SpecialSubframe-Info,

iE-Extensions					ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalSpecialSubframe-Info			CRITICALITY ignore	EXTENSION AdditionalSpecialSubframe-Info				PRESENCE optional}|

{ ID id-eARFCNExtension							CRITICALITY reject	EXTENSION EARFCNExtension									PRESENCE optional}|

{ ID id-AdditionalSpecialSubframeExtension-Info	CRITICALITY ignore	EXTENSION AdditionalSpecialSubframeExtension-Info	PRESENCE optional}|

{ ID id-OffsetOfNbiotChannelNumberToDL-EARFCN	CRITICALITY reject	EXTENSION OffsetOfNbiotChannelNumberToEARFCN		PRESENCE optional}|

{ ID id-NBIoT-UL-DL-AlignmentOffset				CRITICALITY reject	EXTENSION NBIoT-UL-DL-AlignmentOffset					PRESENCE optional},

...

}

TDD-InfoNeighbourServedNRCell-Information ::= SEQUENCE {

nRFreqInfo				NRFreqInfo,

iE-Extensions			ProtocolExtensionContainer { {TDD-InfoNeighbourServedNRCell-Information-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoNeighbourServedNRCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Threshold-RSRP ::= INTEGER(0..97)

Threshold-RSRQ ::= INTEGER(0..34)

TimeToWait ::= ENUMERATED {

v1s,

v2s,

v5s,

v10s,

v20s,

v60s,

...

}

Time-UE-StayedInCell ::= INTEGER (0..4095)

Time-UE-StayedInCell-EnhancedGranularity ::= INTEGER (0..40950)

Transport-UP-Layer-Addresses-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Add-Item

Transport-UP-Layer-Addresses-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Remove-Item

Transport-UP-Layer-Addresses-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TNLConfigurationInfo  ::= SEQUENCE {

transport-UP-Layer-Addresses-Info-To-Add-List		Transport-UP-Layer-Addresses-Info-To-Add-List						OPTIONAL,

transport-UP-Layer-Addresses-Info-To-Remove-List	Transport-UP-Layer-Addresses-Info-To-Remove-List					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { TNLConfigurationInfo-ExtIEs } }											OPTIONAL,

...

}

TNLConfigurationInfo-ExtIEs 	X2AP-PROTOCOL-EXTENSION ::= {

...

}

TraceActivation ::= SEQUENCE {

eUTRANTraceID					EUTRANTraceID,

interfacesToTrace				InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TraceCollectionEntityIPAddress,

iE-Extensions					ProtocolExtensionContainer { {TraceActivation-ExtIEs} } OPTIONAL,

...

}

TraceActivation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-MDTConfiguration			CRITICALITY ignore	EXTENSION MDT-Configuration			PRESENCE optional}|

{ ID id-UEAppLayerMeasConfig		CRITICALITY ignore	EXTENSION UEAppLayerMeasConfig		PRESENCE optional},

...

}

TraceCollectionEntityIPAddress ::= BIT STRING (SIZE(1..160, ...))

TraceDepth 		::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

Transmission-Bandwidth ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...,

bw1

}

TransportLayerAddress			::= BIT STRING (SIZE(1..160, ...))

TunnelInformation ::= SEQUENCE {

transportLayerAddress	TransportLayerAddress,

uDP-Port-Number			Port-Number			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {Tunnel-Information-ExtIEs} } OPTIONAL,

...

}

Tunnel-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEaggregateMaximumBitRateDownlink	BitRate,

uEaggregateMaximumBitRateUplink		BitRate,

iE-Extensions						ProtocolExtensionContainer { {UEAggregate-MaximumBitrate-ExtIEs} } OPTIONAL,

...

}

UEAggregate-MaximumBitrate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-extended-uEaggregateMaximumBitRateDownlink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-uEaggregateMaximumBitRateUplink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

UEAppLayerMeasConfig ::= SEQUENCE {

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..1000)),

areaScopeOfQMC		AreaScopeOfQMC,

iE-Extensions		ProtocolExtensionContainer { {UEAppLayerMeasConfig-ExtIEs} } OPTIONAL,

...

}

UEAppLayerMeasConfig-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-serviceType	CRITICALITY ignore	EXTENSION ServiceType	PRESENCE optional},

...

}

UE-ContextKeptIndicator ::= ENUMERATED {

true,

...

}

UEID ::= BIT STRING (SIZE (16))

UE-HistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCells)) OF LastVisitedCell-Item

UE-HistoryInformationFromTheUE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the VisitedCellInfoList field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UE-S1AP-ID ::= INTEGER (0.. 4294967295)

UE-X2AP-ID ::= INTEGER (0..4095)

UE-X2AP-ID-Extension ::= INTEGER (0..4095, ...)

UE-RLF-Report-Container::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UE-RLF-Report-Container-for-extended-bands ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-v9e0 field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UESecurityCapabilities ::= SEQUENCE {

encryptionAlgorithms				EncryptionAlgorithms,

integrityProtectionAlgorithms		IntegrityProtectionAlgorithms,

iE-Extensions						ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} }		OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UESidelinkAggregateMaximumBitRate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {UE-Sidelink-Aggregate-MaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UE-Sidelink-Aggregate-MaximumBitRate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UEsToBeResetList ::= SEQUENCE (SIZE (1.. maxUEsinengNBDU)) OF UEsToBeResetList-Item

UEsToBeResetList-Item::= SEQUENCE {

meNB-ID					UE-X2AP-ID,

meNB-ID-ext				UE-X2AP-ID-Extension														OPTIONAL,

sgNB-ID					SgNB-UE-X2AP-ID																OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {UEsToBeResetList-Item-ExtIEs} }				OPTIONAL,

...

}

UEsToBeResetList-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ULandDLSharing ::= SEQUENCE{

uLResourcesULandDLSharing				ULResourcesULandDLSharing,

dLResourcesULandDLSharing				DLResourcesULandDLSharing,

iE-Extensions			ProtocolExtensionContainer { {ULandDLSharing-ExtIEs} }				OPTIONAL,

...

}

ULandDLSharing-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ULConfiguration::= SEQUENCE {

uL-PDCP			UL-UE-Configuration,

iE-Extensions					ProtocolExtensionContainer { {ULConfiguration-ExtIEs} } OPTIONAL,

...

}

ULConfiguration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UL-UE-Configuration::= ENUMERATED { no-data, shared, only, ... }

UL-GBR-PRB-usage::= INTEGER (0..100)

UL-HighInterferenceIndicationInfo ::= SEQUENCE (SIZE(1..maxCellineNB)) OF UL-HighInterferenceIndicationInfo-Item

UL-HighInterferenceIndicationInfo-Item ::= SEQUENCE {

target-Cell-ID					ECGI,

ul-interferenceindication		UL-HighInterferenceIndication,

iE-Extensions					ProtocolExtensionContainer { {UL-HighInterferenceIndicationInfo-Item-ExtIEs} } OPTIONAL,

...

}

UL-HighInterferenceIndicationInfo-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UL-HighInterferenceIndication ::= BIT STRING (SIZE(1..110, ...))

UL-InterferenceOverloadIndication ::= SEQUENCE (SIZE(1..maxnoofPRBs)) OF UL-InterferenceOverloadIndication-Item

UL-InterferenceOverloadIndication-Item ::= ENUMERATED {

high-interference,

medium-interference,

low-interference,

...

}

UL-non-GBR-PRB-usage::= INTEGER (0..100)

ULOnlySharing ::= SEQUENCE{

uLResourceBitmapULOnlySharing	DataTrafficResources,

iE-Extensions					ProtocolExtensionContainer { {ULOnlySharing-ExtIEs} }				OPTIONAL,

...

}

ULOnlySharing-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ULResourceBitmapULandDLSharing ::= DataTrafficResources

ULResourcesULandDLSharing ::= CHOICE {

unchanged			NULL,

changed				ULResourceBitmapULandDLSharing,

...

}

UL-scheduling-PDCCH-CCE-usage::= INTEGER (0..100)

UL-Total-PRB-usage::= INTEGER (0..100)

UsableABSInformation ::= CHOICE {

fdd					UsableABSInformationFDD,

tdd					UsableABSInformationTDD,

...

}

UsableABSInformationFDD ::= SEQUENCE {

usable-abs-pattern-info				BIT STRING (SIZE(40)),

iE-Extensions						ProtocolExtensionContainer { {UsableABSInformationFDD-ExtIEs} } OPTIONAL,

...

}

UsableABSInformationFDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UsableABSInformationTDD ::= SEQUENCE {

usaable-abs-pattern-info			BIT STRING (SIZE(1..70, ...)),

iE-Extensions						ProtocolExtensionContainer { {UsableABSInformationTDD-ExtIEs} } OPTIONAL,

...

}

UsableABSInformationTDD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

UserPlaneTrafficActivityReport ::= ENUMERATED {inactive, re-activated, ...}

#### -- V

V2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 			PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {V2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

V2XServicesAuthorized-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

#### -- W

WidebandCQI ::= SEQUENCE {

widebandCQICodeword0		INTEGER (0..15, ...),

widebandCQICodeword1		WidebandCQICodeword1		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {WidebandCQI-ExtIEs} } OPTIONAL,

...

}

WidebandCQI-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

WidebandCQICodeword1::= CHOICE {

four-bitCQI								INTEGER (0..15, ...),

three-bitSpatialDifferentialCQI			INTEGER (0..7, ...),

...

}

WLANMeasurementConfiguration ::= SEQUENCE {

wlanMeasConfig				WLANMeasConfig,

wlanMeasConfigNameList		WLANMeasConfigNameList				OPTIONAL,

wlan-rssi					ENUMERATED {true, ...}				OPTIONAL,

wlan-rtt					ENUMERATED {true, ...}				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {WLANMeasurementConfiguration-ExtIEs} } OPTIONAL,

...

}

WLANMeasurementConfiguration-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANName

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))

WTID ::= CHOICE {

wTID-Type1			WTID-Type1,

wTID-Type2			WTID-Long-Type2,

...

}

WTID-Type1 ::= SEQUENCE {

pLMN-Identity					PLMN-Identity,

shortWTID						BIT STRING (SIZE(24)),

...

}

WTID-Long-Type2 ::= BIT STRING (SIZE(48))

WT-UE-XwAP-ID ::= OCTET STRING (SIZE (3))

#### -- X

X2BenefitValue ::= INTEGER (1..8, ...)

#### -- Y

#### -- Z

END

-- ASN1STOP

*** Next change, skipped text not changed ***

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-handoverCancel												ProcedureCode ::= 1

id-loadIndication												ProcedureCode ::= 2

id-errorIndication												ProcedureCode ::= 3

id-snStatusTransfer												ProcedureCode ::= 4

id-uEContextRelease												ProcedureCode ::= 5

id-x2Setup														ProcedureCode ::= 6

id-reset														ProcedureCode ::= 7

id-eNBConfigurationUpdate										ProcedureCode ::= 8

id-resourceStatusReportingInitiation							ProcedureCode ::= 9

id-resourceStatusReporting										ProcedureCode ::= 10

id-privateMessage												ProcedureCode ::= 11

id-mobilitySettingsChange										ProcedureCode ::= 12

id-rLFIndication												ProcedureCode ::= 13

id-handoverReport												ProcedureCode ::= 14

id-cellActivation												ProcedureCode ::= 15

id-x2Release													ProcedureCode ::= 16

id-x2APMessageTransfer											ProcedureCode ::= 17

id-x2Removal													ProcedureCode ::= 18

id-seNBAdditionPreparation										ProcedureCode ::= 19

id-seNBReconfigurationCompletion								ProcedureCode ::= 20

id-meNBinitiatedSeNBModificationPreparation						ProcedureCode ::= 21

id-seNBinitiatedSeNBModification								ProcedureCode ::= 22

id-meNBinitiatedSeNBRelease										ProcedureCode ::= 23

id-seNBinitiatedSeNBRelease										ProcedureCode ::= 24

id-seNBCounterCheck												ProcedureCode ::= 25

id-retrieveUEContext											ProcedureCode ::= 26

id-sgNBAdditionPreparation										ProcedureCode ::= 27

id-sgNBReconfigurationCompletion									ProcedureCode ::= 28

id-meNBinitiatedSgNBModificationPreparation						ProcedureCode ::= 29

id-sgNBinitiatedSgNBModification									ProcedureCode ::= 30

id-meNBinitiatedSgNBRelease										ProcedureCode ::= 31

id-sgNBinitiatedSgNBRelease										ProcedureCode ::= 32

id-sgNBCounterCheck												ProcedureCode ::= 33

id-sgNBChange													ProcedureCode ::= 34

id-rRCTransfer													ProcedureCode ::= 35

id-endcX2Setup													ProcedureCode ::= 36

id-endcConfigurationUpdate										ProcedureCode ::= 37

id-secondaryRATDataUsageReport									ProcedureCode ::= 38

id-endcCellActivation											ProcedureCode ::= 39

id-endcPartialReset												ProcedureCode ::= 40

id-eUTRANRCellResourceCoordination								ProcedureCode ::= 41

id-SgNBActivityNotification										ProcedureCode ::= 42

id-endcX2Removal												ProcedureCode ::= 43

id-dataForwardingAddressIndication								ProcedureCode ::= 44

id-gNBStatusIndication											ProcedureCode ::= 45

id-deactivateTrace												ProcedureCode ::= 46

id-traceStart													ProcedureCode ::= 47

id-endcConfigurationTransfer										ProcedureCode ::= 48

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 65535

maxEARFCNPlusOne							INTEGER ::= 65536

newmaxEARFCN								INTEGER ::= 262143

maxInterfaces								INTEGER ::= 16

maxCellineNB								INTEGER ::= 256

maxnoofBands								INTEGER ::= 16

maxnoofBearers								INTEGER ::= 256

maxNrOfErrors								INTEGER ::= 256

maxnoofPDCP-SN								INTEGER ::= 16

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofForbLACs								INTEGER ::= 4096

maxnoofForbTACs								INTEGER ::= 4096

maxnoofBPLMNs								INTEGER ::= 6

maxnoofAdditionalPLMNs						INTEGER ::= 6

maxnoofNeighbours							INTEGER ::= 512

maxnoofPRBs									INTEGER ::= 110

maxPools									INTEGER ::= 16

maxnoofCells								INTEGER ::= 16

maxnoofMBSFN								INTEGER ::= 8

maxFailedMeasObjects						INTEGER ::= 32

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofTAforMDT								INTEGER ::= 8

maxnoofMBMSServiceAreaIdentities			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofCoMPHypothesisSet					INTEGER ::= 256

maxnoofCoMPCells							INTEGER ::= 32

maxUEReport									INTEGER ::= 128

maxCellReport								INTEGER ::= 9

maxnoofPA									INTEGER ::= 3

maxCSIProcess								INTEGER ::= 4

maxCSIReport								INTEGER ::= 2

maxSubband									INTEGER ::= 14

maxofNRNeighbours							INTEGER ::= 1024

maxCellinengNB								INTEGER ::= 16384

--	maxnoofNRCarriers						INTEGER ::= 32

maxnooftimeperiods							INTEGER ::= 2

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofTAforQMC								INTEGER ::= 8

maxnoofPLMNforQMC							INTEGER ::= 16

maxUEsinengNBDU								INTEGER ::= 8192

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoNRcellsSpectrumSharingWithE-UTRA		INTEGER ::= 64

maxnoofNrCellBands							INTEGER ::= 32

maxnoofBluetoothName						INTEGER ::= 4

maxnoofWLANName								INTEGER ::= 4

maxnoofextBPLMNs							INTEGER ::= 12

maxnoofextBPLMNsminus1						INTEGER ::= 11

maxnoofBPLMNsminus1							INTEGER ::= 11

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

id-CellInformation															ProtocolIE-ID ::= 6

id-CellInformation-Item														ProtocolIE-ID ::= 7

id-New-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 9

id-Old-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 10

id-TargetCell-ID															ProtocolIE-ID ::= 11

id-TargeteNBtoSource-eNBTransparentContainer								ProtocolIE-ID ::= 12

id-TraceActivation															ProtocolIE-ID ::= 13

id-UE-ContextInformation													ProtocolIE-ID ::= 14

id-UE-HistoryInformation													ProtocolIE-ID ::= 15

id-UE-X2AP-ID																ProtocolIE-ID ::= 16

id-CriticalityDiagnostics													ProtocolIE-ID ::= 17

id-E-RABs-SubjectToStatusTransfer-List										ProtocolIE-ID ::= 18

id-E-RABs-SubjectToStatusTransfer-Item										ProtocolIE-ID ::= 19

id-ServedCells																ProtocolIE-ID ::= 20

id-GlobalENB-ID																ProtocolIE-ID ::= 21

id-TimeToWait																ProtocolIE-ID ::= 22

id-GUMMEI-ID																ProtocolIE-ID ::= 23

id-GUGroupIDList															ProtocolIE-ID ::= 24

id-ServedCellsToAdd															ProtocolIE-ID ::= 25

id-ServedCellsToModify														ProtocolIE-ID ::= 26

id-ServedCellsToDelete														ProtocolIE-ID ::= 27

id-Registration-Request														ProtocolIE-ID ::= 28

id-CellToReport																ProtocolIE-ID ::= 29

id-ReportingPeriodicity														ProtocolIE-ID ::= 30

id-CellToReport-Item														ProtocolIE-ID ::= 31

id-CellMeasurementResult													ProtocolIE-ID ::= 32

id-CellMeasurementResult-Item												ProtocolIE-ID ::= 33

id-GUGroupIDToAddList														ProtocolIE-ID ::= 34

id-GUGroupIDToDeleteList													ProtocolIE-ID ::= 35

id-SRVCCOperationPossible													ProtocolIE-ID ::= 36

id-Measurement-ID															ProtocolIE-ID ::= 37

id-ReportCharacteristics													ProtocolIE-ID ::= 38

id-ENB1-Measurement-ID														ProtocolIE-ID ::= 39

id-ENB2-Measurement-ID														ProtocolIE-ID ::= 40

id-Number-of-Antennaports													ProtocolIE-ID ::= 41

id-CompositeAvailableCapacityGroup											ProtocolIE-ID ::= 42

id-ENB1-Cell-ID																ProtocolIE-ID ::= 43

id-ENB2-Cell-ID																ProtocolIE-ID ::= 44

id-ENB2-Proposed-Mobility-Parameters										ProtocolIE-ID ::= 45

id-ENB1-Mobility-Parameters													ProtocolIE-ID ::= 46

id-ENB2-Mobility-Parameters-Modification-Range								ProtocolIE-ID ::= 47

id-FailureCellPCI															ProtocolIE-ID ::= 48

id-Re-establishmentCellECGI													ProtocolIE-ID ::= 49

id-FailureCellCRNTI															ProtocolIE-ID ::= 50

id-ShortMAC-I																ProtocolIE-ID ::= 51

id-SourceCellECGI															ProtocolIE-ID ::= 52

id-FailureCellECGI															ProtocolIE-ID ::= 53

id-HandoverReportType														ProtocolIE-ID ::= 54

id-PRACH-Configuration														ProtocolIE-ID ::= 55

id-MBSFN-Subframe-Info														ProtocolIE-ID ::= 56

id-ServedCellsToActivate													ProtocolIE-ID ::= 57

id-ActivatedCellList														ProtocolIE-ID ::= 58

id-DeactivationIndication													ProtocolIE-ID ::= 59

id-UE-RLF-Report-Container													ProtocolIE-ID ::= 60

id-ABSInformation															ProtocolIE-ID ::= 61

id-InvokeIndication															ProtocolIE-ID ::= 62

id-ABS-Status																ProtocolIE-ID ::= 63

id-PartialSuccessIndicator													ProtocolIE-ID ::= 64

id-MeasurementInitiationResult-List											ProtocolIE-ID ::= 65

id-MeasurementInitiationResult-Item											ProtocolIE-ID ::= 66

id-MeasurementFailureCause-Item												ProtocolIE-ID ::= 67

id-CompleteFailureCauseInformation-List										ProtocolIE-ID ::= 68

id-CompleteFailureCauseInformation-Item										ProtocolIE-ID ::= 69

id-CSG-Id																	ProtocolIE-ID ::= 70

id-CSGMembershipStatus														ProtocolIE-ID ::= 71

id-MDTConfiguration															ProtocolIE-ID ::= 72

id-ManagementBasedMDTallowed												ProtocolIE-ID ::= 74

id-RRCConnSetupIndicator													ProtocolIE-ID ::= 75

id-NeighbourTAC																ProtocolIE-ID ::= 76

id-Time-UE-StayedInCell-EnhancedGranularity									ProtocolIE-ID ::= 77

id-RRCConnReestabIndicator													ProtocolIE-ID ::= 78

id-MBMS-Service-Area-List													ProtocolIE-ID ::= 79

id-HO-cause																	ProtocolIE-ID ::= 80

id-TargetCellInUTRAN														ProtocolIE-ID ::= 81

id-MobilityInformation														ProtocolIE-ID ::= 82

id-SourceCellCRNTI															ProtocolIE-ID ::= 83

id-MultibandInfoList														ProtocolIE-ID ::= 84

id-M3Configuration															ProtocolIE-ID ::= 85

id-M4Configuration															ProtocolIE-ID ::= 86

id-M5Configuration															ProtocolIE-ID ::= 87

id-MDT-Location-Info														ProtocolIE-ID ::= 88

id-ManagementBasedMDTPLMNList												ProtocolIE-ID ::= 89

id-SignallingBasedMDTPLMNList												ProtocolIE-ID ::= 90

id-ReceiveStatusOfULPDCPSDUsExtended										ProtocolIE-ID ::= 91

id-ULCOUNTValueExtended														ProtocolIE-ID ::= 92

id-DLCOUNTValueExtended														ProtocolIE-ID ::= 93

id-eARFCNExtension															ProtocolIE-ID ::= 94

id-UL-EARFCNExtension														ProtocolIE-ID ::= 95

id-DL-EARFCNExtension														ProtocolIE-ID ::= 96

id-AdditionalSpecialSubframe-Info											ProtocolIE-ID ::= 97

id-Masked-IMEISV															ProtocolIE-ID ::= 98

id-IntendedULDLConfiguration												ProtocolIE-ID ::= 99

id-ExtendedULInterferenceOverloadInfo										ProtocolIE-ID ::= 100

id-RNL-Header																ProtocolIE-ID ::= 101

id-x2APMessage																ProtocolIE-ID ::= 102

id-ProSeAuthorized															ProtocolIE-ID ::= 103

id-ExpectedUEBehaviour														ProtocolIE-ID ::= 104

id-UE-HistoryInformationFromTheUE											ProtocolIE-ID ::= 105

id-DynamicDLTransmissionInformation											ProtocolIE-ID ::= 106

id-UE-RLF-Report-Container-for-extended-bands								ProtocolIE-ID ::= 107

id-CoMPInformation															ProtocolIE-ID ::= 108

id-ReportingPeriodicityRSRPMR												ProtocolIE-ID ::= 109

id-RSRPMRList																ProtocolIE-ID ::= 110

id-MeNB-UE-X2AP-ID															ProtocolIE-ID ::= 111

id-SeNB-UE-X2AP-ID															ProtocolIE-ID ::= 112

id-UE-SecurityCapabilities													ProtocolIE-ID ::= 113

id-SeNBSecurityKey															ProtocolIE-ID ::= 114

id-SeNBUEAggregateMaximumBitRate											ProtocolIE-ID ::= 115

id-ServingPLMN																ProtocolIE-ID ::= 116

id-E-RABs-ToBeAdded-List													ProtocolIE-ID ::= 117

id-E-RABs-ToBeAdded-Item													ProtocolIE-ID ::= 118

id-MeNBtoSeNBContainer														ProtocolIE-ID ::= 119

id-E-RABs-Admitted-ToBeAdded-List											ProtocolIE-ID ::= 120

id-E-RABs-Admitted-ToBeAdded-Item											ProtocolIE-ID ::= 121

id-SeNBtoMeNBContainer														ProtocolIE-ID ::= 122

id-ResponseInformationSeNBReconfComp										ProtocolIE-ID ::= 123

id-UE-ContextInformationSeNBModReq											ProtocolIE-ID ::= 124

id-E-RABs-ToBeAdded-ModReqItem												ProtocolIE-ID ::= 125

id-E-RABs-ToBeModified-ModReqItem											ProtocolIE-ID ::= 126

id-E-RABs-ToBeReleased-ModReqItem											ProtocolIE-ID ::= 127

id-E-RABs-Admitted-ToBeAdded-ModAckList										ProtocolIE-ID ::= 128

id-E-RABs-Admitted-ToBeModified-ModAckList									ProtocolIE-ID ::= 129

id-E-RABs-Admitted-ToBeReleased-ModAckList									ProtocolIE-ID ::= 130

id-E-RABs-Admitted-ToBeAdded-ModAckItem										ProtocolIE-ID ::= 131

id-E-RABs-Admitted-ToBeModified-ModAckItem									ProtocolIE-ID ::= 132

id-E-RABs-Admitted-ToBeReleased-ModAckItem									ProtocolIE-ID ::= 133

id-E-RABs-ToBeReleased-ModReqd												ProtocolIE-ID ::= 134

id-E-RABs-ToBeReleased-ModReqdItem											ProtocolIE-ID ::= 135

id-SCGChangeIndication														ProtocolIE-ID ::= 136

id-E-RABs-ToBeReleased-List-RelReq											ProtocolIE-ID ::= 137

id-E-RABs-ToBeReleased-RelReqItem											ProtocolIE-ID ::= 138

id-E-RABs-ToBeReleased-List-RelConf											ProtocolIE-ID ::= 139

id-E-RABs-ToBeReleased-RelConfItem											ProtocolIE-ID ::= 140

id-E-RABs-SubjectToCounterCheck-List										ProtocolIE-ID ::= 141

id-E-RABs-SubjectToCounterCheckItem											ProtocolIE-ID ::= 142

id-CoverageModificationList													ProtocolIE-ID ::= 143

id-ReportingPeriodicityCSIR													ProtocolIE-ID ::= 145

id-CSIReportList															ProtocolIE-ID ::= 146

id-UEID																		ProtocolIE-ID ::= 147

id-enhancedRNTP																ProtocolIE-ID ::= 148

id-ProSeUEtoNetworkRelaying													ProtocolIE-ID ::= 149

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18									ProtocolIE-ID ::= 150

id-ULCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 151

id-DLCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 152

id-UE-ContextReferenceAtSeNB												ProtocolIE-ID ::= 153

id-UE-ContextKeptIndicator													ProtocolIE-ID ::= 154

id-New-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 155

id-Old-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 156

id-MeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 157

id-SeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 158

id-LHN-ID																	ProtocolIE-ID ::= 159

id-FreqBandIndicatorPriority												ProtocolIE-ID ::= 160

id-M6Configuration															ProtocolIE-ID ::= 161

id-M7Configuration															ProtocolIE-ID ::= 162

id-Tunnel-Information-for-BBF												ProtocolIE-ID ::= 163

id-SIPTO-BearerDeactivationIndication										ProtocolIE-ID ::= 164

id-GW-TransportLayerAddress													ProtocolIE-ID ::= 165

id-Correlation-ID															ProtocolIE-ID ::= 166

id-SIPTO-Correlation-ID														ProtocolIE-ID ::= 167

id-SIPTO-L-GW-TransportLayerAddress											ProtocolIE-ID ::= 168

id-X2RemovalThreshold														ProtocolIE-ID ::= 169

id-CellReportingIndicator													ProtocolIE-ID ::= 170

id-BearerType																ProtocolIE-ID ::= 171

id-resumeID																	ProtocolIE-ID ::= 172

id-UE-ContextInformationRetrieve											ProtocolIE-ID ::= 173

id-E-RABs-ToBeSetupRetrieve-Item											ProtocolIE-ID ::= 174

id-NewEUTRANCellIdentifier													ProtocolIE-ID ::= 175

id-V2XServicesAuthorized													ProtocolIE-ID ::= 176

id-OffsetOfNbiotChannelNumberToDL-EARFCN									ProtocolIE-ID ::= 177

id-OffsetOfNbiotChannelNumberToUL-EARFCN									ProtocolIE-ID ::= 178

id-AdditionalSpecialSubframeExtension-Info									ProtocolIE-ID ::= 179

id-BandwidthReducedSI														ProtocolIE-ID ::= 180

id-MakeBeforeBreakIndicator													ProtocolIE-ID ::= 181

id-UE-ContextReferenceAtWT													ProtocolIE-ID ::= 182

id-WT-UE-ContextKeptIndicator												ProtocolIE-ID ::= 183

id-UESidelinkAggregateMaximumBitRate										ProtocolIE-ID ::= 184

id-uL-GTPtunnelEndpoint														ProtocolIE-ID ::= 185

id-DL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 193

id-UL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 194

id-UEAppLayerMeasConfig														ProtocolIE-ID ::= 195

id-extended-e-RAB-MaximumBitrateDL											ProtocolIE-ID ::= 196

id-extended-e-RAB-MaximumBitrateUL											ProtocolIE-ID ::= 197

id-extended-e-RAB-GuaranteedBitrateDL										ProtocolIE-ID ::= 198

id-extended-e-RAB-GuaranteedBitrateUL										ProtocolIE-ID ::= 199

id-extended-uEaggregateMaximumBitRateDownlink								ProtocolIE-ID ::= 200

id-extended-uEaggregateMaximumBitRateUplink									ProtocolIE-ID ::= 201

id-NRrestrictioninEPSasSecondaryRAT											ProtocolIE-ID ::= 202

id-SgNBSecurityKey															ProtocolIE-ID ::= 203

id-SgNBUEAggregateMaximumBitRate												ProtocolIE-ID ::= 204

id-E-RABs-ToBeAdded-SgNBAddReqList											ProtocolIE-ID ::= 205

id-MeNBtoSgNBContainer														ProtocolIE-ID ::= 206

id-SgNB-UE-X2AP-ID															ProtocolIE-ID ::= 207

id-RequestedSplitSRBs														ProtocolIE-ID ::= 208

id-E-RABs-ToBeAdded-SgNBAddReq-Item											ProtocolIE-ID ::= 209

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList								ProtocolIE-ID ::= 210

id-SgNBtoMeNBContainer														ProtocolIE-ID ::= 211

id-AdmittedSplitSRBs															ProtocolIE-ID ::= 212

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item								ProtocolIE-ID ::= 213

id-ResponseInformationSgNBReconfComp											ProtocolIE-ID ::= 214

id-UE-ContextInformation-SgNBModReq											ProtocolIE-ID ::= 215

id-E-RABs-ToBeAdded-SgNBModReq-Item											ProtocolIE-ID ::= 216

id-E-RABs-ToBeModified-SgNBModReq-Item										ProtocolIE-ID ::= 217

id-E-RABs-ToBeReleased-SgNBModReq-Item										ProtocolIE-ID ::= 218

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList									ProtocolIE-ID ::= 219

id-E-RABs-Admitted-ToBeModified-SgNBModAckList								ProtocolIE-ID ::= 220

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList								ProtocolIE-ID ::= 221

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item									ProtocolIE-ID ::= 222

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item								ProtocolIE-ID ::= 223

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item								ProtocolIE-ID ::= 224

id-E-RABs-ToBeReleased-SgNBModReqdList										ProtocolIE-ID ::= 225

id-E-RABs-ToBeModified-SgNBModReqdList										ProtocolIE-ID ::= 226

id-E-RABs-ToBeReleased-SgNBModReqd-Item										ProtocolIE-ID ::= 227

id-E-RABs-ToBeModified-SgNBModReqd-Item										ProtocolIE-ID ::= 228

id-E-RABs-ToBeReleased-SgNBChaConfList										ProtocolIE-ID ::= 229

id-E-RABs-ToBeReleased-SgNBChaConf-Item										ProtocolIE-ID ::= 230

id-E-RABs-ToBeReleased-SgNBRelReqList										ProtocolIE-ID ::= 231

id-E-RABs-ToBeReleased-SgNBRelReq-Item										ProtocolIE-ID ::= 232

id-E-RABs-ToBeReleased-SgNBRelConfList										ProtocolIE-ID ::= 233

id-E-RABs-ToBeReleased-SgNBRelConf-Item										ProtocolIE-ID ::= 234

id-E-RABs-SubjectToSgNBCounterCheck-List										ProtocolIE-ID ::= 235

id-E-RABs-SubjectToSgNBCounterCheck-Item										ProtocolIE-ID ::= 236

id-RRCContainer																ProtocolIE-ID ::= 237

id-SRBType																	ProtocolIE-ID ::= 238

id-Target-SgNB-ID															ProtocolIE-ID ::= 239

id-HandoverRestrictionList													ProtocolIE-ID ::= 240

id-SCGConfigurationQuery														ProtocolIE-ID ::= 241

id-SplitSRB																	ProtocolIE-ID ::= 242

id-NRUeReport																ProtocolIE-ID ::= 243

id-InitiatingNodeType-EndcX2Setup											ProtocolIE-ID ::= 244

id-InitiatingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 245

id-RespondingNodeType-EndcX2Setup											ProtocolIE-ID ::= 246

id-RespondingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 247

id-NRUESecurityCapabilities													ProtocolIE-ID ::= 248

id-PDCPChangeIndication														ProtocolIE-ID ::= 249

id-ServedEUTRAcellsENDCX2ManagementList										ProtocolIE-ID ::= 250

id-CellAssistanceInformation													ProtocolIE-ID ::= 251

id-Globalen-gNB-ID															ProtocolIE-ID ::= 252

id-ServedNRcellsENDCX2ManagementList											ProtocolIE-ID ::= 253

id-UE-ContextReferenceAtSgNB													ProtocolIE-ID ::= 254

id-SecondaryRATUsageReport													ProtocolIE-ID ::= 255

id-ActivationID																ProtocolIE-ID ::= 256

id-MeNBResourceCoordinationInformation										ProtocolIE-ID ::= 257

id-SgNBResourceCoordinationInformation										ProtocolIE-ID ::= 258

id-ServedEUTRAcellsToModifyListENDCConfUpd									ProtocolIE-ID ::= 259

id-ServedEUTRAcellsToDeleteListENDCConfUpd									ProtocolIE-ID ::= 260

id-ServedNRcellsToModifyListENDCConfUpd										ProtocolIE-ID ::= 261

id-ServedNRcellsToDeleteListENDCConfUpd										ProtocolIE-ID ::= 262

id-E-RABUsageReport-Item														ProtocolIE-ID ::= 263

id-Old-SgNB-UE-X2AP-ID														ProtocolIE-ID ::= 264

id-SecondaryRATUsageReportList												ProtocolIE-ID ::= 265

id-SecondaryRATUsageReport-Item												ProtocolIE-ID ::= 266

id-ServedNRCellsToActivate													ProtocolIE-ID ::= 267

id-ActivatedNRCellList														ProtocolIE-ID ::= 268

id-SelectedPLMN																ProtocolIE-ID ::= 269

id-UEs-ToBeReset															ProtocolIE-ID ::= 270

id-UEs-Admitted-ToBeReset													ProtocolIE-ID ::= 271

id-RRCConfigIndication														ProtocolIE-ID ::= 272

id-DownlinkPacketLossRate													ProtocolIE-ID ::= 273

id-UplinkPacketLossRate														ProtocolIE-ID ::= 274

id-SubscriberProfileIDforRFP												ProtocolIE-ID ::= 275

id-serviceType																ProtocolIE-ID ::= 276

id-AerialUEsubscriptionInformation											ProtocolIE-ID ::= 277

id-SGNB-Addition-Trigger-Ind												ProtocolIE-ID ::= 278

id-MeNBCell-ID																ProtocolIE-ID ::= 279

id-RequestedSplitSRBsrelease												ProtocolIE-ID ::= 280

id-AdmittedSplitSRBsrelease													ProtocolIE-ID ::= 281

id-NRS-NSSS-PowerOffset														ProtocolIE-ID ::= 282

id-NSSS-NumOccasionDifferentPrecoder										ProtocolIE-ID ::= 283

id-ProtectedEUTRAResourceIndication											ProtocolIE-ID ::= 284

id-InitiatingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 285

id-RespondingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 286

id-DataTrafficResourceIndication											ProtocolIE-ID ::= 287

id-SpectrumSharingGroupID													ProtocolIE-ID ::= 288

id-ListofEUTRACellsinEUTRACoordinationReq									ProtocolIE-ID ::= 289

id-ListofEUTRACellsinEUTRACoordinationResp									ProtocolIE-ID ::= 290

id-ListofEUTRACellsinNRCoordinationReq										ProtocolIE-ID ::= 291

id-ListofNRCellsinNRCoordinationReq											ProtocolIE-ID ::= 292

id-ListofNRCellsinNRCoordinationResp										ProtocolIE-ID ::= 293

id-E-RABs-AdmittedToBeModified-SgNBModConfList								ProtocolIE-ID ::= 294

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item								ProtocolIE-ID ::= 295

id-UEContextLevelUserPlaneActivity											ProtocolIE-ID ::= 296

id-ERABActivityNotifyItemList												ProtocolIE-ID ::= 297

id-InitiatingNodeType-EndcX2Removal											ProtocolIE-ID ::= 298

id-RespondingNodeType-EndcX2Removal											ProtocolIE-ID ::= 299

id-RLC-Status																ProtocolIE-ID ::= 300

id-CNTypeRestrictions														ProtocolIE-ID ::= 301

id-uLpDCPSnLength															ProtocolIE-ID ::= 302

id-BluetoothMeasurementConfiguration										ProtocolIE-ID ::= 303

id-WLANMeasurementConfiguration												ProtocolIE-ID ::= 304

id-NRrestrictionin5GS														ProtocolIE-ID ::= 305

id-dL-Forwarding															ProtocolIE-ID ::= 306

id-E-RABs-DataForwardingAddress-List										ProtocolIE-ID ::= 307

id-E-RABs-DataForwardingAddress-Item										ProtocolIE-ID ::= 308

id-Subscription-Based-UE-DifferentiationInfo								ProtocolIE-ID ::= 309

id-GNBOverloadInformation													ProtocolIE-ID ::= 310

id-dLPDCPSnLength															ProtocolIE-ID ::= 311

id-secondarysgNBDLGTPTEIDatPDCP												ProtocolIE-ID ::= 312

id-secondarymeNBULGTPTEIDatPDCP												ProtocolIE-ID ::= 313

id-lCID																		ProtocolIE-ID ::= 314

id-duplicationActivation													ProtocolIE-ID ::= 315

id-ECGI																		ProtocolIE-ID ::= 316

id-RLCMode-transferred														ProtocolIE-ID ::= 317

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList							ProtocolIE-ID ::= 318

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item							ProtocolIE-ID ::= 319

id-E-RABs-ToBeReleased-SgNBRelReqdList										ProtocolIE-ID ::= 320

id-E-RABs-ToBeReleased-SgNBRelReqd-Item										ProtocolIE-ID ::= 321

id-NRCGI																	ProtocolIE-ID ::= 322

id-MeNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 323

id-SgNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 324

id-new-drb-ID-req															ProtocolIE-ID ::= 325

id-endcSONConfigurationTransfer												ProtocolIE-ID ::= 326

id-NRNeighbourInfoToAdd														ProtocolIE-ID ::= 327

id-NRNeighbourInfoToModify													ProtocolIE-ID ::= 328

id-DesiredActNotificationLevel												ProtocolIE-ID ::= 329

id-LocationInformationSgNBReporting											ProtocolIE-ID ::= 330

id-LocationInformationSgNB													ProtocolIE-ID ::= 331

id-LastNG-RANPLMNIdentity													ProtocolIE-ID ::= 332

id-EUTRANTraceID															ProtocolIE-ID ::= 333

id-additionalPLMNs-Item														ProtocolIE-ID ::= 334

id-InterfaceInstanceIndication												ProtocolIE-ID ::= 335

id-BPLMN-ID-Info-EUTRA														ProtocolIE-ID ::= 336

id-BPLMN-ID-Info-NR															ProtocolIE-ID ::= 337

id-NBIoT-UL-DL-AlignmentOffset												ProtocolIE-ID ::= 338

id-ERABs-transferred-to-MeNB												ProtocolIE-ID ::= 339

id-AdditionalRRMPriorityIndex												ProtocolIE-ID ::= 340

id-LowerLayerPresenceStatusChange											ProtocolIE-ID ::= 341

id-FastMCGRecovery-SN-to-MN													ProtocolIE-ID ::= 342

id-RequestedFastMCGRecoveryViaSRB3											ProtocolIE-ID ::= 343

id-FastMCGRecoveryViaSRB3Enabled											ProtocolIE-ID ::= 344

id-RequestedFastMCGRecoveryViaSRB3Release									ProtocolIE-ID ::= 345

id-FastMCGRecoveryViaSRB3Disabled									ProtocolIE-ID ::= 346

id-FastMCGRecovery-MN-to-SN													ProtocolIE-ID ::= 347

id-PartialListIndicator														ProtocolIE-ID ::= 348

id-MaximumCellListSize														ProtocolIE-ID ::= 349

id-MessageOversizeNotification												ProtocolIE-ID ::= 350

id-CellandCapacityAssistInfo												ProtocolIE-ID ::= 351

id-TNLConfigurationInfo														ProtocolIE-ID ::= 352

END

-- ASN1STOP

*** Remaining text not changed ***