3GPP TSG-RAN WG3 #107-e																		 R3-200258

24 February-6 March 2020

E-Meeting

Agenda item:    17.2.1

Source:              ZTE

Title:                  (TP for Introduction of NR_IIOT support to 38.463): Supporting duplication for more than 2 RLC entities

Document for:   Discussion

1 Introduction

At the RAN3 #106 meeting, the issue on establishing NR-U tunnels to support duplication for more than 2 RLC entities is left and summarized in offline discussion[1]. Furthermore, In RAN2, there are some agreements related to initial UL duplication configuration for more than 2 RLC entities at UE side.

We have given further discussion on these issues. In this contribution, the text proposal for TS38.463 based on the BLCR [2] is provided.

2 References

- R3-197704, Summary of the offline on support for DL duplication with more than 2 RLCs, Nokia, Nokia Shanghai Bell

- R3-200059 Introduction of NR_IIOT support to 38.463, ZTE

3. 	Text Proposal for TS 38.463

Beginning of Text Proposal for TS 38.463

8.3.1.2	Successful Operation

<Unchanged Text Omitted>

For each requested DRB, if the PDCP Duplication IE is included and the Additional RLC Number for PDCP Duplication IE is not included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-UP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path. If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message, gNB-CU-UP should take it into account when activing/deactiving CA based DL PDCP duplication.

For each requested DRB, if the PDCP Duplication IE and the Additional RLC Number for PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-UP shall include the same number of UP Transport Layer Information IEs indicated by the Additional RLC Number for PDCP Duplication IE in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication for intra-gNB-DU CA. If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message, gNB-CU-UP should take it into account when deactiving CA based DL PDCP duplication, and when activing the corresponding number of RLC eneities indicated by the Minimum Number of Activated RLC IE and Maximum Number of Activated RLC IE contained in the BEARER CONTEXT SETUP REQUEST message for CA based DL PDCP duplication.

<Unchanged Text Omitted>

Next of Text Proposal for TS 38.463

8.3.2.2	Successful Operation

<Unchanged Text Omitted>

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, and one cell group is included in Cell Group Information IE, then the gNB-CU-CP shall include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION REQUEST message, and the gNB-CU-UP shall also include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION RESPONSE message to support packet duplication for intra-gNB-DU CA. The first UP Transport Layer Information IE of these UP Transport Layer Information IEs is for the primary path.

For a certain DRB which was allocated with two or more GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the Bearer Context Modification (gNB-CU-CP initiated) procedure, i.e. only one UP Transport Layer Information per Cell Group ID is present in DL UP Parameters IE for the concerned DRB, then the gNB-CU-UP shall consider that PDCP duplication is deconfigured for this DRB. If such Bearer Context Modification (gNB-CU-CP initiated) procedure occurs, the Duplication Activation IE ,the Minimum Number of Activated RLC IE and  the Maximum Number of Activated RLC IE shall not be included for the concerned DRB.

<Unchanged Text Omitted>

Next of Text Proposal for TS 38.463

9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. |
| PDCP SN DL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN DL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. |
| ROHC Parameters | O |  | 9.3.1.40 |  |
| T-Reordering Timer | O |  | 9.3.1.41 |  |
| Discard Timer | O |  | 9.3.1.42 |  |
| UL Data Split Threshold | O |  | 9.3.1.43 |  |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. |
| PDCP Re-establishment | O |  | ENUMERATED (true,…) | Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17] |
| PDCP Data Recovery | O |  | ENUMERATED (true,…) | Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17] |
| Duplication Activation | O |  | ENUMERATED ( Active, Inactive, …) | Information on the initial state of DL PDCP duplication |
| Out Of Order Delivery | O |  | ENUMERATED (true,…) | Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured. Out of order delivery is configured only when the radio bearer is established. |
| Additional RLC Number for PDCP Duplication | O |  | ENUMERATED (three, four, …) | The additional number of the RLC entity configured for PDCP duplication for the DRB. |
| Minimum Number of Activated RLC | O |  | INTEGER (1.. maxnoofDupRLCs, ...) | The minimum number of the RLC entity could be activated for DL PDCP duplication. |
| Maximum Number of Activated RLC | O |  | INTEGER (1.. maxnoofDupRLCs, ...) | The maximum number of the RLC entity could be activated for DL PDCP duplication. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDupRLCs | Maximum no of RLC entity for duplication. Value is 4. |

End of Text Proposal for TS 38.463