3GPP TSG-RAN WG3 #107-e 																		R3-200508

24 February-6 March 2020 E-Meeting

Title: 	(TP for NR_Mob_enh BL CR for TS 38.423): Target Node Initiated CHO Modification

Source: 	Huawei

Agenda item:	15.3.1.3

Document for:	Other

1. Introduction

This paper contains a TP for NR_Mob_enh BL CR for TS 38.42 as proposed in [1].

2. Reference

[1] R3-200507	Target Node Initiated CHO Modification.

ANNEX - TP for NR_Mob_enh BL CR for TS 38.423

Start of first change

### 8.2.B	Conditional Handover Cancel

#### 8.2.B.1	General

The Conditional Handover Cancel procedure is used to enable a target NG-RAN node to cancel an already prepared conditional handover.

The procedure uses UE-associated signalling.

#### 8.2.B.2	Successful Operation

Figure 8.2.B.2-1: Conditional Handover Cancel, successful operation

The target NG-RAN node initiates the procedure by sending the CONDITIONAL HANDOVER CANCEL message to the source NG-RAN node. The target NG-RAN node shall indicate the reason for cancelling the conditional handover by means of an appropriate cause value.

At the reception of the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that the target NG-RAN node is about to remove any reference to, and release any resources previously reserved to the concerned UE context for the concerned UE-associated signalling.

If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the cells identified by the included NG-RAN CGI are about to be released.

If the Candidate Cells To Be Cancelled List IE is included in CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that only the resources reserved for the candidate cells associated to the same UE-associated signaling identified by the Source NG-RAN node UE XnAP ID and Target NG-RAN node UE XnAP ID are about to be released.

If the CHO modification IE is included in the CONDITIONAL HANDOVER CANCEL message, the source NG-RAN node shall consider that it is a target NG-RAN node initiated conditional handover modification request, and initiate a conditional handover preparation procedure towards the same target cells indicated in the Candidate Cells To Be Cancelled List IE in CONDITIONAL HANDOVER CANCEL message as specified in section 8.2.1.

End of first change

Start of second change

9.1.1.BB	CONDITIONAL HANDOVER CANCEL

This message is sent by the target NG-RAN node to the source NG-RAN node to cancel an already prepared conditional handover.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | - | - |
| CHO Modification Information | O |  | ENUMERATED (CHO-Modification Required, …) |  | YES | ignore |

End of second change

Start of third change

-- TEXT OMITTED –

id-RequestedFastMCGRecoveryViaSRB3,

id-AdmittedFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-AdmittedFastMCGRecoveryViaSRB3Release,

id-CHOinformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-CHO-ModificationInfo,

-- TEXT OMITTED –

ConditionalHandoverCancel-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetCellsToCancel							CRITICALITY reject		TYPE TargetCellList								PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore		TYPE CHO-ModificationInfo						PRESENCE optional},

...

}

-- TEXT OMITTED –

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-AdmittedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-CHOinformation																					ProtocolIE-ID ::= AAA

id-targetCellsToCancel																				ProtocolIE-ID ::= AA1

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= AA2

id-procedureStage																					ProtocolIE-ID ::= AA3

id-CHO-ModificationInfo																				ProtocolIE-ID ::= AA4

-- TEXT OMITTED –

End of third change