3GPP TSG-RAN3 Meeting #107-e	R3-200551

24 February - 6 March 2020 E-Meeting

Agenda item:	15.4.1.2

Source:	ZTE

Title:	TP for NR_Mob_enh BL CR for TS 38.423 Correction of CHO Per Target Cell TEID

Document for:	Agreement

1	Introduction

In the agreed R3-196698 at RAN3#106, some confusions regarding HO success procedure were still left.

The “Late data forwarding” is purely NW behavior, and it cannot be really configured or conducted by UE. Hence it is suggested to reword relevant statement like “If the conditional handover is configured for this UE...”.

The source node may conditional prepare multiple candidate target nodes in parallel, so once the real target node is fixed by UE via sending the HANDOVER SUCCESS message, the source node should also cancel all other candidate target nodes if any, otherwise the CHO resources shall be kept useless for longer time. Such NW side behavior should be captured.

Some cleanups in IE semantics.

2	Text proposal

This TP is based on the BL CR endorsed after RAN3 #106 [R3-197814].

*** First change, skipped text not changed ***

8.2.A	Handover Success

8.2.A.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover, to enable a target NG-RAN node to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

8.2.A.2	Successful Operation

Figure 8.2.A.2-1: Handover Success, successful operation

The target NG-RAN node initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

If the conditional handover was configured for this UE, the source NG-RAN node shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source NG-RAN node receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE in the target NG-RAN node as cancelled, and it shall also cancel all other CHO preparations accepted for this UE in all other target NG-RAN nodes if any.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover.

Editor's note: The impact on the NG-RAN node's behaviour should be verified if the source NG-RAN node receives the HANDOVER SUCCESS message for a UE from the target NG-RAN node after receiving the CONDITIONAL HANDOVER CANCEL message for the same UE from the same target NG-RAN node.

8.2.A.3	Unsuccessful Operation

Not applicable.

8.2.A.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

*** Next change, skipped text not changed ***

9.1.1.AA	HANDOVER SUCCESS

This message is sent by the target NG-RAN node to the source NG-RAN node to indicate the successful access of the UE toward the target NG-RAN node.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell which UE successfully accessed | YES | reject |

*** Remaining text not changed ***