3GPP TSG-RAN WG3 #107-e																		R3-200683

Electronic meeting, 24th February - 6th March, 2020

Agenda item:	31

Source:	NTT DOCOMO, INC.

Title:	gNB-CU resiliency

Document for:	Discussion and Decision

1	Introduction

In RAN3#96, TP for gNB-CU/DU architecture against TS38.401 was agreed [1] including following note.

NOTE:	For resiliency, a gNB-DU may be connected to multiple gNB-CU by appropriate implementation.

It seems better to clarify what to be performed for resiliency because, on above description, the intended behaviour is not clear.

2	Discussion

Firstly, it seems better to analyse what is intended by the note. As shown in note, the case where multiple gNB-CUs against one gNB-DU case is assumed. Because “resiliency” is discussed, failure of gNB-CU needs to be considered.

Observation 1: failure of gNB-CU where one gNB-DU is connected to multiple gNB-CU is assumed by the note in TS38.401.

On the other hand, considering one gNB-CU can fully control the connected gNB-DU (e.g. cell activation/deactivation), it may be difficult to connect multiple gNB-CUs simultaneously over F1AP without very tight coordination (e.g. via OAM) on inter-vendor operation.

Observation 2: Simultaneous connection to multiple gNB-CU over F1AP seems to be difficult on inter-vendor operation because tight coordination (e.g. OAM) between gNB-CUs would be required.

Considering on the observations, following would be one of the scenarios aimed here.

Figure 1: resiliency on gNB-CU failure

Procedure would be as follows.

- gNB-DU connects to one gNB-CUs over F1AP and the gNB-CU serves for every UEs in the gNB-DU.

(gNB-DU may connect to other  gNB-CUs over transport only.).

- Once one gNB-CU (or its transport) is failed, the gNB-DU tries to connect to other gNB-CUs over F1AP

(And, UEs will reconnect to gNB-DU and new gNB-CU.)

However, whether the note directly intends to such behaviour seems unclear. So, it would be better to clarify the one of intended behaviour by the note is as mentioned above.

Proposal: Because the intention of the “resiliency” note in current TS38.401 is not clear, RAN3 to clarify following is the one of intended behaviour.

- gNB-DU connects to one gNB-CUs over F1AP and the gNB-CU serves for every UEs in the gNB-DU.

(gNB-DU may connect to other  gNB-CUs over transport only.).

- Once one gNB-CU (or its transport) is failed, the gNB-DU tries to connect to other gNB-CUs over F1AP

(And, UEs will reconnect to gNB-DU and new gNB-CU.)

3	Conclusion

It seems better to clarify what is the actual intended behaviour on the note.

Following observations and proposals were obtained.

Observation 1: failure of gNB-CU where one gNB-DU is connected to multiple gNB-CU is assumed by the note in TS38.401.

Observation 2: Simultaneous connection to multiple gNB-CU over F1AP seems to be difficult on inter-vendor operation because tight coordination (e.g. OAM) between gNB-CUs would be required.

Proposal: Because the intention of the “resiliency” note in current TS38.401 is not clear, RAN3 to clarify following is the one of intended behaviour.

- gNB-DU connects to one gNB-CUs over F1AP and the gNB-CU serves for every UEs in the gNB-DU.

(gNB-DU may connect to other  gNB-CUs over transport only.).

- Once one gNB-CU (or its transport) is failed, the gNB-DU tries to connect to other gNB-CUs over F1AP

(And, UEs will reconnect to gNB-DU and new gNB-CU.)

Note that same issue is in CP-UP separation as similar description is there.

Corresponding CRs are available in [2]

References

- R3-171996, “TP of gNB-CU/DU architecture,” Nokia, Alcatel-Lucent Shanghai Bell

- R3-20xxxx, “CR for clarification on gNB-CU resiliency,” NTT DOCOMO, INC.