3GPP TSG-RAN WG3 #107-e	R3-201013

# Online, 24th February - 06th March 2020

Agenda Item:	14.4.3

Source:	Ericsson

Title: 	Considerations on SON support for reporting

Document for:	Discussion

# 1	Introduction

The Rel-16 Work Item Description for NB-IoT contains a common objective on network management tool enhancement:

Network management tool enhancement:

- SON support for reporting of [RAN2, RAN3]

  - Cell Global Identity and strongest measured cell(s) (ANR)

  - Random access performance

  - Radio link failure (RLF), if needed

In this contribution, we discuss some general RAN3 aspects of SON support, namely ANR reporting and RLF.

# Discussion

In the previous RAN2 meetings, the following agreements were made:

RAN2#105bis agreements: - RACH report: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. At least RACH report for NB-IoT based on UE information request/response procedure is supported. It is up to the network to decide when to request for the RACH report. RAN2#106 agreements: - SON-ANR: ANR report does not trigger EDT. ANR report is not carried in EDT Msg3. anr-InfoAvailable indication can be reported in EDT Msg3. ANR procedure steps: - 1) UE receives “measurement” configuration in dedicated signalling. - 2) When the UE is released to idle - The UE performs the instructed measurements according to existing RAN4 cell reselection measurement performance requirements. - CGI reading is limited to cells being above a certain threshold. Threshold details FFS. - 3) Reporting takes place, upon network request, next time UE is connected mode again. - RACH report: Not support EDT RACH report in EDT Msg3. RAN2#107 agreements: - SON-ANR: - Configuration: Dedicated ANR measurement configuration is provided in RRCConnectionRelease message only. The list of frequencies where to perform the measurements can be optionally provided in dedicated signalling, as a subset of the frequencies signalled in system information. Per default, the list frequencies signalled in system information is used. Maximum of 8 carriers can be configured. A blacklist of cells can be optionally signalled per carrier. - Measurements: ANR measurements are only performed while UE remains camped on the same cell from which the ANR measurement configuration was received. (i.e. When the UE changes cell the ANR measurements are stopped). Neighbour cell relaxed monitoring criteria and Ssearch criteria don’t apply while performing an ANR measurement. ANR measurement doesn’t affect DRX / eDRX operation. CGI-reading applies to strongest cell on each frequency, if above the RSRP threshold. The RSRP threshold is an absolute threshold that is common to all frequencies. - Reporting: anr-InfoAvailable indication is reported in and only in RRCConnectionSetupComplete, RRCConnectionResumeComplete, RRCConnectionReestablishmentComplete and RRCConnectionResumeRequest for EDT. The ANR report might be empty for a given carrier, if a measurement is performed and no strong cell is found. UE only indicates the availability of the report if 1) a report is available and 2) if the RPLMN is included in the list of EPLMNs stored by the UE when the ANR measurement was configured. The ANR report is discarded after: - Timer expiry. Value FFS. - Power off or detach. - Reporting anr-InfoAvailable and returning to idle. UE Information Request / Response is the only reporting procedure for ANR measurement. For a measured carrier the following information is included: - Carrier frequency. - Strongest cell PCI, RSRP/RSRQ. - Strongest cell CGI-info if RSRP above the threshold. Serving cell RSRQ/RSRP and CGI are included in the ANR report. ANR reporting is optional for NB-IoT UEs with capability signaling. FFS: ANR applicability to 5GC.

## 2.1 SON ANR Reporting

From the latest RAN2 agreements highlighted above, we can see that RAN2 agreed that ANR measurements reporting in NB-IoT is only performed while the UE remains camped on the same cell from which the ANR measurement configuration was received. (i.e. When the UE changes cell the ANR measurements are stopped).

From our understanding, the UE will perform the ANR measurement and send it to the eNB, which can accept the ANR report. The eNB can forward it then to the OAM entity. In case the UE happens to perform a mobility action towards a new cell, RAN2 has specified that the UE will send a flag to indicate that the ANR report is available. If the eNB is capable of retrieving it, then it will fetch it directly from the UE; otherwise, the UE may discard it after a period of time (e.g., 48 hours). Thus, what RAN3 can take from the RAN2’s agreements is that X2 setup/handover is not needed when UE performs ANR reporting.

Observation 1: For SON ANR reporting, the transfer of ANR measurements report from new eNB to old eNB is not required as it can be sent to OAM entity for post processing.

## 2.2 Radio Link failure

If RAN2 decides to introduce a new RRC IE (e.g. RLF-Report-NB) to include the identified parameters, RAN3 will need to update the definition of the UE RLF Report Container IE like below (with the red text):

| UE RLF Report Container | OCTET STRING | RLF-Report-r9 IE or the RLF-Report-NB IE contained in the UEInformationResponse message (TS 36.331) |
| --- | --- | --- |

While there is no handover support for NB-IoT, introducing the IE in the X2AP messages RLF INDICATION and HANDOVER REPORT, could be used to optimize when (at a certain power/quality threshold) to release the UE to another frequency for better coverage. This could be performed when the UE is still communicating or when it has ended its communication. The RLF Report could also indicate neighbour cells, which info could be used to setup X2 and then exchange specific information such as e.g. RACH info.

Observation 2: Having information on RLF-Report-NB in the UE RLF report is beneficial and can be used to setup X2 and then exchange specific information such as e.g. RACH info. Future discussion should be pending on RAN2 progress

RAN2 also mention in the received LS R2-1916580 that:

The successful HO Report is not supported in R16. Can be further discussed in future release.

Considering that RLF report applies also to NB-IoT, there are therefore not many impacts from RAN3 to support the exchange of these parameters during HO.

Observation 3: For SON ANR reporting for NB-IoT; there are no tangible and strong motivations, for supporting the exchange of such information between eNBs over X2.

Besides, since SON ANR has not been discussed at all during Rel-16 and due to lack of time to finalize discussion on the potential aspects of SON ANR reporting - including RAN3 impacts and benefits,  it is proposed to push this discussion to next release.

Proposal: Due to lack of time, it is proposed to push discussion about SON ANR reporting for NB-IoT to Rel-17

# Conclusion

In this paper, we have discussed some general RAN3 impacts related to the WID on SON support for reporting. We have made the following observations:

Observation 1: For SON ANR reporting, the transfer of ANR measurements report from new eNB to old eNB is not required as it can be sent to OAM entity for post processing.

Observation 2: Having information on RLF-Report-NB in the UE RLF report is beneficial and can be used to setup X2 and then exchange specific information such as e.g. RACH info. Future discussion should be pending on RAN2 progress

Observation 3: For SON ANR reporting for NB-IoT; there are no tangible and strong motivations, for supporting the exchange of such information between eNBs over X2

From the above observations, we propose for RAN3 the following:

Proposal: Due to lack of time, it is proposed to push discussion about SON ANR reporting for NB-IoT to Rel-17

# References

[1]  R3-193412, “Consideration on RACH optimization for NB-IoT”, Huawei, RAN3#105