3GPP TSG-RAN WG3 #107-e	R3-20xxxx

# Online, 24th February - 06th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.455 | CR | 0107 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter-RAT Measurement of NR Cells for E-CID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An LTE UE is likely to be able to measure a number of neighbor NR cells from any 5G network. It seems highly beneficial to leverage this capability to increase positioning accuracy of E-CID measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of measurements for NR cells in Inter-RAT Measurement Quantities IE and Inter-RAT Measured Results IE. The measured NR cells may come from any 5G network (the measurement does not distinguish between e.g. SA and NSA). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No way to support NR measurements as inter-RAT measurements in E-CID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.1, 9.2.13, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by E-SMLC to initiate E-CID measurements.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED(OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…) |  | - | - |
| Inter-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Inter-RAT Measurement Quantities Item | M |  | ENUMERATED(GERAN, UTRAN,…, NR) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value “Periodic”. |

NEXT CHANGE

### 9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. |
| >>Result NR | M | 1..<maxNRMeas> |  |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) |  |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 32. |

NEXT CHANGE

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs

FROM LPPA-Constants

UNCHANGED TEXT OMITTED

#### -- I

InterRATMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {InterRATMeasurementQuantities-ItemIEs} }

InterRATMeasurementQuantities-ItemIEs LPPA-PROTOCOL-IES ::= {

{ ID id-InterRATMeasurementQuantities-Item	CRITICALITY reject	TYPE InterRATMeasurementQuantities-Item PRESENCE mandatory}}

InterRATMeasurementQuantities-Item ::= SEQUENCE {

interRATMeasurementQuantitiesValue			InterRATMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { InterRATMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

InterRATMeasurementQuantitiesValue-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

InterRATMeasurementQuantitiesValue ::= ENUMERATED {

geran,

utran,

...,

nr

}

InterRATMeasurementResult ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF InterRATMeasuredResultsValue

InterRATMeasuredResultsValue ::= CHOICE {

resultGERAN			ResultGERAN,

resultUTRAN			ResultUTRAN,

...,

resultNR			ResultNR

}

#### -- J

UNCHANGED TEXT OMITTED

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

NPRSConfiguration ::=			SEQUENCE {

nPRSSubframePartA			NPRSSubframePartA 		OPTIONAL,

nPRSSubframePartB			NPRSSubframePartB 		OPTIONAL,

...

}

NPRSMutingConfiguration ::= CHOICE {

two 				BIT STRING (SIZE (2)),

four 				BIT STRING (SIZE (4)),

eight 				BIT STRING (SIZE (8)),

sixteen 			BIT STRING (SIZE (16)),

...

}

NPRSSubframePartA ::=			SEQUENCE {

bitmapsforNPRS				BitmapsforNPRS,

nPRSMutingConfiguration		NPRSMutingConfiguration 		OPTIONAL,

...

}

NPRSSubframePartB ::=			SEQUENCE {

numberofNPRSOneOccasion		ENUMERATED {sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, ..., sf2560},

periodicityofNPRS			ENUMERATED {sf160, sf320, sf640, sf1280, ..., sf2560},

startingsubframeoffset		ENUMERATED {zero, one-Eighth, two-Eighths, three-Eighths, four-Eighths, five-Eighths, six-Eighths, seven-Eighths, ...},

nPRSMutingConfiguration		NPRSMutingConfiguration 		OPTIONAL,

...,

sIB1-NB-Subframe-TDD			ENUMERATED {	sf0, sf4, sf0and5, ...} 		OPTIONAL

}

NumberOfAntennaPorts ::= ENUMERATED {

n1-or-n2,

n4,

...

}

NumberOfDlFrames ::= ENUMERATED {

sf1,

sf2,

sf4,

sf6,

...

}

NumberOfDlFrames-Extended ::= INTEGER (1..160,...)

NumberOfFrequencyHoppingBands ::= ENUMERATED {

twobands,

fourbands,

...

}

NPRSSequenceInfo ::= INTEGER (0..174,...)

NRARFCN ::= INTEGER (0.. 3279165)

NRPCI ::= INTEGER (0..1007)

#### -- O

UNCHANGED TEXT OMITTED

#### -- Q

#### -- R

RepetitionNumberofSIB1-NB ::= ENUMERATED {

r4,

r8,

r16,

...

}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...),

bandwidth				INTEGER (1..100, ...),

...

}

ResultRSRP ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-Item

ResultRSRP-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRP		ValueRSRP,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRP-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-Item

ResultRSRQ-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRQ		ValueRSRQ,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRQ-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultNR ::= SEQUENCE (SIZE (1.. maxNRmeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nRARFCN				NRARFCN,

nRPCI				NRPCI,

sS-NRRSRP			SS-NRRSRP	OPTIONAL,

sS-NRRSRQ			SS-NRRSRQ	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} } OPTIONAL,

...

}

ResultNR-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

#### -- S

SFNInitialisationTime ::= BIT STRING (SIZE (64))

SRSConfigurationForAllCells ::= SEQUENCE (SIZE (1.. maxServCell)) OF SRSConfigurationForOneCell

SRSConfigurationForOneCell ::= SEQUENCE {

pci							PCI,

ul-earfcn					EARFCN,

ul-bandwidth				ENUMERATED {n6, n15, n25, n50, n75, n100},

ul-cyclicPrefixLength		CPLength,

srs-BandwidthConfig			ENUMERATED {bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7},

srs-Bandwidth				ENUMERATED {bw0, bw1, bw2, bw3},

srs-AntennaPort				ENUMERATED {an1, an2, an4, ...},

srs-HoppingBandwidth		ENUMERATED {hbw0, hbw1, hbw2, hbw3},

srs-cyclicShift				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7},

srs-ConfigIndex				INTEGER (0..1023),

maxUpPts					ENUMERATED {true}			OPTIONAL,	-- Cond ifTDD

transmissionComb			INTEGER (0..1),

freqDomainPosition			INTEGER (0..23),

groupHoppingEnabled			BOOLEAN,

deltaSS						INTEGER (0..29)				OPTIONAL,

sfnInitialisationTime		SFNInitialisationTime,

...

}

Subframeallocation ::= CHOICE {

oneFrame						BIT STRING (SIZE(6)),

fourFrames						BIT STRING (SIZE(24))

}

SS-NRRSRP ::= INTEGER (0..127)

SS-NRRSRQ ::= INTEGER (0..127)

SSID ::= OCTET STRING (SIZE(1..32))

SystemInformation ::= SEQUENCE (SIZE (1.. maxNrOfPosSImessage)) OF SEQUENCE {

broadcastPeriodicity				BroadcastPeriodicity,

posSIBs								PosSIBs,

iE-Extensions						ProtocolExtensionContainer { { SystemInformation-ExtIEs} }	OPTIONAL,

...

}

SystemInformation-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

NEXT CHANGE

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

LPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM LPPA-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-uTDOAInformationExchange										ProcedureCode ::= 7

id-uTDOAInformationUpdate										ProcedureCode ::= 8

id-assistanceInformationControl									ProcedureCode ::= 9

id-assistanceInformationFeedback								ProcedureCode ::= 10

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellineNB								INTEGER ::= 256

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxGERANMeas								INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxCellineNB-ext							INTEGER ::= 3840

maxMBSFN-Allocations						INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNRmeas									INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

UNCHANGED TEXT OMITTED

END OF CHANGES