3GPP TSG-RAN3 Meeting #107-e	R3-201060

E-Meeting, 24 February – 6 March, 2020

Title: 	(TP for NR_IIOT BL CR for TS 38.413):TSC Assistance Information

Source: 	Huawei

Agenda item:	17.3.2

Document Type:	other

1. Introduction

In this document we intend to introduce the CN PDB for UL and DL based on progress in other groups.

2. Discussion

- Last RAN2#107 meeting discussed the CN PDB, and agreed to include both the UL and DL CN PDB from the CN in [3].

- Q3) SA WG2 would like to ask RAN WG2 whether for QoS Flows of Delay critical GBR resource type direction-specific values for the CN component of the PDB can be used by the NG-RAN to operate with different delay budgets for the uplink and the downlink direction and helpful to improve the resource scheduling for the NG-RAN. - [RAN2 answer]: RAN2 thinks that it is helpful to improve the resource scheduling for the uplink and downlink scheduling for the RAN respectively.

- Based on this LS, last SA2#136 Ad hoc agreed to introduce the CN PDB for uplink and downlink respectively.

- ….. - Different dynamic value for CN PDB may be configured per uplink and downlink direction. - NOTE 3: The configuration of transport network on CN tunnel can be different per UL and DL, which can be different value for CN PDB per UL and DL

- Generally there are two options to update the BL CR as follows.

- Option 1: Add a new CN PDB for UL in Non Dynamic 5QI Descriptor and Dynamic 5QI Descriptor

- Option 2: Redefine the CN PDB to include the value for UL and DL.

- It seems simple to agree option 1, and the following provides an example.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255, …) | Indicates the standardized or pre-configured 5QI as specified in TS 23.501 [9]. | - |  |
| Priority Level | O |  | 9.3.1.84 | Priority Level is specified in TS 23.501 [9]. When included, it overrides standardized or pre-configured value. | - |  |
| Averaging Window | O |  | 9.3.1.82 | Averaging Window is specified in TS 23.501 [9]. When included, it overrides standardized or pre-configured value. | - |  |
| Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. When included, it overrides standardized or pre-configured value. | - |  |
| CN Packet Delay Budget Downlink | O |  | Packet Delay Budget 9.3.1.80 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Packet Delay Budget 9.3.1.80 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

- Hence based on the analysis, the following proposal is made.

The CN PDB for uplink and downlink are introduced in Dynamic 5QI Descriptor and Non Dynamic 5QI Descriptor.

- The TPs for TS 38.423 and TS 38.473 can be found in [4-5].

5. Reference

- R2-1914201 Reply LS on potential improvements for delay critical QoS flows, Huawei

- S2-2001098 CN component of the PDB is configured per UL and DL, OPPO, China Telecom, China Unicom, Qualcomm Incorporated

- R3-197712 TSCAI and CN PDB: Summary of offline discussion, Nokia, Nokia Shanghai Bell

- R2-1914223 LS on granularity of periodicity and burst arrival time in TSCAI, Qualcomm

- R3-201061	(TP for NR_IIOT BL CR for TS 38.423):TSC Assistance Information,	Huawei

- R3-201062	(TP for NR_IIOT BL CR for TS 38.473): TSC Assistance Information, Huawei

5. TP for TS 38.413 on top of [R3-200056]

Beginning of Text Proposal for TS 38.413

9.3.1.18	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Priority Level | M |  | 9.3.1.84 | Priority Level is specified in TS 23.501 [9]. | - |  |
| Packet Delay Budget | M |  | 9.3.1.80 | Packet Delay Budget is specified in TS 23.501 [9]. This IE is ignored if the Extended Packet Delay Budget IE is present. | - |  |
| Packet Error Rate | M |  | 9.3.1.81 | Packet Error Rate is specified in TS 23.501 [9]. | - |  |
| 5QI | O |  | INTEGER (0..255, …) | Indicates the dynamically assigned 5QI as specified in TS 23.501 [9]. | - |  |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical, …) | Indicates whether the GBR QoS flow is delay critical as specified in TS 23.501 [9]. | - |  |
| Averaging Window | C-ifGBRflow |  | 9.3.1.82 | Averaging Window is specified in TS 23.501 [9]. | - |  |
| Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE shall be included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | - |  |
| Extended Packet Delay Budget | O |  | 9.3.1.x2 | Packet Delay Budget is specified in TS 23.501 [9]. | YES | ignore |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.x2 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | Ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.x2 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | Ignore |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

Next Change

9.3.1.28	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255, …) | Indicates the standardized or pre-configured 5QI as specified in TS 23.501 [9]. | - |  |
| Priority Level | O |  | 9.3.1.84 | Priority Level is specified in TS 23.501 [9]. When included, it overrides standardized or pre-configured value. | - |  |
| Averaging Window | O |  | 9.3.1.82 | Averaging Window is specified in TS 23.501 [9]. When included, it overrides standardized or pre-configured value. | - |  |
| Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. When included, it overrides standardized or pre-configured value. | - |  |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.x2 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | Ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.x2 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | Ignore |

Next Change

<Unchanged Text Omitted>

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

Dynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

<Unchanged Text Omitted>

NonDynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDownlink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUplink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= 181

id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= xxx

End of Text Proposal for TS 38.413