3GPP TSG-RAN3 Meeting #107-	R3-201067

E-Meeting, 24 February – 6 March, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter-RAT HO from SA to EN-DC with shared gNB/sgNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Direct_data_fw_NR-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For inter-system handover from SA to EN-DC,the source gNB and the target sgNB could be the same node, e.g., in EPS fallack case. The data fowarding between the source gNB and target sgNB can be handled internally. Hence it becomes necessary for the target sgNB to be aware of the shared gNB/sgNB case for the SA to ENDC handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)Add the Global NG-RAN Node ID and RAN UE NGAP ID within the Source eNB to Target eNB Transparent Container 2)Add the RAN UE NGAP ID in the SGNB ADDITION REQUEST message 3)If the data forwarding is omitted for SN terminated bearers, the SN maps the data received from the 5GC to the respective E-RAB without PDCP SN and QFI based on the mapping between QoS flow ID(s) and E-RAB ID(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data forwarding of the SN termianted bearers at the sgNB will increase the latency for the gNB/sgNB shared case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the CR R2-1916586 of RAN2 which has been agreed in principle. The new changes are marked using “Huawei_3” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<********************************Start of change*****************************************>

## 10.9	eNB/gNB to Master Node change

### 10.9.1	EN-DC

The eNB to Master Node change procedure is used to transfer context data from a source eNB to a target MN that adds an SN during the handover.

Figure 10.9.1-1: eNB to Master Node change

Figure 10.9.1-1 shows an example signaling flow for eNB to Master Node change:

1.	The source eNB starts the handover procedure by initiating the X2 Handover Preparation procedure.

2.	The target MN sends SgNB Addition Request to the target SN.

3.	The target SN replies with SgNB Addition Request Acknowledge. If data forwarding is needed, the target SN provides forwarding addresses to the target MN.

4.	The target MN includes within the Handover Request Acknowledge message a transparent container to be sent to the UE as an E-UTRA RRC message, including a NR RRC configuration message which also includes the SCG configuration, to perform the handover, and may also provide forwarding addresses to the source eNB.

5.	The source eNB triggers the UE to apply the new configuration.

6/7.	The UE synchronizes to the target MN and replies with RRCConnectionReconfigurationComplete message.

8.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the target SN.

9.	If the RRC connection reconfiguration procedure was successful, the target MN informs the target SN.

10.	For bearers using RLC AM, the source eNB sends the SN Status transfer to the target MN.

11.	Data forwarding from the source eNB takes place.

12-15.	The target MN initiates the S1 Path Switch procedure.

NOTE 1:	If new UL TEIDs of the S-GW are included, the target MN performs MN initiated SN Modification procedure to provide them to the target SN.

16.	The target MN initiates the UE Context Release procedure towards the source eNB.

NOTE 2:	Inter-system HO from NR to E-UTRA with EN-DC configuration is also supported in Rel-16. N26 based inter-system HO will be executed when source RAN is gNB. The source gNB includes the Global NG-RAN Node ID and RAN UE NGAP ID within the Source eNB to Target eNB Transparent Container in the Handover Required. If the target eNB decides to use the source gNB as the target SgNB, the target eNB sends SN Addition Request to the target SgNB including the RAN UE NGAP ID as a reference to the UE context in the gNB. If the source gNB and the target SgNB are the same gNB, data forwarding may be omitted for SN-terminated bearers and the target SgNB maps the data received from the 5GC to the respective E-RAB without PDCP SN and QFI based on the mapping between QoS flow ID(s) and E-RAB ID(s).

NOTE 3:	Inter-system HO from E-UTRA connected to 5GC to E-UTRA with EN-DC configuration is not supported. Inter-system HO from E-UTRA with EPC to MR-DC with 5GC is not supported.

<********************************End of change*****************************************>