3GPP TSG-RAN WG3 Meeting #107-e	R3-201111

Electronic Meeting, February 24th – March 6th, 2020

Agenda item:		15.3.1

Source:	Intel Corporation

Title:	(TP for NR_Mob_enh-Core BL CR for TS 38.300): Clean-up for Stage-2 CHO

Document for:		Other

# 1.	Introduction

This contribution proposes TP for TS 38.300 based on [1], RAN2 running CR, and also enhancements proposed in [2].

# 2		Reference

[1] R3-201109, “Wrap-up for CHO”, Intel Corporation

[2] R3-201105, “(TP for NR_Mob_enh-Core BL CR for TS 38.300): Support for per DRB DAPS”, Intel Corporation

# 3		TP for BL CR for TS 38.300

///////////////////////////////////////////////////////////// taken from RAN2 running CR ////////////////////////////////////////////////////////////////

#### 9.2.3.X	Conditional Handover

##### 9.2.3.x.1	General

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once the execution condition(s) is met.

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in [12]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evalution of CHO execution condition of a single candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 9.2.3.2, regardless of any previously received CHO configuration.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

CHO is not supported for N2 based handover in this release of the specification.

##### 9.2.3.x.2	C-plane handling

As in intra-NR RAN handover, in intra-NR RAN CHO, the preparation and execution phase of the conditional handover procedure is performed without involvement of the 5GC; i.e. preparation messages are directly exchanged between gNBs. The release of the resources at the source gNB during the conditional handover completion phase is triggered by the target gNB. The figure below depicts the basic conditional handover scenario where neither the AMF nor the UPF changes:

Figure 9.2.3.x.2-1: Intra-AMF/UPF Conditional Handover

0/1.	Same as step 0, 1 in Figure 9.2.3.2.1-1 of section 9.2.3.2.1.

2.	The source gNB decides to use CHO.

3.	The source gNB requests CHO to one or more candidate gNBs. A CHO request message is sent for each candidate cell.

4.	Same as step 4 in Figure 9.2.3.2.1-1 of section 9.2.3.2.1.

5.	The candidate gNB(s) sends CHO response including configuration of CHO candidate cell to the source gNB. The response mssage is also sent for each candidate cell.

6.	The source gNB sends an RRCReconfiguration message to the UE, containing the configuration of CHO candidate cell(s) and CHO execution condition(s).

7.	UE sends an RRCReconfigurationComplete message to the source gNB.

7a	If the early data forwarding is applied for a candidate cell, the source gNB sends the EARLY FORWARDING TRANSFER message.

8.	UE maintains connection with source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). If at least one CHO candidate cell satisfies the corresponding CHO execution condition, the UE detaches from the source gNB, applies the stored corresponding configuration for that selected candidate cell, synchronises to that candidate cell and completes the RRC handover procedure by sending RRCReconfigurationComplete message to the target gNB. The UE releases stored CHO configurations after successful completation of RRC handover procedure.

8a/b	The target gNB sends the HO SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message.

NOTE:	The late data forwarding may be initiated as soon as the source gNB receives the HO SUCCESS message.

11c.	The source gNB also sends the HO CANCEL message to the other potential target gNBs, if any, to cancel CHO for the UE.

##### 9.2.3.x.3	U-plane handling

The U-plane handling for Conditional Handover follows the same principles for DAPS Handover in 9.2.3.2.2, if the early data forwarding is applied. If the late data forwarding is applied, the U-plane handling follows the RLC-AM or RLC-UM bearer principles defined in 9.2.3.2.2.

////////////////////////////////////////////////////////////// taken from RAN3 BL CR ….////////////////////////////////////////////////////////////////

##### 9.2.3.x.3	Data Forwarding

If the late data forwarding is applied, the source NG-RAN node initiates data forwarding once it knows which target NG-RAN node the UE has successfully accessed. In that case the behavior of the Conditional Handover data forwarding follows the same behavior as defined in 9.2.3.2.3 for the intra-system handover data forwarding, except the behaivor for DRBs configured with DAPS Handover.

If the early data forwarding is applied instead, the source NG-RAN node initiates data forwarding before the UE executes the handover, to a candidate target node of interest. The behavior of the early data forwarding for the Conditional Handover follows the same principles for DRBs configured with DAPS Handver in the intra-system handover as defined in 9.2.3.2.3.