3GPP TSG-RAN WG3 Meeting #107e	R3-201118

- Online, 24th – 28th February 2020

Agenda Item:	9.3.1

Source:	Ericsson

Title:	Summary of offline discussion for CB: # 5_Email005-RAN_sharing

Document for:	Discussions & Approval

# 1	Introduction

This is the summary of email discussions on CB number five of the RAN3 number 107e meeting.

Chairs summary is as follows:

-  NG-RAN node sends all the supported Global NG-RAN node ID list towards AMF? (ZTE)

- Specify in st2: “The included PLMN identity is the first PLMN identity broadcast for this NCI in SIB1.” (Nok) or similar St2 NOTE? (HW,CT,CU)

- State in St2: “Any combination of cell ID/associated PLMN IDs is valid” + specify st3 abnormal condition; add statements in S1AP, X2AP? (E///)

- rev if needed; check details

(E///)

Summary of offline disc R3-201118

# 2	Discussion

## 2.1	Common Understanding

Proposal 1:	It is confirmed that the current information elements contained in the messages of Application Protocols under RAN3 control are sufficient to cope with any RAN sharing scenario and related requirements.

NOTE:	This proposal explicitly rules out the approach shown in [1] and [2], which is not further discussed.

Proposal 2:	It is also confirmed, that the stage 2 and/or stage 3 specifications have to be amended by additional clarifications (e.g. in non-normative text) or requirements (in normative text). This aspect is the main part of discussions at RAN3#107e.

Proposal 3:	It is proposed to apply changes from Rel-15 onwards.

## 2.2	Specification Approach

### 2.2.1	Approach number one

This approach is followed in [3] and [9] foresees to specify explicitly in stage 2 normative text (38.300) that the global NCI contains the first of the broadcast PLMN IDs associated with the NR cell ID.

### 2.2.2	Approach number two

This approach is followed in [4] - [8] foresees to include a NOTE recommending that specify explicitly in stage 2 normative text (38.300) that the global NCI contains the first of the broadcast PLMN IDs associated with the NR cell ID.

### 2.2.3	Approach number three

This approach is followed in [10] - [22] and foresees stage 2 and stage 3 changes, whereas stage 3 changes refer to stage 2 specification text, which requires involved network nodes being able to cope with RAN node IDs constructed from any possible combination of the node-ID part in the cell identifier and the PLMN IDs associated to it.

NOTE:	The other aspect in these set of contributions on solving S1/NG Setup issues with a RAN node ID not using a PLMN ID of the PLMN IDs supported by both, RAN and CN, will not be part of 107e discussions.

### 2.2.4	Proposal on how to combine aspects of approach one, two and three

Proposal 4:	It is proposed to confine specification changes to informative text in TS 36.300 and TS 38.300 containing recommendations only (“should”).

Proposal 5:	It is proposed to include only informative text in stage 2 which contains two aspects: 
(1) the fact that involved network nodes should be able to cope with any combination of node/cell/TAC identifiers and PLMN IDs
(2) that, preferably, global node/cell/TAC identifier should contain the first PLMN ID, within the common set of PLMN IDs broadcast and supported by the CN.

# 3	Conclusion and Proposals

Proposal 1:	It is confirmed that the current information elements contained in the messages of Application Protocols under RAN3 control are sufficient to cope with any RAN sharing scenario and related requirements.

Proposal 2:	It is also confirmed, that the stage 2 and/or stage 3 specifications have to be amended by additional clarifications (e.g. in non-normative text) or requirements (in normative text). This aspect is the main part of discussions at RAN3#107e.

Proposal 3:	It is proposed to apply changes from Rel-15 onwards.

Proposal 4:	It is proposed to confine specification changes to informative text in TS 36.300 and TS 38.300 containing recommendations only (“should”).

Proposal 5:	It is proposed to include only informative text in stage 2 which contains two aspects: 
(1) the fact that involved network nodes should be able to cope with any combination of node/cell/TAC identifiers and PLMN IDs
(2) that, preferably, global node/cell/TAC identifier should contain the first PLMN ID, within the common set of PLMN IDs broadcast and supported by the CN.

Examples of how this approach can be followed in stage 2 and stage 3 is given in the Annex A of this document.

# 4	References

[1]	R3-200324, Further discussion on RAN sharing impact over NG interface (ZTE Corporation), discussion

[2]	R3-200325, Stage3 CR for RAN sharing over NG interface (ZTE Corporation), CR0322r, TS 38.413 v16.0.0, Rel-16, Cat. F

[3]	R3-200388, Further discussion on construction of NCGI when multiple cell IDs are broadcast in SIB1 (Nokia, Nokia Shanghai Bell), discussion

[4]	R3-200639, Further discussions on NCGI and TAI when multiple cell IDs are broadcast in SIB1 (Huawei, China Telecom, China Unicom), discussion

[5]	R3-200640, Clarification to 36.300 on constructing NCGI and TAI when multiple cell IDs are broadcast in SIB1 (Huawei, China Telecom, China Unicom), draftCRr, TS 36.300 v15.8.0, Rel-15, Cat. F

[6]	R3-200641, Clarification to 36.300 on constructing NCGI and TAI when multiple cell IDs are broadcast in SIB1 (Huawei, China Telecom, China Unicom), draftCRr, TS 36.300 v16.0.0, Rel-16, Cat. A

[7]	R3-200642, Clarification to 38.300 on constructing NCGI and TAI when multiple cell IDs are broadcast in SIB1 (Huawei, China Telecom, China Unicom), draftCRr, TS 38.300 v15.8.0, Rel-15, Cat. F

[8]	R3-200643, Clarification to 38.300 on constructing NCGI and TAI when multiple cell IDs are broadcast in SIB1 (Huawei, China Telecom, China Unicom), draftCRr, TS 38.300 v16.0.0, Rel-16, Cat. A

[9]	R3-200775, Construction of NCGI when multiple cell IDs are broadcast in SIB1 (Nokia, Nokia Shanghai Bell), draftCRr, TS 38.300 v15.8.0, Rel-15, Cat. F

[10]	R3-200837, RAN Sharing and Global RAN node Identifiers (Ericsson), discussion

[11]	R3-200838, eNB node identification in case it is associated with more than one PLMN ID (Ericsson), draftCRr, TS 36.300 v15.8.0, Rel-15, Cat. F

[12]	R3-200839, eNB node identification in case it is associated with more than one PLMN ID (Ericsson), draftCRr, TS 36.300 v16.0.0, Rel-16, Cat. A

[13]	R3-200842, eNB node identification in case it is associated with more than one PLMN ID (Ericsson), CR1756r, TS 36.413 v15.8.0, Rel-15, Cat. F

[14]	R3-200843, eNB node identification in case it is associated with more than one PLMN ID (Ericsson), CR1757r, TS 36.413 v16.0.0, Rel-16, Cat. A

[15]	R3-200844, eNB node identification in case it is associated with more than one PLMN ID (Ericsson), CR1457r, TS 36.423 v15.8.0, Rel-15, Cat. F

[16]	R3-200845, eNB node identification in case it is associated with more than one PLMN ID (Ericsson), CR1458r, TS 36.423 v16.0.0, Rel-16, Cat. A

[17]	R3-200848, NG-RAN node identification in case it is associated with more than one PLMN ID (Ericsson), draftCRr, TS 38.300 v15.8.0, Rel-15, Cat. F

[18]	R3-200849, NG-RAN node identification in case it is associated with more than one PLMN ID (Ericsson), draftCRr, TS 38.300 v16.0.0, Rel-16, Cat. A

[19]	R3-200852, NG-RAN node identification in case it is associated with more than one PLMN ID (Ericsson), CR0348r, TS 38.413 v15.6.0, Rel-15, Cat. F

[20]	R3-200853, NG-RAN node identification in case it is associated with more than one PLMN ID (Ericsson), CR0349r, TS 38.413 v16.0.0, Rel-16, Cat. A

[21]	R3-200856, NG-RAN node identification in case it is associated with more than one PLMN ID (Ericsson), CR0324r, TS 38.423 v15.6.0, Rel-15, Cat. F

[22]	R3-200857, NG-RAN node identification in case it is associated with more than one PLMN ID (Ericsson), CR0325r, TS 38.423 v16.0.0, Rel-16, Cat. A

# Annex A	Example implementation of the proposals

## A.1	36.300

Each Cell Identity associated with a subset of PLMNs identifies its serving eNB.

NOTE 1:	Network nodes should be prepared to cope with any possible combination of a Cell Identity with one of its associated PLMN identifiers for identifying its serving eNB, or, in case of EN-DC, its serving en-gNB. E.g., identification e.g. of the target eNB in the course of handover or transfer of RAN configuration information may use a combination with a different PLMN identifier than the one used at S1 Setup.

NOTE 2:	S1AP and X2AP signalling should use the first PLMN ID within the common subset of PLMN IDs supported by the serving MME and broadcast in SIB1 for the cell, following the order broadcast in SIB1.

## A.2	38.300

Each Cell Identity associated with a subset of PLMNs identifies its serving NG-RAN node.

NOTE 1:	Network nodes should be prepared cope with any possible combination of a Cell Identity with one of its associated PLMN identifiers for identifying its serving NG-RAN node. E.g., identification e.g. of the target NG-RAN node in the course of handover or transfer of RAN configuration information may use a combination with a different PLMN identifier than the one used at NG Setup.

NOTE 2:	NGAP and XnAP signalling should use the first PLMN ID within the common subset of PLMN IDs supported by the serving AMF and broadcast for the cell, following the order broadcast in SIB1.