3GPP TSG-RAN WG3 Meeting#107-e	R3-200198

24th – 28th February 2020

Agenda item:	30.1

Source: 		Qualcomm Incorporated

Title: 	Summary of offline discussion on NR-U impacts

Document for:	Discussion

# 1. Introduction

This is the summary for the following email discussion:

CB: # 11_Email011-NR-U

-  add RAT type?

- RAT restriction handling for primary and secondary RATs?

- rev and merge if needed; check details

(QC)

Summary of offline disc R3-201198

# 2. Discussion [may be moved to annex at end]

Summary as of Tuesday 22.00 CET

Based on the discussion so far, the following proposals can be made for checking (see below for some added detail / reasoning). At least P2 and P4 seem clearly agreeable so far, others may need checking.

Proposal 1: Use RAT Information IE, remove NB-IOT codepoint [request NB-IOT CR editor to align].

Proposal 2: Define new IE “Extended RAT Restriction Information” including separate primary and secondary restrictions.

Proposal 3: Use the following codepoints:

| Primary | E-UTRA, NR, NR-u |
| --- | --- |
| Secondary | E-UTRA, E-UTRA-U, NR, NR-u |

Proposal 4: Add two new codepoints (nr-u, e-utra-u) to RAT Type in data volume reporting in 5GS.

Proposal 5: Expand the use of the existing Unlicensed Spectrum Restriction to cover also NR-u in EPS

Proposal 5a: Add same IE to X2AP

Proposal 6: Introduce a codepoint for NR-U in data volume reporting in 36.423

There are two input discussion documents and respective sets of CRs (2x4). I propose we start by considering each of the functional issues, below I have a list but please propose any additions if you spot anything missing. In passing, I would recommend checking and using SA2 requirements and CT specs (related to subscription in particular) as much as possible (although of course these may still change):

1. Enforcing the primary access restrictions in 5GS

Both CRs use a similar approach i.e. attaching a new IE to the TAC (as done for NB-IOT in EPS), so perhaps this can be agreeable.

Main difference between CRs is in the scope as 666 introduces RAT-Type with a single codepoint of "unlicensed", whereas 282 introduces RAT Information and has two code points, unlicensed and NB-IOT. Note that 282 aligns with the WI list used in the equivalent CRS in SA2 (i.e. also adds TEI-16 so scope is not limited in general to NR-U).

| Company | Comment /Answer |
| --- | --- |
| QC | The approach of associating the IE with TAC seems common. We have also introduced NB-IOT as this is part of the SA2 CRs (note that like the SA2 CRs, we have also introduced a TEI16 WI in cover sheet to cover aspects that are not strictly NR-U specific, there are several). Note also that (although it is a detail) we have used the name RAT Information for the IE. There are several reasons for this, one of them being that RAT Type is used in data volume reporting (and not reusable), and also this is not truly RAT Type as the default RAT is derived from the node ID. |
| Nokia | Regarding the naming of the IE, we prefer not to use “RAT Type” since there is an existing data type with this name in CT4 that has different semantics. We are OK with “RAT Information” as proposed in 282. |
| ZTE | OK with “NB-IOT” and naming “RAT Information” |
| Huawei | The “NB-IoT” is not needed since it seems to collide with the agreed BL CR for NB-IoT. We slightly prefer to use the “RAT type”. But ok with the “RAT Information” |

Based on the feedback below, the open issue is whether it is necessary to have two separate IEs. One possible approach would be to coordinate with the NB-IOT CR editor to use the same IE (and the two codepoints can be merged at the implementation).

Proposal 1: Use RAT Information IE, remove NB-IOT codepoint [request NB-IOT CR editor to align]

2. Mobility restrictions in 5GS: general approach

Both CR sets take the approach of using IEs rather than relying on Forbidden TACs, so perhaps this could be also taken as an agreement.

There are some differences in coding. Doc 282 adds a new IE Extended RAT Restriction Information, which contains separate primary and secondary RAT restrictions. If the new IE is not present, then for backward compatibility, the existing IE continues to be used for all mobility actions. Doc 666 adds a new IE for secondary RAT only. The main difference between the two approaches is in terms of backwards compatibility, i.e. how the behaviour of rel-15 and rel-16 nodes will be controlled.

| Company | Comment /Answer |
| --- | --- |
| QC | Initially we considered the approach of adding a new IE for secondary only as well (this is in the November Reno submission). However after some discussion (also with co-signers), we think it is better to use an IE with both primary and secondary restrictions because: - If not, the behaviour of rel-15 and 16 nodes is tied in specific way: it is not possible for example to have a RAT that is forbidden for primary use only in release 16 nodes, without this resulting in being forbidden for ALL usage in release 15 nodes. Conversely if you wish to restrict a RAT for secondary use only in release 16 nodes, it is not possible to have any rel-15 restrictions. NOTE we assume that the rel-15 IE applies to both primary and secondary. - CT1 has used specifically the same approach in their specifications (new IE with independent primary and secondary; if received, legacy IE is ignored) |
| Nokia | We also assume that the legacy IE applies to both primary and secondary access. Therefore, the legacy IE should include only the RATs which are allowed for both primary and secondary access (so e-UTRA-U should not be included in the legacy IE, since it is allowed only for secondary access). We are fine with the proposal in 282. |
| ZTE | fine with the proposal in 282, i.e. separate primary and secondary RAT restrictions. |
| Huawei | We also understand the legacy IE applies both primary and secondary access. After further thinking, we are fine to have a new “Extended RAT Restriction Information” for Rel-16. |

Seems like it is possible to have an agreement on the below proposal:

Proposal 2: Define new IE “Extended RAT Restriction Information” including separate primary and secondary restrictions.

3. Codepoints for mobility restrictions in 5GS

Below summarizes the codepoints in the two proposal, and the difference

|  | 282 | 666 | Diff |
| --- | --- | --- | --- |
| Primary | E-UTRA, NR, NR-u | E-UTRA, NR, E-UTRA-u, NR-u | Need for E-UTRA-u ? |
| Secondary | E-UTRA, NR, NR-u | E-UTRA-u, NR-u | Need for E-UTRA-u, E-UTRA, NR |

We need to discuss the reason for differences. Note that SA2 spec calls for separate restriction on NR and E-UTRA as secondary access (separate from unlicensed spectrum usage).

| Company | Comment /Answer |
| --- | --- |
| QC | The first point to make in this context, is that primary RAT is the RAT used during access or more generally in the PCell. With that our understanding, LTE-u codepoint is not required as primary RAT. On the other hand (checking SA2 requirements), both LTE and NR should be codepoints for secondary RAT. Also LTE-u should probably also be a codepoint in secondary RAT, as in general it is possible to use it in both MN and SN (note this is not included in the SA2 requirements, but may have been an oversight) |
| Nokia | We support the encoding in 282, with the addition from 666 of e-utra-unlicensed to the Secondary RAT Restriction assuming there is no impact to RAN2/RAN1. |
| ZTE | For primary, no need for E-UTRA-U; for secondary, need for E-UTRA-U. |
| Huawei | Ok not to include the E-UTRA-U for primary access now. Include the E-URATR for secondary access, following the SA2 specification. |

Let’s check if it is possible to agree the following codepoint proposal:

Proposal 3: Use the following codepoints:

| Primary | E-UTRA, NR, NR-u |
| --- | --- |
| Secondary | E-UTRA, E-UTRA-U, NR, NR-u |

4. Data volume reporting in 5GS

Both documents add two new codepoints (nr-u, e-utra-u) to RAT Type in data volume reporting. It is proposed to take this as an agreement.

| Company | Comment /Answer |
| --- | --- |
| QC | Fine for us to take this as agreement |
| Nokia | Also fine for us. |
| ZTE | ok |
| Huawei | ok |

This (p4) can be considered agreed

Proposal 4: Add two new codepoints (nr-u, e-utra-u) to RAT Type in data volume reporting in 5GS.

5. Handover Restriction List in EPS

We have two approaches: 284 expands the use of the existing Unlicensed Spectrum Restriction to cover also NR-u; 68 adds a new IE specific to NR-u. Both approaches add to XnAP, with the difference that by adding the expanded "legacy IE" to X2AP, 285 claims to also correct an error in legacy due to possible use of LAA in the secondary.

| Company | Comment /Answer |
| --- | --- |
| QC | When checking the SA2 CR for 23.401 (S2-1910667), this treats NR-u as another form of use of unlicensed spectrum to add to a list that includes LAA and LWA/LWIP. For example it talks of “UE's permission to use NR as a secondary RAT, Unlicensed Spectrum in the form of LAA/LWA/LWIP/NR-U (as specified in clause 4.3.30)”. Therefore it seems to make sense to just reuse the existing IE. The legacy IE does need to be added to X2AP, and strictly this is a quasi-correction, as the restriction could be used to stop use of LAA in SN. However this can be done using the same approach as in S1AP. |
| Nokia | We are fine with the proposal in 284, unless there are requirements in EPS to have separate handover restriction for NR-U than other unlicensed access (but seems not the case since already LAA was bundled together with LWA/LWIP). |
| ZTE | Fine with 284. |
| Huawei | The reason we introduce this new “NR unlicensed restriction in EPS as secondary RAT” IE is to explicitly describe the restriction of “NR as a secondary RAT”. This can align with the legacy IE “NR Restriction in EPS as Secondary RAT”. If we follow 284, this cannot support the following sceanrio. For example, if the LAA is restricted for primary access while NR-U is restricted for secondary access. |

It seems there is more support for the approach in 284; it is understood that there is no support for separating restriction of LAA from NR-U in EPS, as per SA2 requirement.  Can we try:

Proposal 5: Expand the use of the existing Unlicensed Spectrum Restriction to cover also NR-u in EPS

Proposal 5a: Add same IE to X2AP

6. Data volume reporting in EPS

Doc 669 does not add anything here. Doc 285 adds a code point for secondary RAT (unlicensed), which brings it into line with 36.413.

| Company | Comment /Answer |
| --- | --- |
| QC | No change in TS 36.413 is needed as there is a codepoint for “unlicensed”, and no semantics. However we see a need to add this codepoint in the data volume report for TS 36.423, since NR-u could be used in the SN. |
| Nokia | After some further thinking, I am wondering whether the new codepoint in 285 should be “nR-U” instead of “unlicensed”, since “unlicensed” codepoint may be considered as introducing (as side-effect) secondary RAT reporting for LAA and LWA/LWIP (which I am not sure is possible, at least not without further investigation and seems beyond the scope of the NR-U work item). In other words, in places where there is already an “unlicensed” codepoint, we can expand this to include NR-U. But in places where there is not currently an “unlicensed” codepoint (e.g. data volume reporting), perhaps we should include only NR-U (and not generalize to “unlicensed”). |
| ZTE | Similar concern as Nokia’s, NR-U is more specific. |
| Huawei | For 36.413, no need to change For 36.423, agree Nokia to change to “NR-U” |

Let’s then go with this:

Proposal 6: Introduce a codepoint for NR-U in data volume reporting in 36.423

# 3. Summary and conclusions