3GPP TSG-RAN WG3 #107-e		R3-201276

E-meeting, 24th February – 6th March 2020

Source:	CATT

Title:	                  Summary on CB: # 106_HO_Timer_handling

Agenda Item:	9.3.8.1

Document for:	Discussion and decision

Introduction

This contribution summarizes discussions on the following CB:

CB: # 106_HO_Timer_handling

-  check details

- make consistent with CHO handling (there timer handling is captured)

- further check against CHO handling in Rel-16

(CATT)

Summary

After further check the NGAP baseline CR for Rel-16 mobility enhancement WI, the following paragraph is introduced

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall stop the timer TXnRELOCprep and terminate the Handover Preparation procedure. If the procedure was initiated for an immediate handover, the source NG-RAN node shall start the timer TXnRELOCoverall. The source NG-RAN node is then defined to have a Prepared Handover for that Xn UE-associated signalling.

Editor’s note: the wording “immediate handover” for legacy HO (i.e. non-CHO) is FFS

So, in Rel-16, the CR for mobility enhancement WI has already included this timer related description, Rel-16 CR dedicated for this issue is not needed.However,Rel-15 CR is still necessary which could use similar sentence, just remove the words highlight with yellow.

Proposal: It is proposed to agree the Rel-15 CR in R3-201275.