3GPP TSG-RAN WG3 #107-e		R3-201277

E-meeting, 24th February – 6th March 2020

Source:	CATT

Title:	                  Summary on CB: # 105_OptionalIEs_incl_security_key

Agenda Item:	9.3.8.1

Document for:	Discussion and decision

Introduction

This contribution summarizes discussions on the following CB:

CB: # 105_OptionalIEs_incl_security_key

-  find appropriate wording for statements on optional IEs

- fix the Index to RAT/Freq sel prio IE

- add impact to other specs

- tick CN box in cover page

- cat A CR needed

- check details

- should leave abnormal conditions for re-keying to implementation?

(CATT)

Summary

The CR in [1] focuses on the following two open issues:

- Three IEs i.e. Security Key IE , UE Security Capabilities IE and Index to RAT/Frequency Selection Priority IE are optional, the corresponding text should be updated as statement on optional IE

- The current description on security key IE is technically not correct.

During initial UE context setup procedure, after NG-RAN node received security key, the AS level security would be activated for the UE. Then if security key is included in UE context modify request message, it is AMF initiated AS re-keying procedure. So, it is not correct to say the NG-RAN node should active security for UE

The CR in [3] focus on the scenario that both security key IE and Emergency Fallback IE is present.

During the offline discussion, there is proposal to combine the two CRs  together  and add description as below:

If the Security Key IE is included in the UE CONTEXT MODIFICATION REQUEST message and if the Emergency Fallback Indicator IE is not included, the NG-RAN node shall, if supported, store it and perform AS key re-keying according to TS 33.501[13]. Otherwise it is up to NG-RAN implementation.

However, due to limited time, there is not enough time to discuss on this proposal,  So the proposal is as follow:

Proposal : It is proposed to continue the discussion in text meeting..

Reference

[1] R3-200449; Discussion on open issue for nested SN modification procedure CATT

[2] R3-201273; Correction on nested SN modification procedure CATT

[3] R3-200937; Clarification of AS re-keying in the UE Context Modification procedure E///