3GPP TSG-RAN WG3 Meeting #107bis-e	R3-201640

20 - 30 April 2020, E-Meeting

Agenda item:		20.2.2

Source:	LG Electronics Inc.

Title:	Remaining issues on F1 support for 5G sidelink resource modes

Document for:		Discussion

1. Introduction

In last meeting, some of the F1 issues were solved, but still there are some remaining issues to handle. This paper is to further investigate the open issues again. The corresponding proposals are also provided.

2. Discussion

The following open issues on F1 support for 5G sidelink resource modes will be investigated in this paper:

  - The way of transmission of the sidelink resource request from CU to DU

    - Whether to re-use UE Context Setup/Modification for sidelink resource request or new message

    - Whether RRC container or parallel IEs in the selected message above

  - Parameters on the SL DRB setup/modification/release

  - Parameters on V2X SIBs

  - Handling on the FFSs in the BLCR

Issue 1: on the way of transmission of the sidelink resource request from CU to DU?

The first issue is on

    - Whether to re-use UE Context Setup/Modification for sidelink resource request or new message

After further checking, the existing UE Context procedures are workable. Also from the discussion of last meeting, it seems that majority prefers to go this solution. So it is proposed to turn it into agreement.

Proposal 1): To turn the working assumption, i.e., re-use UE Context Setup/Modification for sidelink resource request, into agreement.

The second issue is on the format of IE in the procedure, for which we need to decide which one to go within the following two options:

    - Option 1: Parallel IEs in UE Context Setup/Modification message

    - Option 2: RRC container

For that, we sent LS to RAN2 for checking whether it is feasible or not to put sidelinkUEinformation in the existing RRC containers, e.g., CG-ConfigInfo. The following reply LS [3] was received from RAN2:

1. Overall Description:

RAN2 would like to thank RAN3 for their LS. According to the two options described in the RAN3 LS, RAN2 would like to inform RAN3 that Option 2 is feasible but RAN2 assumes RAN3 will make a final decision.

2. Actions:

To RAN3 group.

ACTION: 	 RAN2 kindly ask RAN3 to take the above conclusion into consideration.

So it can be seen that RAN2 answered that option 2 is feasible. The final decision should be done by RAN3.

In RAN3#106 meeting, RAN3 slightly prefers to go for option 2 since the impacts to RAN3 specification are minor. Now that RAN2 thinks it is feasible, then we can go for option 2 and send LS to them to notify our decision and let them take it into account for their specifications.

Proposal 2): To go for option 2, i.e., RRC containers solution, and send LS to RAN2 to notify our decision and let them take it into account for their specifications.

Issue 2: Parameters on V2X SIBs

Since RAN2 has agreed the SIB information for V2X. Therefore, the gNB-DU System Information IE should be updated to align as they are defined by RAN2. The following SIBs are in the BLCR of RAN2 38.331:

- SIB12

~ SIBX contains NR sidelink communication configuration

- SIB13

~ SIBY contains configurations of V2X sidelink communication defined in TS 36.331

- SIB14

~ SIBZ contains configurations of V2X sidelink communication defined in TS 36.331, which can be used jointly with that included in SIB13.

Proposal 3): To update SIB information in gNB-DU System Information IE for aligning with TS 38.331.

Issue 3: other issues

In last meeting, the following agreement has been achieved:

- Introduce the UEAssistanceInformationEUTRA IE in the CU to DU RRC Information IE

However, the specification has not been changed to reflect the agreement above.

Proposal 4): To update F1AP on introducing the UEAssistanceInformationEUTRA IE in the CU to DU RRC Information IE.

Proposal 5): To agree the TP in [4] for BL CR TS 38.473.

3. Conclusion

In this contribution, the remaining issues on F1 interface for supporting V2X were investigated. The following proposals are suggested to RAN3:

Proposal 1): To turn the working assumption, i.e., re-use UE Context Setup/Modification for sidelink resource request, into agreement.

Proposal 2): To go for option 2, i.e., RRC containers solution, and send LS to RAN2 to notify our decision and let them take it into account for their specifications.

Proposal 3): To update SIB information in gNB-DU System Information IE for aligning with TS 38.331.

Proposal 4): To update F1AP on introducing the UEAssistanceInformationEUTRA IE in the CU to DU RRC Information IE.

Proposal 5): To agree the TP in [4] for BL CR TS 38.473.

4. References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- 3GPP TR 38.885 V1.0.0: Study on Vehicle-to-Everything

- R2-2001972, “Reply LS on Sidelink UE Information”, RAN2

- R3-201649, “(TP for BLCR 38.473) remaining Issues on F1”, LG Electronics