3GPP TSG-RAN3 Meeting #107-bis-e	R3-201740

E-meeting, 20 -30 April 2020

Title: 	(TP for SON BL CR for TS 38.413): UE reported history information

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	Other

# 1. Introduction

UE reported history information was discussed during RAN3#107-e and a set of BL CRs were endorsed. In this paper we propose some further refinement for UE reported history information.

# 2. Discusion

The UE reported history information was agreed by RAN2, which contains the visited cell information list including the mobility history information of maximum of 16 most recently visited cells or the time spent outside E-UTRA or NR. It is clear that we will have at least one mobility history report defined in TS 38.331 and one in TS 36.331. There is however no conclusion yet how to support reporting in LTE when connected to 5GC.

In NG-RAN, we have gNB and ng-eNB using different RRC specification. That means, we may see two different mobility history report from LTE and NR. Including two different reports is not a problem, but it may be suboptimal if the receiving node cannot distinguish these two before it tries to decode the container.

Therefore we propose to introduce a choice structure for the UE reported history information containing the mobility history report in 38.331 and the mobility history report in 36.331 in the future.

Proposal 1: Define UE History Information from the UE IE as a choice structure with one entry for the mobility history report in TS 38.331.

For the UE History Information from the UE IE in current BL CR for TS 38.423, the procedural text and ASN.1 make it as Optional, where the tabular makes it as Mandatory. Based on the RAN2 specs, we believe that the the UE History Information from the UE IE should be Optional. This is also how the BL CR for TS 38.413 is currently written.

Proposal 2: Make the presence of UE History Information from the UE IE as optional in BL CR for TS 38.423.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	Define UE History Information from the UE IE as a choice structure with one entry for the mobility history report in TS 38.331.

Proposal 2:	Make the presence of UE History Information from the UE IE as optional in BL CR for TS 38.423.

# Annex – TP

Start of the first change

9.3.1.29	Source NG-RAN Node to Target NG-RAN Node Transparent Container

This IE is produced by the source NG-RAN node and is transmitted to the target NG-RAN node. For inter-system handovers to 5G, the IE is transmitted from the external handover source to the target NG-RAN node.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in TS 38.331 [18] if the target is a gNB. Includes the RRC HandoverPreparationInformation message as defined in TS 36.331 [21] if the target is an ng-eNB. | - |  |
| PDU Session Resource Information List |  | 0..1 |  | For intra-system handovers in NG-RAN. | - |  |
| >PDU Session Resource Information Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>QoS Flow Information List |  | 1 |  |  | - |  |
| >>>QoS Flow Information Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>>>DL Forwarding | O |  | 9.3.1.33 |  | - |  |
| >>>>UL Forwarding | O |  | 9.3.1.118 |  | YES | reject |
| >>DRBs to QoS Flows Mapping List | O |  | 9.3.1.34 |  | - |  |
| E-RAB Information List |  | 0..1 |  | For inter-system handovers to 5G. | - |  |
| >E-RAB Information Item |  | 1..<maxnoofE-RABs> |  |  | - |  |
| >>E-RAB ID | M |  | 9.3.2.3 |  | - |  |
| >>DL Forwarding | O |  | 9.3.1.33 |  | - |  |
| Target Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | - |  |
| UE History Information | M |  | 9.3.1.95 |  | - |  |
| SgNB UE X2AP ID | O |  | 9.3.1.127 | Allocated at the Source en-gNB | - |  |
| UE History Information from the UE | O |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofE-RABs | Maximum no. of E-RABs allowed towards one UE. Value is 256. |

Start of the next change

#### 9.3.1.y	UE History Information from the UE

This IE contains information about mobility history report for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE History Information from the UE | M |  |  |  |
| >NR |  |  |  |  |
| >>NR Mobility History Report | M |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 38.331 [18]). |

Start of the next change

### 9.4.5	Information Element Definitions

<Unchanged Text Omitted>

NRintegrityProtectionAlgorithms ::= BIT STRING (SIZE(16, ...))

NRMobilityHistoryReport ::= OCTET STRING

NRPPa-PDU ::= OCTET STRING

<Unchanged Text Omitted>

UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCellItem

UEHistoryInformationFromTheUE ::= CHOICE {

nR						NRMobilityHistoryReport,

choice-Extensions		ProtocolIE-SingleContainer { {UEHistoryInformationFromTheUE-ExtIEs} }

}

UEHistoryInformationFromTheUE-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

<Unchanged Text Omitted>