3GPP TSG-RAN WG3 #107bis-e						R3-201765

E-meeting, 20 – 30 April 2020

Agenda Item:  15.3.3

Source:       NEC

Title:         CHO in E1 interface

Document for:	decision

- 1. Introduction

Last meeting RAN3#107-e discussed in email#92 for the support of CHO setup/modify/cancel over E1. Summary in [R3-201163].

We give a discussion and proposal for the CHO over E1.

- 2. Discussion

If there will be a scenario that a single gNB-CU-UP will have the multiple preparation for the same UE, and if there will be a case that the gNB-CU-CP and the same target gNB-CU-UP will use the same UE AP ID for multiple preparation, then something else ID than the UE AP ID is needed to identify the prepared bearer context. Target cell ID may be the one.

Alternatively, it come to a very basic view that, for simplicity reason, we can just rule out the case to use same UE AP ID for multiple preparation within the same target node, then we will not need the 3rd ID.

Observation: both ways can work. If for simplicity reason then using different UE AP ID for multiple preparation with the same target node will be same handling as towards the different target nodes.

Proposal 1: If to take simplicity reason, no need to introduce the 3rd ID, instead, avoid using same UE AP ID for multiple preparation with the same target node.

For other process in the E1 interface:

- For the new Bearer Context Setup, same as today, nothing different.

- For the CU-CP-initiated modification, a new Bearer Context Setup procedure is initiated, then release the existing one.

- For the CU-CP-initiated cancellation, use the (gNB-CU-CP initiated) Bearer Context Release procedure.

- For the CU-UP-initiated cancellation, use the (gNB-CU-UP initiated) Bearer Context Release Request procedure.

- For the CU-UP-initiated modification, follow the CU-UP-initiated Bearer Context Release procedure, then CU-UP re-trigger a new Bearer Context Setup procedure.

Therefore, we can use the existing procedures in E1AP for the Rel-16 CHO.

Proposal 2: No need to introduce anything in E1 for the CHO.

- 3. Conclusion and proposal

Proposal 1 : If to take simplicity reason, no need to introduce the 3rd ID, instead, avoid using same UE AP ID for multiple preparation with the same target node.

Proposal 2: No need to introduce anything in E1 for the CHO.

Ref:

R3-201163