3GPP TSG-RAN3 Meeting #107-bis-e	R3-201969

E-meeting, 20 -30 April 2020

Title: 	(TP for WWC BL CR for TS 38.413) Support for interfacing trusted non-3GPP access networks to the 5GC

Source: 	Huawei, BT, Broadcom

Agenda item:	21.2

Document for:	Other

Annex – TP

Change Begins

#### 9.2.5.3	UPLINK NAS TRANSPORT

This message is sent by the NG-RAN node and is used for carrying NAS information over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | Ignore |
| W-AGF Identity Information | O |  | Identity Information 9.3.1.bb |  | YES | reject |
| TNGF Identity Information | O |  | OCTET STRING | Containing the TngfInfo specified in TS 29.510 [z]. | YES | reject |
| TWIF Identity Information | O |  | OCTET STRING | Containing the TwifInfo specified in TS 29.510 [z]. | YES | reject |

#### /********Skip to next change**********/

9.4.4	PDU Definitions

<Unchanged Text Omitted>

-- **************************************************************

--

##### -- UPLINK NAS TRANSPORT

--

-- **************************************************************

UplinkNASTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{ {UplinkNASTransport-IEs} },

...

}

UplinkNASTransport-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE mandatory	}|

{ ID id-W-AGFIdentityInformation	CRITICALITY reject	TYPE IdentityInformation			PRESENCE optional	}|

{ ID id-TNGFIdentityInformation		CRITICALITY reject	TYPE OCTET STRING				PRESENCE optional	}|

{ ID id-TWIFIdentityInformation		CRITICALITY reject	TYPE OCTET STRING				PRESENCE optional	},

...

}

<Unchanged Text Omitted>