3GPP TSG-RAN3 Meeting #107-bis-e	R3-202049

E-meeting, 20 -30 April 2020

Title: 	(TP to BL CR for TS 38.305) Clean up related CU-DU

Source: 	Huawei

Agenda item:	19.1

Document Type:	Approval

# 1. Introduction

This document propose a cleant-up of the TS 38.305 with regard to the CU-DU split and the section related to TS 38.401.

# 2. Discussion

- We would like to stick on previous RAN3 agreement that the disaggregated part of the gNB, when apply, should not be exposed at stage 2 level.

- The changes proposed are:

- to move the section 5.1.1 to TS 38.401. The Editorial note is replace by a note in TS 38.401 (see R3-202048) :

Note: A gNB-DU with TRP may only support positioning functionality by appropriate implementation.

- To remove the subclause F1 (6.1.6) on interface and add a reference to TS 38.401 in the main section (6.1.1). We would like also to note that this section referees to TS 23.501 which does not include any reference to F1…

- Remove the sentence “In case of a split gNB, the NRPPa protocol is terminated at gNB-CU”. This sentence is a redundant specification, usually not welcome in the specification. The same text is captured in section 5.1.1 (then reported in TS 38.401 via R3-202048) “The gNB-CU terminates the NRPPa protocol”

- Remove of section 6.7, the gNB-CU signalling shall be described as other in TS 38.401 (see R3-202048).

# 3. Conclusion

It is propose to be agree on the following text proposal in Annex reflecting the change discussed above.

(Note the reference document is the draft provided on the reflector)

The related CR to capture equivalent text in TS 38.401 is proposed in R3-202048

# Annex A – TP for TS 38.305

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

-

<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>

## 6.1	Network interfaces supporting positioning operations

### 6.1.1	General LCS control plane architecture

The general LCS control plane architecture in the 5GS applicable to a target UE with NG-RAN access is defined in TS 23.501 [2]. The NG-RAN architecture to support positioning, e.g. the gNB F1 interface, is described in TS 38.401 [x4]

### 6.1.2	NR-Uu interface

The NR-Uu interface, connecting the UE to the gNB over the air, is used as one of several transport links for the NR positioning protocol(s) for a target UE with NR access to NG-RAN.

### 6.1.3	LTE-Uu interface

The LTE-Uu interface, connecting the UE to the ng-eNB over the air, is used as one of several transport links for the LTE positioning protocol(s) for a target UE with LTE access to NG-RAN.

### 6.1.4	NG-C interface

The NG-C interface between the gNB and the AMF and between the ng-eNB and the AMF is transparent to all UE-positioning-related procedures. It is involved in these procedures only as a transport link for the NR positioning protocol(s).

For gNB related positioning procedures, the NG-C interface transparently transports both positioning requests from the LMF to the gNB and positioning results from the gNB to the LMF.

For ng-eNB related positioning procedures, the NG-C interface transparently transports both positioning requests from the LMF to the ng-eNB and positioning results from the ng-eNB to the LMF.

For delivery of broadcast assistance data information, the NG-C interface transparently transports both the assistance data information from the LMF to the NG-RAN node for broadcasting and the feedback information on assistance information broadcasting from the NG-RAN node to the LMF. The NG-C interface is also used by an AMF to transparently transport ciphering keys via NG-RAN node to UEs using a NAS message. The ciphering keys are used to decipher broadcast assistance data information, if the broadcast assistance data information is ciphered.

### 6.1.5	NLs interface

The NLs interface, between the LMF and the AMF, is transparent to all UE related, gNB related and ng-eNB related positioning procedures. It is used only as a transport link for the LTE Positioning Protocols LPP and NRPPa.

<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>

## 6.3	NG-RAN Node terminated protocols

### 6.3.1	NR Positioning Protocol A (NRPPa)

The NR Positioning Protocol A (NRPPa) carries information between the NG-RAN Node and the LMF. It is used to support the following positioning functions:

-	E-CID for E-UTRA where measurements are transferred from the ng-eNB to the LMF.

-	Data collection from ng-eNB's for support of OTDOA positioning for E-UTRA.

-	Cell-ID and Cell Portion ID retrieval from gNB's for support of NR Cell ID positioning method.

-	Exchange of information between LMF and NG-RAN node for the purpose of assistance data broadcasting.

-	NR Multi-RTT where measurements are transferred from the gNB to the LMF.

-	NR UL-AoA where measurements are transferred from the gNB to the LMF.

-	NR UL-TDOA where measurements are transferred from the gNB to the LMF.

-	Data collection from gNBs for support of DL-TDOA, DL-AoD, Multi-RTT, UL-TDOA, UL-AoA .

The NRPPa protocol is transparent to the AMF. The AMF routes the NRPPa PDUs transparently based on a Routing ID corresponding to the involved LMF over NG-C interface without knowledge of the involved NRPPa transaction. It carries the NRPPa PDUs over NG-C interface either in UE associated mode or non-UE associated mode.

<<<<<<<<<<<<<<<<<<<<Next change >>>>>>>>>>>>>>>>>>>>

## 6.6	Void

<<<<<<<<<<<<<<<<<<<<End change >>>>>>>>>>>>>>>>>>>>