3GPP TSG-RAN WG3 Meeting #107bis-e	R3-202146

# Online, 20th – 30th April 2020

Agenda Item:	17.2.3

Source:	Ericsson

Title: 	Further discussion on Enhancements for DL PDCP Duplication

Document for:	Discussion and Approval

# 1	Discussion

There have been three enhancements discussed for DL PDCP Duplication.

- Enh1: Allow assigning “discard timer” to each PDU transmitted from the hosting node to the assisting node / DU.

- Enh2: Allow assigning “hold on” flag to each PDU transmitted from the hosting node to the assisting node / DU; then, explicit “go” command is needed to indicate the PDU shall be transmitted (if the command does not arrive before the validity timer expires, the PDU is discarded at the assisting node / DU).

- Enh3: Allow reporting delivery of any PDU, not only those delivered in order

At the RAN3#106 meeting, it is decided that “Considering the proposed Enhancements 1, 2 and 3, it proposed to capture as first step the Enhancement 3 in R3-197706.

Further discussion needs to occurs whether additional solution should be considered.”

This paper discuss the remaining two enhancements.

Currently the node hosting RLC entity discard the PDCP packets according to the information received in the DL Data.

Normally in DC deployment, the node hosting PDCP entity receives from one RLC entity the Downlink delivery status. It then decides to inform the other leg to discard the PDCP packets that being delivered successfully (and perhaps not reported being delivered). The node hosting the RLC entity then discard the indicated PDCP packages.

For low latency NR IIoT deployment, when up to 4 copies might be sent, one enhancement could be that the PDCP entity provide a “discarding timer” to the RLC entity, indicating that if the PDCP packages are not scheduled and delivered within the time, then it is no point to transmit them. The RLC entity could just discard those packets.

Comparing the two enhancements to be further discussed, we propose to agree on Enhancement 1.

Proposal 1:   RAN3 to agree on the Enhancement 1, the “discard timer” proposal.

# 2	Proposal

Proposal 1:   RAN3 to agree on the Enhancement 1, the “discard timer” proposal.

# 3	Reference