3GPP TSG-RAN WG3#107bis-e                                                                           R3-202859

20 - 30 April 2020

Online

Title:	LS on cell-specific signals/channel configurations

Response to:

Release:	Rel-16

Work Item:	IAB_NR-Core

Source:	RAN3

To:	RAN1

Cc:

Contact Person:

Name:	Xueying Diao

E-mail Address:	diao.xueying@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

According to the Rel-16 NR PHY parameter list provided in RAN1 LS (R1-1913675, R3200081), RAN3 is in procedure of designing F1-AP parameter, namely Child-IAB-DU-Resource-Configuration-CellSpecificSignalsChannels-Config (final name TBD), based on following RAN1 agreement:

RAN1 #99 Agreements: A parent IAB node/donor can be provided with cell-specific signals/channels configurations of each child IAB-DU. How/whether to use the information to handle any potential conflict at the parent IAB node/donor is left to network implementation The list of cell-specific signals/channels includes: - resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB; - configured RACH occasions for receiving at the DU - periodic CSI-RS transmission at the DU - scheduled resource for receiving SR at DU

RAN3 would like to point out that CSI-RS and SR resources can be UE/MT-specific. Each child IAB-DU may serve many UEs and IAB-MTs, and these UE/MT-specific parameters are likely to change relatively often, resulting in a F1AP signaling storm between IAB donor and IAB nodes. To address this issue, RAN3 considers the following alternatives.

- Explicitly configure these resources used for CSI-RS and SR as Hard at the child node or Not Available at the parent node. Meanwhile, exclude CSI-RS and SR configurations from the list of cell-specific signals/channels configurations. Make the CSI-RS and SR configurations as optional in the cell-specific signals/channels configurations so that they do not have to be configured if signaling storm becomes a concern.

RAN3 kindly asks RAN1 to provide feedback whether the above alternative approaches are feasible from RAN1 perspective and whether any additional alternatives should be considered.

2. Actions:

To RAN1 group:

ACTION: RAN3 kindly asks RAN1 to provide feedback whether the following approaches are feasible from RAN1 perspective and whether any additional alternatives should be considered.

- Explicitly configure these resources used for CSI-RS and SR as Hard at the child node or Not Available at the parent node. Meanwhile, exclude CSI-RS and SR configurations from the list of cell-specific signals/channels configurations.

- Make the CSI-RS and SR configurations as optional in the cell-specific signals/channels configurations so that they do not have to be configured if signaling storm becomes a concern.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP RAN3#108-e          1 – 12 June 2020			Online

3GPP RAN3#109             24 – 28 August 2020                                          Toulouse, France