3GPP TSG-RAN WG3 Meeting #107bis-e                                                            	R3-202818

E-Meeting, 20th– 30th April 2020

Title:	LS on information needed for MRO in UE RLF Report

Response to:	R2-2002393/R3-201515

Release:	Rel-16

Work Item:	          NR_SON_MDT-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name:	Lixiang Xu

E-mail Address:	lx.xu@samsung.com

- Overall Description:

RAN3 thanks RAN2 for their Reply LS on Information Needed for MRO in UE RLF Report.

After further discussion, RAN3 reached agreement on the information which is necessary for MRO in UE RLF Report in Rel-16 as below:

- CGI of the E-UTRA or NR cell that served the UE at the last handover initialization in NR RLF Report. Previous PCell Id is either NR CGI or E-UTRA CGI. E-UTRA CGI of previous PCell should be added to the NR RLF Report.

- CGI of the target E-UTRA or NR cell of the handover (in case of handover failure) in NR RLF Report. Failed PCell Id is either NR CGI or E-UTRA CGI. E-UTRA CGI of failed PCell should be added to the NR RLF Report.

- CGI of the NR or E-UTRA cell that served the UE at the last handover initialization in LTE RLF Report. Previous PCell Id is either NR CGI or E-UTRA CGI. NR CGI should be added to the LTE RLF Report.

- CGI of the target NR or E-UTRA cell of the handover (in case of handover failure) in LTE RLF Report. Failed PCell Id is either NR CGI or E-UTRA CGI. NR CGI should be added to the LTE RLF Report.

- CGI of successful re-connected NR cell or E-UTRA cell: For inter-RAT and inter-system MRO, inclusion of successful re-connected cell CGI will help the network to detect the root cause of the failure. For E-UTRA cell, the TAC of the successful re-connected cell is also needed. RAN3 already agreed the inter-RAT MRO and inter-system MRO in Rel-16 BL CRs.

- Time interval between HOF/RLF and successful RRC re-connection: This information helps the network to understand whether the re-connection cell could be used to detect the root cause of failure event.

- Source PCell of the failed handover using the NR RRC format in UEInformationResponse message: For handover failure, the UE RLF Report should be forwarded to the source node which triggered the handover. The source PCellId in NR RRC format is needed. failedPCellId-EUTRA should be PCell in which RLF is detected or the source PCell of the failed handover.

- 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account and to update their corresponding specifications.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #108-e	    1st- 12th   June   2020