3GPP TSG-RAN WG3 Meeting #108-e 	R3-204360

1-11 June 2020, Online

Agenda item:	13.2.1.3.

Source:	Nokia, Nokia Shanghai Bell, Qualcomm, Samsung

Title:	(TP for NR-IAB BL CR for TS 38.401) IPv6 mapping

Document for:	Discussion and Decision

# 1	Introduction

This contribution proposes TP to clarify the text on “takes precedence”, when using IPv6 prefix during mapping configuration.

# References

- R3-203067, BL CR to 38.401 Support for IAB (Huawei)

Start of Change

### 8.9.z	IP Address Allocation for IAB-nodes

An IAB-node may obtain IP address(es) either from the IAB-donor or from the OAM system. The IP address(es) is(are) used by the IAB-node for F1 and non-F1 traffic exchange via the backhaul. In case IPsec tunnel mode is used to protect this F1 and non-F1 traffic, the IP address(es) refer to the outer tunnel addresses. The allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

In case of IAB-donor-based IP address allocation, the IP address(es) is(are) allocated by the IAB-donor-CU or IAB-donor-DU. In both cases, the IAB-node requests the IP address(es) via RRC from the IAB-donor-CU. It includes a separate IP address request for each usage, where the usages defined are F1-U, F1-C and non-F1. The IAB-donor-CU may initiate the IAB TNL Address Allocation procedure to obtain IP addresses from the IAB-donor-DU. The IAB-donor-CU sends the IP addresses allocated for each usage to the IAB-node via RRC.

The IAB-node may be allocated one or multiple IPv6 addresses or one 64-bit IPv6 prefix for each usage and/or one or multiple IPv4 addresses for each usage. Each allocated IP address/IPv6 prefix is unique within the IAB network and routable from the wireline network.

In case of OAM-based IP address allocation, the IAB-node informs the IAB-donor-CU via RRC about the IP address(es) it received for each purpose. This occurs before F1 is established. The IAB-donor-CU configures the IAB-donor-DU with mappings between IP header fields and L2 parameters (BAP Routing ID, BH RLC channels) used for DL traffic.

Each mapping configuration may hold an IPv4 address, IPv6 address or a 64-bit IPv6 prefix. In case of two mappings entries matching the same IP header where one mapping holds an IPv6 prefix and the other holds a full IPv6 address, the IAB-donor-DU considers the one with full IPv6 address to take precedence over the one with the IPv6 prefix. Meanwhile, the implementation of avoiding such conflict is not precluded at the IAB-donor-CU.

In case of IAB-donor-allocated IP addresses, the IAB-node’s IP address(es) can be updated using DL RRC signalling.

End of Change